3GPP TSG RAN WG2 #128		R2-2410040

Orlando, US, Nov 18th – 22th, 2024

Title:	Discussion on LCM for UE-sided model for BM

Agenda item:	8.1.2.2

Source:	Google

Document for:	Discussion and Decision

# 1 Introduction

In RAN2#127bis meeting, there were some agreements and FFSs for UE-sided model for BM case.

Agreements for BM

- UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality.   We should aim to align the design on how the applicable functionality are signaled.   FFS on the applicability reporting content.

- FFS if inference configuration can be signalled in step3.

- UE can report to the network when an applicable AI functionality becomes non-applicable.  FFS how this is signaled (e.g. explicitly/implicitly).  Consider different scenarios, whether it is regarding an active functionality)

- Data collection initiation and configuration for data collection is under network control.  FFS how the NW determines whether data collection should be initiated (e.g. via UE requests (UE directly or UE server)

- For the purpose of discussion of AI/ML BM LCM operations, existing procedures and terminologies from the CSI Framework should be used, including those defined for aperiodic, semipersistent on PUCCH, semipersistent on PUSCH, and periodic reporting configurations (as/if defined in RAN1 pending response LS from RAN1).

- For now, RAN2 will not define terminology specific to the activation or deactivation for AI/ML models.  Can come back to this discussion later.

In RAN2#127 meeting, there were some agreements and FFSs for UE-sided model for BM case. Some FFSs were captured in the LS [1] to RAN1 and RAN2 can wait for the related progress in RAN1.

Agreements on definitions

- Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling)

- Applicable functionalities refers to functionalities that the UE is ready to apply for inference

- Activated functionalities refers to functionalities already enabled for performing inference

Agreements on procedures - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc - Step 5: 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. - The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state. FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation. FFS if multiple applicable functionalities can be activated at the same time. FFS what is the granularity of functionality - We will write an LS to RAN1 to provide our agreements and ask specific questions that RAN2 needs to enable progress.

In this paper, we further discuss the remaining FFSs and other issues not dependent on RAN1 and give our considerations.

# 2 Discussion

## Supported functionality information

According to the previous progresses, RAN2 has made the following signalling procedure on applicable functionality reporting for beam management:

Figure 1 Applicable functionality reporting

As shown in figure 1, the UE will report the supported functionalities by UECapablityInformation message in step 2. To enable the identification of the supported functionalities between UE and NW, it is an intuitive idea to introduce the functionality identification (ID) for each supported functionality.

Proposal 1: UE reports the functionality ID of the supported functionalities in UECapablityInformation message.

In RAN1#118bis meeting, it was agreed that feature group can be considered as one possible granularity for supported functionality.

Agreement

Incorporating below text into the general part of the LS

In RAN1’s discussion of RAN 2 terminologies on beam management,

- The concept/terminology “functionality“ of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-specific FGs

- The concept/terminology “ functionality“ of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters or information/parameters indicated by UE

- The Activated functionalities may be enabled based on CSI framework.

Therefore, the meaning and the granularity of “functionality“ for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same, depends on certain option in RAN1, and the discussion is still ongoing.

Proposal 2: The reporting of supported functionality information is feature group specific.

## Inference configuration

Based on the agreements related to figure 1, it is known that the NW may provide the inference configuration to the UE. The inference configuration can guide the UE to derive necessary inputs for the inference operation. Based on the following agreements in RAN1#118bis meeting, it is worth noting that the inference configuration for beam management refers to the parameters in the field CSI-ReportConfig.

Agreement

Incorporating below text into the general part of the LS

In RAN1’s discussion of RAN 2 terminologies on beam management,

- The concept/terminology “functionality“ of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-specific FGs

- The concept/terminology “ functionality“ of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters or information/parameters indicated by UE

- The Activated functionalities may be enabled based on CSI framework.

Therefore, the meaning and the granularity of “functionality“ for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same, depends on certain option in RAN1, and the discussion is still ongoing.

Proposal 3: The inference configuration refers to the parameters in the field CSI-ReportConfig for beam management.

One solution is that the NW initially provides one CSI-ReportConfig in step 5 for the functionality to be activated. The UE stores the parameters in the received CSI-ReportConfig and utilizes them for inference input derivation. For functionality control, the NW may need to switch the activated functionality to adapt to different requirements. To enable the activated functionality switching, the NW may need to provide new CSI-ReportConfig. Generally, the UE will delete the previously stored CSI-ReportConfig and store the new one for future inference. If there is more functionality switching, the NW has to reconfigure the CSI-ReportConfig accordingly.

It is known that the signalling overhead for the CSI-ReportConfig is quite large. If the switching of the activated functionality is dynamic, it will require frequent changes to the CSI-ReportConfig. This will lead to significant signalling overhead.

The alternative solution is to provide more than one inference configuration, e.g., a list of CSI-ReportConfig in step 3. In step 5, the NW simply indicates which pre-configured CSI-ReportConfig to be usedfor inference input derivation. When the functionality switching is performed, the NW indicates a new CSI-ReportConfig to the UE, without explicitly providing the whole CSI-ReportConfig. This seems more efficient and flexible for inference configuration updates of the functionality switching.

Proposal 4: The NW provides multiple inference configurations, e.g., a list of CSI-ReportConfig, in step 3.

Proposal 5: The NW indicates the information of the inference configuration to be used for inference in step 5.

## Applicable functionality Content and Reporting

### Content of applicable functionality information

The reporting of applicable functionality information can assist the NW to identify the one(s) which can be chosen for activation. If the functionality ID is defined and introduced, it is straightforward to explicitly report the functionality ID of the applicable functionality, as previously reported in the supported functionalities.

As mentioned in the above section, the UE initially reports the supported functionalities, e.g., functionality ID, to the NW. Subsequently, it reports the applicable functionality information. The alternative solution may be to report implicit information for the applicable functionalities. For the implicit information, it can be mapped to the supported functionality list and finally used to identify the actual functionality information. Technically speaking, for the implicit reporting scheme, there may be less signalling overhead. For example, a bitmap can be reported as the applicable functionality information. For each bit in the bitmap, it corresponds to a specific functionality in the supported functionality reported in step 2. With the above knowledge, the NW can still deduce the explicit functionality ID of the applicable functionality information.

Proposal 6: For the content of applicable functionality reporting, the following two options can be considered:

- Opt 1: explicit functionality ID of the applicable functionality

- Opt 2: implicit information (e.g., bitmap) mapped to the supported functionality ID

### Reporting of applicable functionality information

In RAN2#127bis meeting, it was agreed that UE can support the applicable functionality reporting in both RRCReconfigurationComplete and UAI messages. The combination of the two messages for applicable functionality reporting still need more detailed designs.

As discussed in the previous section, the NW may provide the inference configuration to the UE in step 3 RRCReconfiguration message.  Technically speaking, the UE needs to reply the step 4 RRCReconfigurationComplete message within a limited time window (response window). The response window can ensure the prompt confirmation of the reconfiguration to the NW. Instead, if the NW doesn’t receive the RRCReconfigurationComplete message, it will consider that the failure occurs for the reconfiguration procedure.

Upon reception of the inference configuration in step 3, the UE performs the applicable functionality determination. As agreed in RAN2#127 meeting, the UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. Therefore, the applicability checking of each functionality requires significant processing on the UE side. Furthermore, the determination of the applicable functionality depends on the amount of the functionalities in device.

Observation: The status for applicable functionality determination depends on the UE processing capability, the amount of functionalities to be checked or other internal implementation.

There may be several cases depending on different UE capabilities and/or the quantity of the functionalities at UE side.

Case 1: complete checking: UE completes the applicability checking for all functionalities at UE side

This is the ideal scenario. In this case, the UE has sufficient processing power to evaluate the applicability checking of all the functionalities at UE side. The UE completes all the checking and derives the checking results before the content determination of the RRCReconfigurationComplete message.

In one possible case, there is the applicable functionality information. The UE can directly include the applicable functionality into the RRCReconfigurationComplete message. Alternatively, it is possible that no applicable functionality which satisfies the checking conditions.

Case 2: partial checking: UE completes the applicability checking for some functionalities at UE side

In this case, the UE processing capability may be limited. And there may be a large number of functionalities in device. Although the UE starts the applicability checking, the UE cannot complete all the checking within the response window. Similar as case 1, the UE may get partial or no applicable functionality information based on the partial checking.

One flexible reporting is to enable the UE to report applicable functionality as they are determined in the RRCReconfigurationComplete message, rather than waiting for all checks to complete. Once the comprehensive checking is finished, the UE can report the new applicable functionality information in the UAI message.

Case 3: no checking: UE does not start the checking for all any functionalities at UE side

In this case, due to some UE implementation, the UE might not begin the applicability checking procedure with the response window. Correspondingly, there is no applicable functionality information available when determining the contents of the RRCReconfigurationComplete message.

Proposal 7: The following cases can be considered for the applicable functionality checking when determining the content of the RRCReconfigurationComplete message:

- complete checking: UE completes the applicability checking for all functionalities

- partial checking: UE completes the applicability checking for some functionalities

- no checking: UE does not start the checking for any functionality

Here is a small summary of the potential information provided in the RRCReconfigurationComplete message for the previous three cases.

|  | Checking results | Information in RRCReconfigurationComplete message |
| --- | --- | --- |
| Case 1: complete checking | Case 1-1: all applicable functionality | applicable functionality |
|  | Case 1-2: no applicable functionality | Void |
| Case 2: partial checking | Case 2-1: part of applicable functionality | applicable functionality |
|  | Case 2-2: no applicable functionality based on completed checking | Void |
| Case 3: no checking | no applicable functionality | Void |

Based on the above table, it is noted that for subcase 1-1 and 2-1, the UE will report the applicable functionality information in the RRCReconfigurationComplete message. But for subcase 2-1, it is the intermediate checking result of the applicable functionality. There may be more applicable functionality in the on-going checking procedure.

The UE should provide an indication to differ the two sub-cases for the NW when providing the applicable functionality information. The indication can indicate whether there is on-going checking at UE side. With this information in mind, the NW may be aware of the status of the applicability checking at UE side. The NW can decide whether to wait for more applicable functionality information in UAI, or proceed with the received  applicable functionality information.

For other subcases,  the UE doesn’t find any applicable functionality. The UE doesn’t provide any applicable functionality information in the RRCReconfigurationComplete message. In subcase 1-2, the UE completes the applicability checking for all functionalities and finds no applicable one. The UE should inform NW of this situation. The NW definitely understands that no functionality is applicable currently. The NW may adjust the NW-side condition for UE to re-check the applicable functionality.  However, for subcase 2-2 and 3, the UE has not completed all the checking or has not started the checking. If the NW can know this, it may anticipate a petential UAI message with applicable functionality information later.

To distinguish the subcases, the UE might also include an indication e if no applicable functionality is provided in the RRCReconfigurationComplete message. The indication can indicate whether there is on-going checking or whether the checking is started or not.

Proposal 8: The UE includes an indication, e.g., on-going, no checking, in the RRCReconfigurationComplete message for applicable functionality reporting.

### Reporting of updated applicable functionality information

Generally, the UE firstly completes the applicability checking based on the current checking conditions. The UE reports the initial applicable functionality information to the network for functionality control.

When the UE detects that the applicability changes for a certain functionality including the active one, the UE should send the updated information to the network to synchronize the applicable functionality information.

An applicable functionality may become non-applicable, and vice versa. The updating of the applicable functionality information can be the addition or deletion of some applicable functionality information. As discussed in section 2.3.1, the design of the applicability report content can be explicit or implicit. The reporting of the change should apply the same principle.

Proposal 9: The reporting of the change of  the applicable functionality information follows the design on the content of the applicable functionality reporting.

As stated above, it is possible that the active functionality may become non-applicable. If the UE continues the inference based on the functionality and waits for the control command from NW, the accuracy or the performance may greatly degrade. It is a bit straightforward for UE to autonomously fall back to non-AI operation for this case.

Proposal 10: The UE autonomously deactivates the activated functionality when it becomes non-applicable and reports the non-applicability of the activated functionality to NW.

To avoid the unwanted autonomous deactivation at UE side, it is beneficial for NW to configure the criteria for autonomous deactivation at UE side. The possible criteria may be the monitoring performance, overheating state, power saving mode, etc.

Proposal 11: The NW configures the criteria (e.g., monitoring performance, overheating state, power saving mode) for the autonomous deactivation of the activated functionality.

### Other procedures for applicable functionality reporting

To enable the flexible request and reporting for the applicable functionality, the NW can configure the request in the RRC Resume and Reestablishment messages. Correspondingly, the UE can indicate the applicable functionality information in the RRC Resume/Reestablishment Complete messages respectively.

Proposal 12: RRC Resume and Reestablishment procedures can be used for request and reporting of the applicable functionality information.

## Signalling for UE-side data collection

In RAN2#127bis meeting, it was agreed that:

- Data collection initiation and configuration for data collection is under network control.  FFS how the NW determines whether data collection should be initiated (e.g. via UE requests (UE directly or UE server)

To assist the data collection for UE-side data training, the NW should provide data collection configuration to the UE. The request of the data collection configuration can be directly initiated by the UE or UE server. The request initiated by the UE server is out of RAN2 scope. RAN2 focus on the data collection initiation and configuration for data collection between UE and NW.

Technically speaking, there may be proactive and reactive schemes under network control for the data collection initiation and configuration for data collection. In the proactive scheme, the UE reports the UE capability of the data collection configuration or preferred data collection configuration(s) to the NW. The NW can proactively provide the related data collection configuration to the UE based on the information from the UE and NW conditions. For example, when the NW is not congested and has available resources, it might provide the data collection configuration to UE. In the reactive scheme, the NW firstly provides a request instruction to the UE. The request instruction can guide the UE whether to initiate the request, or whether to initiate the request relevant to specific use case or functionality.

Proposal 13: For data collection initiation and configuration under NW control for UE-side model training, RAN2 considers the following solutions:

- Opt 1: proactive solution

- UE sends the data collection related capability or preference to the NW;

- NW decides when to provide the corresponding data collection configuration based on the UE capability or preference.

- Opt 2: reactive solution

- NW sends a request instruction to the UE;

- UE sends the request to the NW for data collection configuration based on the request instruction;

- NW provides the corresponding data collection configuration based on the request.

# 3 Conclusion

In this contribution, we discussed the potential solutions on the remaining FFSs and gave more considerations on the applicable functionality reporting schemes. The related observation and proposals are summarized below:

Supported functionality information:

Proposal 1: UE reports the functionality ID of the supported functionalities in UECapablityInformation message.

Proposal 2: The reporting of supported functionality information is feature group specific.

Inference configuration:

Proposal 3: The inference configuration refers to the parameters in the field CSI-ReportConfig for beam management.

Proposal 4: The NW provides multiple inference configurations, e.g., a list of CSI-ReportConfig, in step 3.

Proposal 5: The NW indicates the information of the inference configuration to be used for inference in step 5.

Applicable functionality content and reporting:

Observation: The status for applicable functionality determination depends on the UE processing capability, the amount of functionalities to be checked or other internal implementation.

Proposal 6: For the content of applicable functionality reporting, the following two options can be considered:

- Opt 1: explicit functionality ID of the applicable functionality

- Opt 2: implicit information (e.g., bitmap) mapped to the supported functionality ID

Proposal 7: The following cases can be considered for the applicable functionality checking when determining the content of the RRCReconfigurationComplete message:

- complete checking: UE completes the applicability checking for all functionalities

- partial checking: UE completes the applicability checking for some functionalities

- no checking: UE does not start the checking for any functionality

Proposal 8: The UE includes an indication, e.g., on-going, no checking, in the RRCReconfigurationComplete message for applicable functionality reporting.

Proposal 9: The reporting of the change of the applicable functionality information follows the design on the content of the applicable functionality reporting.

Proposal 10: The UE autonomously deactivates the activated functionality when it becomes non-applicable and reports the non-applicability of the activated functionality to NW.

Proposal 11: The NW configures the criteria (e.g., monitoring performance, overheating state, power saving mode) for the autonomous deactivation of the activated functionality.

Proposal 12: RRC Resume and Reestablishment procedures can be used for request and reporting of the applicable functionality information.

UE-side data collection:

Proposal 13: For data collection initiation and configuration under NW control for UE-side model training, RAN2 considers the following solutions:

- Opt 1: proactive solution

- UE sends the data collection related capability or preference to the NW;

- NW decides when to provide the corresponding data collection configuration based on the UE capability or preference.

- Opt 2: reactive solution

- NW sends a request instruction to the UE;

- UE sends the request to the NW for data collection configuration based on the request instruction;

- NW provides the corresponding data collection configuration based on the request.