3GPP TSG-RAN WG2 Meeting #128	R2-2410082

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:	8.7.3 RRM measurement gaps/restrictions related enhancements

Source:	NEC

Title:	Impacts on DSR and DRX from MG skipping

Document for:	Discussion, Decision

Introduction

On the objective, RRM measurement gaps/restrictions related enchantments, previous RAN2 meeting achieved following agreements:

Agreements for RRM measurement gap skipping - RAN2 assumes that at least some impact on DSR from MG skipping can be avoided by NW implementation. FFS whether there is an impact which would require some specification changes/enhancements. - No need to have delay-aware LCP enhancements specific for MG skipping, i.e. MG skipping and delay-aware LCP are designed as independent features - RAN2 can further evaluate whether there is any impact on DRX from MG skipping. For the moment, the issue is unclear. - RAN2 will focus its work on supporting the solution chosen by RAN1/RAN4

This contribution further discusses impacts on DSR and DRX from MG skipping.

Discussion

Impacts on DSR from MG skipping

Regarding the UE behavior when an occasion(s) of gaps/restrictions that are caused by RRM measurements are cancelled/skipped fully, RAN1#116 and RAN1#118bis meetings achieved following agreements:

Agreement@RAN1#116 From RAN1 perspective, when an occasion(s) of gaps/restrictions that are caused by RRM measurements are cancelled/skipped fully, UE is assumed to receive/transmit in the gaps/restrictions that are caused by RRM measurements as it would without any (measurement etc. related) gaps/restrictions that are caused by RRM measurements. - FFS: Whether or not/How to support of the case where an occasion(s) of gap/restrictions that are caused by RRM measurements are cancelled/skipped partially Agreement@RAN1#118bis From RAN1 perspective, the case where an occasion(s) of gap/restrictions that is caused by RRM measurements is cancelled/skipped partially is not supported in Release 19.

Based on the above RAN1 agreements, when a particular gap/restriction is skipped with an explicit indication by DCI, the skipped MG period can be considered as a period during which the UE receives downlink channels/signalling and transmits uplink channels/signalling. Such a UE behaviour change caused by skipping MG is mainly at lower layer, i.e., physical layer.

When a particular gap/restriction is skipped with an explicit indication by DCI, the skipped MG period can be considered as a period during which the UE receives downlink channels/signalling and transmit uplink channels/signalling.  Such a UE behaviour change caused by skipping MG is mainly at lower layer, i.e., physical layer.

During RAN2#127bis meeting, whether there are impacts on DSR trigger, cancellation from MG skipping was discussed. The DSR triggers (highlighted in yellow) and cancellation (highlighted in green) defined in Clause 5.4.9 [1] are shown below.

If an LCG is configured for delay status reporting, the MAC entity shall for each logical channel within the LCG: 1> if the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel that have not been transmitted in any MAC PDU and have not been reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and 1> if there is no DSR pending for the logical channel: 2> trigger a DSR for the logical channel. If there is at least one DSR pending, the MAC entity shall: 1> if UL-SCH resources are available for a new transmission and the UL-SCH resources can accommodate the DSR MAC CE plus its subheader as a result of logical channel prioritization: 2> instruct the Multiplexing and Assembly procedure to generate the DSR MAC CE as specified in clause 6.1.3.72. 1> else if there is no pending SR already triggered by the DSR procedure for the same logical channel as of this DSR: 2> trigger a Scheduling Request. NOTE 1: The availability of UL-SCH resources for the transmission of the DSR MAC CE follows the same critieria specified in clause 5.4.5. A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and is a delay-critical PDCP SDU (as defined in TS 38.323 [4]) associated with the logical channel which triggered the DSR. A MAC PDU shall contain at most one DSR MAC CE. A MAC PDU shall not contain a DSR MAC CE if it includes all PDCP SDUs associated with all the pending DSRs. After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR. NOTE 2: It is up to UE implementation whether the MAC entity includes a DSR MAC CE in a MAC PDU if the MAC PDU can accommodate all PDCP SDUs associated with all the pending DSRs but is not sufficient to additionally accommodate the DSR MAC CE plus its subheader.

It is obviously that both DSR triggers (highlighted in yellow) and cancellation (highlighted in green) above are based on PDCP layer timer or PDCP layer behaviour (e.g., SDU discard) or MAC layer behaviour which is independent from lower layer behaviour. Therefore in our understanding, there is no impacts on DSR trigger, cancellation from lower layer behaviour change caused from MG skipping.

Another aspect is whether there is impact on remaining time calculation/definition. As shown above, the remaining time is defined as remaining value of the running PDCP discardTimer, that is also independent from lower layer behavior, we understand that there is also no impacts on remaining time calculation/definition as well.

There might be some impacts on DSR configuration, e.g., how to configure a proper remainingTimeThreshold by considering MG skipping. However, as concluded during previous meeting, such impacts can be avoided by NW implementation. We propose RAN2 to confirm that regarding impacts on DSR from MG skipping, there are no impacts requiring specification changes/enhancements.

Regrading impacts on DSR from MG skipping, RAN2 confirm that there are no impacts requiring specification changes/enhancements.

Impacts on DRX from MG skipping

As observed in Observation 1, when a particular gap/restriction is skipped with an explicit indication by DCI, the skipped MG period can be considered as a period during which the UE receives downlink channels/signalling and transmits uplink channels/signalling.

To support UE power saving, currently the PDCCH monitoring activity of the UE in RRC connected mode is governed by DRX, BA, DCP and cell DTX.

Since UE power saving features and measure gap are independent from each other, we observe that during the skipped MG period, the PDCCH monitoring activity of the UE in RRC connected mode is also governed by DRX, BA, DCP and cell DTX.

During the skipped MG period, the PDCCH monitoring activity of the UE in RRC connected mode is also governed by DRX, BA, DCP and cell DTX.

Taking DRX as an example, as specified in Clause 11 [3], when DRX is configured, the UE does not have to continuously monitor PDCCH and the total duration that the UE monitors PDCCH is defined as active-time, which includes the “on-duration” of the DRX cycle, the time UE is performing continuous reception which the inactivity timer has not expired, and the time when the UE is performing continuous reception while waiting for a retransmission opportunity.

Given that both DRX and MG are configured by NW, it is reasonable to assume that NW to configure “on duration” of the DRX cycle and measurement gap without overlap, i.e., during skipped MG period, it may not have active-time driven by “on-duration” of the DRX cycle. However, the intention of skipping MG is to realize system capacity gains while keep impact to mobility performance limited, therefore the intended UEs would be UEs which have either DL data or UL data to transmit. Since when the PDCCH indicates a new transmission (DL or UL), the drx-inacitvityTimer will be started or restarted, we think it is reasonable to assume to have active-time driven by inacitivity timer running (i.e., has not expired) during skipped MG period. Therefore, as per our understanding, regarding impacts on DRX from MG skipping, there are no impacts requiring specification changes/enhancements.

Regrading impacts on DRX from MG skipping, RAN2 confirm that there are no impacts requiring specification changes/enhancements.

Summary

This contribution further discussed impacts on DSR and DRX from MG skipping.

Impacts on DSR from MG skipping

Observation 1	When a particular gap/restriction is skipped with an explicit indication by DCI, the skipped MG period can be considered as a period during which the UE receives downlink channels/signaling and transmit uplink channels/signaling. Such a UE behavior change caused by skipping MG is mainly at lower layer, i.e., physical layer.

Proposal 1: Regrading impacts on DSR from MG skipping, RAN2 confirm that there are no impacts requiring specification changes/enhancements.

Impacts on DRX from MG skipping

Observation 2	During the skipped MG period, the PDCCH monitoring activity of the UE in RRC connected mode is also governed by DRX, BA, DCP and cell DTX.

Proposal 2: Regrading impacts on DRX from MG skipping, RAN2 confirm that there are no impacts requiring specification changes/enhancements.

- References

[1] TS38.321

[2] TR38.835

[3] TS38.300