# 3GPP TSG RAN WG2 Meeting #128      	                        R2-2410103
Orlando, USA, 18 - 22 Nov 2024

Agenda item:	8.1.5

Source:	China Telecom

Title:	Discussion on model transfer/delivery

WID/SID:	NR_AIML_Air-Core

Document for:	Discussion and Decision

# 1 Introduction

In the Table 4.3-1 of TR 38.843 [1], RAN1 identified the following cases for model delivery/transfer.

Table 4.3-1: Model delivery/transfer cases Case; Model delivery/transfer; Model storage location; Training location. y; model delivery (if needed) over-the-top.; Outside 3GPP Network; UE-side / NW-side / neutral site. z1; model transfer in proprietary format.; 3GPP Network; UE-side / neutral site. z2; model transfer in proprietary format.; 3GPP Network; NW-side. z3; model transfer in open format.; 3GPP Network; UE-side / neutral site. z4; model transfer in open format of a known model structure at UE, i.e., an exact model structure as has been previously identified between NW and UE and for which the UE has explicitly indicated its support.; 3GPP Network; NW-side. z5; model transfer in open format of an unknown model structure at UE, i.e., any other model structure not covered in z4, including any model structure that is only partially known.; 3GPP Network; NW-side. Note: The definition of various Cases is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization..

Following the RAN2#128 agenda, this contribution is to provide the comprehensive analysis on model delivery/transfer based on both RAN1 and RAN2 progress.

# 2 Discussion

According to Table 4.3-1 of TR 38.843 [1], model delivery/transfer options could be categorized by training location as follows:

- Models trained on the network (NW) side: case y, z2, z4, and z5.

- Models trained on the UE-side: case y, z1 and z3.

From RAN1 perspective [2-5], cases z2, z3, and z5 have been deprioritized in Rel-19. Additionally, RAN1 has also concluded that the model transfer is needed at least for some (e.g., Option 3b) of inter-vendor training collaboration option(s) of CSI compression using two-sided model (if supported). So we have the following observations.

Observation 1: RAN1 will focus on cases y and z4 for models trained on the NW side, and cases y and z1 for models trained on the UE side.

Observation 2: From RAN1’s perspective, model transfer/delivery is primarily relevant to two-sided models.

Furthermore, based on the conclusions from the RAN1#118 meeting, the following conclusions regarding model transfer/delivery for Case z4 were made.

Conclusion [RAN1#118] From RAN1 perspective, the “known model structure(s)” of the model transfer/delivery Case z4 at least include known information on the following aspects - Model type/backbone (e.g., Transformer, CNN and so on) - In case model type is a neural network - Number of layers - Layer types/structure (e.g., full connected, activation layer and so on) - Layer size (e.g., the number of parameters of a layer) - Connection between different layers - model input/output related information

In our understanding, RAN2 can continue the discussion based on the conclusion from RAN1. So we have the following proposal:

Proposal 1: RAN2 discussion on model transfer/delivery shall focus on a two-sided model with a known model structure.

In Rel-19, RAN1 have analyzed the model transfer/delivery that the case z4 for two-side model is focused. However, there has been no further discussion on cases y and z1, nor any conclusion regarding the need for model transfer/delivery for one-sided models. Based on this, we propose:

Proposal 2: RAN2 should wait for further input from RAN1 on operation of model transfer/delivery for case y and z1.

Proposal 3: RAN2 should wait for further progress from RAN1 on model transfer/delivery for one-sided model.

In the clause 7.2.1.4 of TR 38.843 [1], RAN2 clarified several candidate solutions for model transfer/delivery, including the pro and cons and specification gap for each solution.

7.2.1.4 Model transfer/delivery Whether there is a need to consider standardized solutions for transferring/delivering AI/ML model(s) is unclear from the outcome of the present study. Nonetheless, to support AI/ML model transfer/delivery, the following solutions are considered: - Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling. - Solution 2a: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling. - Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling. - Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 2b: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via User Plane (UP) data. - Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 4a: OTT server can transfer/deliver AI/ML model(s) to UE (e.g., transparent to 3GPP). - Solution 4b: OAM can transfer/deliver AI/ML model(s) to UE.

For one-sided model at the UE, model training is done on the OTT server and the trained model is sent to the UE. There is no need to introduce 3GPP standardized signaling, as the 3GPP transparent solution (option 4a / Case y) is effective and widely adopted in industry. However, for two-sided models, where model training is distributed between the network (gNB/LFM/OAM, etc.) and the UE, the model must be both visible and controllable.

Therefore, we propose:

Proposal 4: For a one-sided model at the UE, there is no requirement for model controllability and visibility.

Proposal 5: For a two-sided model, the model shall be controllable and visible.

# 3 Conclusion

In this contribution, we further discuss AI/ML model transfer/delivery. We have following observations:

Observation 1: RAN1 will focus on cases y and z4 for models trained on the NW side, and cases y and z1 for models trained on the UE side.

Observation 2: From RAN1’s perspective, model transfer/delivery is primarily relevant to two-sided models.

Based on observations, we have following proposals:

Proposal 1: RAN2 discussion on model transfer/delivery shall focus on a two-sided model with a known model structure.

Proposal 2: RAN2 should wait for further input from RAN1 on operation of model transfer/delivery for case y and z1.

Proposal 3: RAN2 should wait for further progress from RAN1 on model transfer/delivery for one-sided model.

Proposal 4: For a one-sided model at the UE, there is no requirement for model controllability and visibility.

Proposal 5: For a two-sided model, the model shall be controllable and visible.

# 4 References

[1] TR 38.843 v2.0.0, Study on Artificial Intelligence (AI) / Machine Learning (ML) for NR air interface, Qualcomm

[2] RAN1#116, Final Report of 3GPP TSG RAN WG1 #116 v1.0.0.

[3] RAN1#116b, Final Report of 3GPP TSG RAN WG1 #116b v1.0.0.

[4] RAN1#118, Session notes for 9.1 (Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface).

[5] RAN1#118b, Session notes for 9.1 (Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface).