3GPP TSG-RAN2 Meeting # 128	R2-2410161

# Orlando, USA,  18th – 22nd November, 2024

Agenda item:	8.16

Source: 	Huawei, HiSilicon

Title: 	Discussion on the support of NavIC L1 SPS

Document for:	Discussion and Decision

1. Introduction

In this paper, we discuss on the potential issues on the support of NavIC L1 SPS [1] in NR and LTE system.

2. Discussion

With the previous RANP decision, we are going to support NavIC L1 SPS signal in NR and LTE in R19 [2]. The main task of standardization in 3GPP will be to deliver the contents of the navigation message for the NavIC L1 SPS signal in the 3GPP system, either by dedicated signaling with LPP, or by broadcast signaling by posSIB.

For the support of content delivery by LPP, since the spec is 37 series inter-RAT spec between E-UTRA and NR, introduction of the support of NavIC L1 SPS in the LPP naturally introduces the support for E-UTRA and NR simultaneously. Similarly for broadcast, the information elements within the posSIB are also defined in the LPP and LPP already defined a framework well enough that the new GNSS signal can be easily added to the posSIB framework without the need to add new posSIBs.

Hence, for the support of the new NavIC signal, all we need to do is to examine the LPP spec for the NavIC specific information elements that are not currently supported for L1 SPS from the ICD document. For the description of the NavIC L1, the following description has been given in [2].

India has now introduced second generation of NavIC satellite services. Compared to the first-generation satellite series, the second-generation satellite series includes L1 navigation band and incorporates indigenously developed rubidium atomic lock. The inclusion of L1 navigation band improves the positioning, navigation, and timing services for civilian users, and facilitates better interoperability with other GNSS services. The NavIC Signal-in-Space ICD (ISRO-IRNSS-ICD-SPS-L1-1.0) for SPS in L1 Frequency was released to the public in August 2023 for providing the essential information on the NavIC signal-in-space, to facilitate research & development and aid the commercial use of the NavIC signals for navigation-based applications. NavIC L1 signal is optimized to enhance overall system performance on NavIC SPS which included introduction of a new ionospheric model, update to Ephemeris representation, update to transformation of TOWC among others.

In the following, we examine the changes that are needed for the introduction of SPS in L1 frequency to LPP spec

| L1 SPS data contents | Descriptions | Required LPP spec support |
| --- | --- | --- |
| URA | URA is a statistical indicator of the ranging accuracies obtained with a specific SV. URA is a one-sigma estimate of the user range errors in the navigation data for the transmitting satellite and its index value ranges from -16 to 15 | Can be added to the new Ephemeris IE |
| Ephemeris | The coordinates of the NavIC satellite’s antenna phase center position in ECEF reference frame shall be computed by the user utilizing the Ephemeris parameters | The new ephemeris is better to be supported by the introduction of a new IE. The L1 SPS ephemeris is quite different from the legacy IE NavModel-NavIC-KeplerianSet |
| Signal delays |  | Can be added to the existing clock model |
| EOP parameters | The Earth Orientation Parameters message provides the users with parameters to construct the ECEF and ECI coordinate transformation | Can be supported by the current spec |
| IGP information | Message type 5 contains the ionosphere grid corrections for grid points over Indian region. The ionospheric delay corrections are broadcasted as vertical delay estimates at specified Ionospheric Grid Points (IGPs), applicable to a signal on L5 for the single frequency users over the Indian land mass. This includes 90 IGPs at 350 km above the Earth surface. | This is new for LPP |
| Almanac parameters | Message Type 6 contains satellite almanac data. The almanac is a subset of the clock and ephemeris data, with reduced precision | Can reuse the AlmanacMidiAlmanacSet field in LPP |
| Ionospheric error correction based on NeQuick Model | Ionospheric error correction based on NeQuick model for single frequency users of NavIC (NeQuick-N) will be computed through a set of 3 coefficients given for 3 regions (Refer Table 17) defined by maximum and minimum MOdified DIP latitude (MODIP) coverage, maximum and minimum longitude coverage, the Ionosphere Disturbance Flag (IDF) for each set and the Issue of Data NeQuick-N (IODN). | The current LPP NeQucik model is not quite the same as that for L1 SPS. Suggest to introduce new IE for this. |
| Klobuchar like Ionosphere Coefficients | Ionospheric error correction for single frequency users of the NavIC will be provided through a set of eight Klobuchar like coefficients. The calculated value of ionosphere correction parameter (Tiono) in the model is referred to the L5 frequency | adding new fields to KlobucharModelParameter |
| NAVIC TO GNSS time/UTC offsets | Message Type 17 contains the parameters required to relate the NavIC time to UTC, UTC (NPLI) and other GNSS viz. GPS, GALILEO and GLONASS | Introduce new IE for the offset between NAVIC L1 SPS and GNSS/UTC. Clarify that only GPS, GALILEO and GLONASS are supported for the GNSS offset |

Based on the discussion above we propose the following:

Proposal: For the support of L1 SPS for NAVIC in LPP, adopt the following changes:

Introduce a new IE for the ephemeris of L1 SPS

Add IGP to the generic assistance data

Introduce new IE for NeQuick model

Introduce new IE for Klobuchar like Ionosphere Coefficients by adding new fields to KlobucharModelParameter

Introduce support for NAVIC time offset with respect to UTC (NPLI), UTC and other GNSS (GPS, Galileo, and GLONASS)

3. Conclusion

In this paper, we discuss the support of L1 SPS in LPP, we propose the following

Proposal: For the support of L1 SPS for NAVIC in LPP, adopt the following changes:

Introduce a new IE for the ephemeris of L1 SPS

Add IGP to the generic assistance data

Introduce new IE for NeQuick model

Introduce new IE for Klobuchar like Ionosphere Coefficients by adding new fields to KlobucharModelParameter

Introduce support for NAVIC time offset with respect to UTC (NPLI), UTC and other GNSS (GPS, Galileo, and GLONASS)

4. References

[1]	NavIC SPS L1 ICD, https://www.isro.gov.in/media_isro/pdf/SateliteNavigation/NavIC_SPS_ICD_L1_final.pdf

[2] RP-242414, Introduction of A-GNSS support for NavIC L1 SPS in NR & LTE