3GPP TSG-RAN WG2 Meeting #128	R2-2410186

Orlando, USA, Nov. 18th – 22th, 2024

Agenda Item:	8.3.1

Source:	OPPO

Title:	Discussion on text proposal

Document for:	Discussion, Decision

Discussion

The text proposal intends to capture the agreements without grey color in RAN2#127bis meeting. The agreements with grey color is either related to simulation result template or subject to post email discussion [POST127bis][022][AI mobility] Simulation Assumptions (OPPO).

Agreements on intra-frequency

- For intra-frequency temporal domain, higher UE speeds result in larger prediction errors

- Initially, increasing the OW length can enhance prediction accuracy in the temporal domain case A, especially when the OW is relatively short. However, once the OW exceeds a certain threshold, further increases do not yield significant benefits. Conversely, for PW, longer durations correlate with decreased prediction accuracy.  RAN2 will not define the actual threshold and fast fading assumption.

- Majority of companies observe that among sub cases 1, 2, and 3, at least with shorter prediction window sub case 2 demonstrates the highest prediction accuracy

- Companies can provide multiple real time RSRP value(s) and/or average RSRP value over the entire window and should indicate in their simulation results what they have used.   The companies should at least provide the results of only one value it should be the last value at the end of the PW.   We will add two columns in the spreadsheet to capture the last value and the average value.

Agreements on inter-frequency

- Companies need to report whether earlier predicted results are also used as inputs for future RRM prediction.

- Companies should report with their simulation the correlation coefficient

- Higher-to-lower and lower-to-higher frequency prediction is comparable

- For co-located scenario,  the UE speed in the inter-frequency case has minor impact on prediction accuracy

Agreements

Measurement event prediction simulations will at least focus on intra-frequency FR2, case A, and second study goal (i.e. HO KPI improvement).   FFS what is KPI.

Companies can bring simulation results for intra-frequency measurement reduction for FR1 and report what they are doing.  Focus on temporal case B.

Companies will prioritize simulations on indirect method.   Companies can bring simulations on direct method and should report what method is being used.

Measurement event prediction results are expected in RAN2#129

Agreements on simulation assumptions

The Simulation assumption of RRM measurement prediction can be reused unless otherwise specified.

Companies can pick and report what they are using for filtering options (similar to RRM prediction)

Companies will focus on sub-case 2 for measurement event prediction.   Companies can simulate other sub-cases if they wish and report what they are using.

Leave the simulation parameter discussion for email discussion.  Pick only one value for A3.

It is up to company implementation how to model UE behavior after A3 event is trigger.  Focus on intermediate KPIs for this exercise.  System level KPI is FFFs

For measurement event prediction, traffic is not simulated.

Agreements on inputs/outputs and KPIs

- For indirect measurement event prediction, the intermediate output (i.e., the output of RRM prediction model) is RSRP of serving/neighboring cells.  The final output is the expected occurrence time of a certain measurement event (ex. event A3).

- For direct measurement event prediction, the model output is the probability of event occurrence within a time window.

- A3 event prediction should follow legacy rules (i.e. the “predicted” conditions have to persist for the duration of TTT).

- As baseline, we will use RLF event prediction KPI:

for indirect: F1 score.  the following can be reported: RSRP difference, missed event detection, false event detection.  FFS how to define F1 score.

time difference of true time event reporting triggered and predicted time event reporting triggered, true event prediction.

for direct: F1 score.  The following can be reported: missed event detection, false event detection,

Continue discussion over email discussion to see if there is a difference.