3GPP TSG-RAN WG2 #128	R2-2410212

Orlando, USA, 18th – 22nd Nov. 2024

Source:                    	DENSO CORPORATION

Title:  	Discussion on DSR enhancements for XR

Document for:        	Discussion and decision

Agenda Item:         	8.7.4.2		DSR enhancements

- Introduction

In RAN #105, revised WID on Rel-19 XR enhancements was approved [1], and the following objective was specified regarding scheduling enhancements.

- Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

And the following agreements related to DSR enhancements were reached in RAN2#127bis meeting.

Agreements on DSR enhancements - We do not change the definition of delay-critical data - For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s) - Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc. - For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: - Buffer size of data volume in each portion - Shortest remaining time among PDCP SDUs buffered in each portion. - There is no need to include PSI in the enhanced DSR MAC CE. - A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. - FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

In this contribution, we provide the discussion on DSR enhancements for XR.

- Discussion

In the previous meeting, RAN2 discussed whether DSR triggering threshold should always be larger than reporting threshold(s). From our perspective, there is no need to limit the threshold setting. If triggering threshold > reporting threshold, it is possible to report the delay information of delay-critical data with finer granularity. Conversely, if triggering threshold < reporting threshold, it is possible to report delay information of non-delay-critical data that may soon become delay-critical. Either information could be useful for the NW, and thus the threshold setting can be left to the NW implementation.

Observation 1:	If trigger threshold < reporting threshold, it is possible to report delay information of non-delay-critical data that may soon become delay-critical, which could also be useful for NW.

Proposal 1:	DSR threshold setting can be left to NW implementation.

If triggering threshold < reporting threshold is allowed (i.e., information of non-delay-critical data can be included in new DSR), the impact on DSR procedure needs to be considered.

In the current specifications, DSR is not triggered again by data that has already been reported as data volume. However, if the DSR reports non-delay-critical data, it is necessary to consider whether another DSR can be triggered when the remaining time of the non-delay-critical data already reported becomes below the triggering threshold. If another DSR is not triggered, frequent DSR reporting can be reduced. On the other hand, this may cause the NW to miss resource allocation for the data which has become delay critical. From this aspect, the following specification change can be considered without affecting Rel-18 DSR. Specifically, even if the data has already been reported in a DSR as non-delay-critical, another DSR can be triggered when its remaining time becomes below the triggering threshold.

TS38.321 5.4.9 If an LCG is configured for delay status reporting, the MAC entity shall for each logical channel within the LCG: 1> if the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel that have not been transmitted in any MAC PDU and have not been reported as delay-critical data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and 1> if there is no DSR pending for the logical channel: 2> trigger a DSR for the logical channel.

Proposal 2:	If non-delay-critical data can be included in new DSR, RAN2 to consider whether another DSR can be triggered when the remaining time of the non-delay-critical data already reported becomes below the triggering threshold.

If non-delay-critical data can be included in DSR, the same buffer size information may be reported more frequently in both BSR and DSR. In this case, the information included in DSR does not need to be reported again in BSR. In our opinion, it would be beneficial to cancel or disable BSR transmissions to avoid the redundant reporting.

Proposal 3:	If non-delay-critical data can be included in new DSR, RAN2 to consider whether to support enhancements to prevent redundant buffer size information being reported in both BSR and the new DSR (e.g., BSR cancellation by DSR or disabling BSR).

FFS also remains as to whether old and new DSR can be configured/used at the same time. Basically, the size of the new DSR is larger than the size of the old DSR because it can include multiple pairs of remaining time and buffer size. Therefore, if UL resources are not sufficient to accommodate the new DSR, it would be better to report the old DSR instead to inform the NW of the delay information as soon as possible. In other words, the new DSR can be used in case there is at least one LCG configured with multiple reporting thresholds and UL resources can accommodate it.

Observation 2:	There could be the cases where UL resources are not sufficient to accommodate new DSR that is larger than Rel-18 DSR. This may delay reporting delay information.

Proposal 4:	New DSR can be used in case there is at least one LCG configured with multiple reporting thresholds and UL resources can accommodate it. Otherwise, Rel-18 DSR can be used.

- Conclusion

In this contribution, we provided the following observations and proposals.

Observation 1:	If trigger threshold < reporting threshold, it is possible to report delay information of non-delay-critical data that may soon become delay-critical, which could also be useful for NW.

Proposal 1:	DSR threshold setting can be left to NW implementation.

Proposal 2:	If non-delay-critical data can be included in new DSR, RAN2 to consider whether another DSR can be triggered when the remaining time of the non-delay-critical data already reported becomes below the triggering threshold.

Proposal 3:	If non-delay-critical data can be included in new DSR, RAN2 to consider whether to support enhancements to prevent redundant buffer size information being reported in both BSR and the new DSR (e.g., BSR cancellation by DSR or disabling BSR).

Observation 2:	There could be the cases where UL resources are not sufficient to accommodate new DSR that is larger than Rel-18 DSR. This may delay reporting delay information.

Proposal 4:	New DSR can be used in case there is at least one LCG configured with multiple reporting thresholds and UL resources can accommodate it. Otherwise, Rel-18 DSR can be used.

- References

[1]	RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3