3GPP TSG RAN WG1 #128          						R2- 2410261

Orlando, US, Nov 18th – 22nd, 2024

Source:	Tejas Networks Ltd.

Title:	Study the A-IoT Random Access Procedure

Agenda item:	8.2.4

Document for:     Discussion and Decision

# Introduction

The A-IoT devices receive Msg0 paging signal from bas station (BS), upon receiving Msg0, multiple A-IoT devices can transmit Msg1 in response to Msg0, which leads to collision. Therefore, slotted aloha-based random access mechanism is studied for the A-IoT scenario. The device 1/2a can backscatter the incident signal at a specific frequency. Since one device is capable of transmitting at one frequency, the BS can allocate multiple subcarriers to multiple devices to receive Msg3 from multiple devices simultaneously, thereby reducing the inventory completion time. The agreements made on the 4-step CB RACH in RAN2 #126 is reproduced below, which shows that the Msg4 is not yet defined.

# A-IoT Frame Structure and Message Signals

Agreements for 4-step CB RACH

1	A-IoT Msg1: the device sends an ID to the reader. ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID).  FFS on ID size

2	A-IoT Msg2: the reader echos the ID received in Msg1. Further information may be included in mgs2 based on RAN1 agreements

3	A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request)

FFS on contention resolution based on step 4

4	A-IoT Msg4: FFS on the need and presence for this

The agreements made in RAN2 #126 [1] on the timing aspects are as follows:

According to RAN2 #126 [3], the device initiates the communication with reader by sending Msg1. However, as per the RAN1 agreements, R2D signal is followed by the D2R signal. Therefore, Msg0 can be transmitted by the reader initially to detect the devices available in the vicinity. We propose to send an ACK/NAK signal in Msg4 from the reader after successful/unsuccessful reception of information transmitted by the device in Msg3.

The timing aspects of the R2D and D2R transmission agreed in the RAN1 #117 [2] meetings are mentioned above. However, the CW transmission time is not included in the above agreement. To ensure the energy availability of the device, the device stores energy from the CW signal initially at the beginning of every slot and then completes the entire communication from Msg0 to Msg4, as shown in Fig. 1.

Fig. 1: The device stores energy from the CW signal at the beginning of every slot.

Fig. 2: Flow diagram of communication from Msg0 to Msg4 between reader and device.

The proposed timing diagram of the communication between reader and device is presented in Figure 2. The CW signal is transmitted at the beginning of every slot to recharge the nodes to ensure their availability for receiving the paging signal Msg0.

Msg4 (R2D): Reader transmits an ACK/NAK signal to the devices

Reader received Msg3 from multiple devices simultaneously that consists of the device ID, device type, etc.  If the reader is able to decode Msg3 successfully for a random ID, it sends ACK signal for that random ID. If the reader is unable to decode Msg3 successfully for a random ID, either it sends NAK signal or do not send any response for that random ID. After receiving ACK, the device never transmits Msg1 in the subsequent slots. If the device receives NAK or do not receive any response, it reattempts to transmit information in the next slot.

Fig. 3: Reader transmits Msg4 in response to Msg3.

Observation 1: We observe that A-IoT Msg4 is essential to send the acknowledgement of Msg3 so that the device can receive an acknowledgement and take decision on reattempting in the next slot.

Proposal 1:

- Step 4: Msg4 (R2D): Reader sends an ACK/NAK signal to the devices

We propose two options:

- Reader sends ACK/NAK signal in Msg4 after successful/unsuccessful reception of information transmitted by the devices in Msg3.

- Reader sends only ACK signal for the devices which the reader successfully decodes Msg3.

Proposal 2: If the device do not receive ACK/NAK or it receive NAK, it will re-attempt to send Msg1 after receiving the next paging signal.

# Random Access of A-IoT for Msg1

Agreement

From RAN1 perspective, at least when a response is expected from multiple devices that are intended to be identified, an A-IoT contention-based access procedure initiated by the reader is used.

Agreement

For A-IoT contention-based access procedure, at least slotted-ALOHA based access is studied.

The agreements made in RAN1 #117 [2] on the access procedure are as follows:

As per the above agreements, slotted-ALOHA is studied for A-IoT as contention-based access procedure, when multiple devices transmit Msg1 in response to the Msg0. According to slotted-ALOHA based access procedure, an ACK/NAK signal should be sent to the device based on successful/unsuccessful reception of the information.

The agreements made in RAN2 #127-bis [3] on TDMA and FDMA for Msg1 are as follows:

From RAN2 perspective, at least for TDMA the device can randomly select one access occasion for A-IoT Msg1 within access occasions provided/assigned by the reader, as the baseline for CBRA.  FFS if this is applicable to FDMA.  Further enhancement option(s) can be considered after more RAN1 progress on TDMA/FDMA.

When multiple devices transmit Mgs1 at the same time in response to Msg0 from the reader, there is a chance of collision. To minimize collision and maximize throughput, the device can use slotted Aloha based random access method to transmit Msg1 frame. The D2R Msg1 transmission time is denoted as T1 which is bounded by [T1_min, T1_max]. T1_min is the Msg1 frame transmission time, which is the minimum time required to transmit Msg1. T1_max = X × T1_min, is the maximum time available to the device to transmit Msg1. After receiving the Msg0, the devices randomly select a time slot t between 1 to X and a frequency f between 1 to Y and transmit Msg1 at that time slot using that frequency. The signal propagation time Tp << T1_min. Thus, T1_min is considered as the slot duration of slotted Aloha. If the device does not get acknowledgement, it randomly choses a time slot from the remaining available slots and retransmit Msg1. The flow diagram of slotted Aloha mechanism to transmit Msg1 is given in Fig. 4. If any device is not able to access the channel to send Msg1 within the time [T1_min, T1_max], it goes to sleep mode and reattempts in the next slot.

Fig. 4: Slotted Aloha-based multiaccess communication.

Observation 2: The maximum time duration for Msg1, considered as T1_max depends on the number of devices available, transmission bandwidth of device, and the receiver bandwidth of the reader which indicates the number of frequency domain resources available to the devices to transmit Msg1.

Proposal 3: RAN1 should define the maximum and minimum time TD2R_min and TD2R_max for transmitting D2R signal Msg1. T1_min = TD2R_min is the Msg1 frame transmission time, which is the minimum time required to transmit Msg1. T1_max = X × T1_min = TD2R_max is the maximum time available to the device to transmit Msg1. T1_min is considered as the slot duration of slotted Aloha.

Proposal 4: The value of X (time domain resources) primarily depends on the total number of devices available in the vicinity of the reader. The value of Y (frequency domain resources) depends on the transmission bandwidth of devices and the receiver bandwidth of the reader.

# Conclusion

This study includes the frame structure and random access procedure of A-IoT. We propose to include the ACK/NAK signal for Msg4 to send acknowledgement of Msg3 to the device. Further, we propose the slotted aloha mechanism to transmit Msg1. The observations and proposals are as follows:

Observation 1: We observe that A-IoT Msg4 is essential to send the acknowledgement of Msg3 so that the device can receive an acknowledgement and take decision on reattempting in the next slot.

Proposal 1:

- Step 4: Msg4 (R2D): Reader sends an ACK/NAK signal to the devices

We propose two options:

- Reader sends ACK/NAK signal in Msg4 after successful/unsuccessful reception of information transmitted by the devices in Msg3.

- Reader sends only ACK signal for the devices which the reader successfully decodes Msg3.

Proposal 2: If the device do not receive ACK/NAK or it receive NAK, it will re-attempt to send Msg1 after receiving the next paging signal.

Observation 2: The maximum time duration for Msg1, considered as T1_max depends on the number of devices available, transmission bandwidth of device, and the receiver bandwidth of the reader which indicates the number of frequency domain resources available to the devices to transmit Msg1.

Proposal 3: RAN1 should define the maximum and minimum time TD2R_min and TD2R_max for transmitting D2R signal Msg1. T1_min = TD2R_min is the Msg1 frame transmission time, which is the minimum time required to transmit Msg1. T1_max = X × T1_min = TD2R_max is the maximum time available to the device to transmit Msg1. T1_min is considered as the slot duration of slotted Aloha.

Proposal 4: The value of X (time domain resources) primarily depends on the total number of devices available in the vicinity of the reader. The value of Y (frequency domain resources) depends on the transmission bandwidth of devices and the receiver bandwidth of the reader.

# 4. References

[1] 3GPP RAN2 Chairman’s Notes, 3GPP TSG RAN2 meeting #126, Fukuoka, Japan, May 20th – 24th, 2024.

[2] 3GPP RAN1 Chairman’s Notes, 3GPP TSG RAN1 meeting #117, Fukuoka, Japan, May 20th – 24th, 2024.

[3] 3GPP RAN2 Chairman’s Notes, 3GPP TSG RAN2 meeting #127-bis, Hefei, China, Oct 14th – 18th, 2024.