3GPP TSG-RAN WG2 Meeting # 128	R2-2410289

# Orlando, USA, 18th – 22nd November 2024

Agenda item:	8.9.4

Source:	Google

Title:	Discussion on PWS for NB-IoT

Document for:	Discussion & Decision

- Introduction

In RAN#127bis meeting, the following agreements were made.

Agreements:

1.	RAN2 confirms the understanding that this WID objective covers all PWS services, including ETWS and CMAS

2.	The support for PWS introduced for NB-IoT NTN can also be made applicable for NB-IoT in TN, if this does not require additional NB-IoT TN specific changes.

3.	For NB-IoT NTN PWS, introduce the following new SIBs, taking the content within the corresponding LTE SIBs as a baseline (but also checking whether we can have some optimization/ skip some unnecessary fields):

- SystemInformationBlockType10-NB for primary ETWS notification;

- SystemInformationBlockType11-NB for secondary ETWS notification;

- SystemInformationBlockType12-NB for CMAS notification.

4.	Add the following PWS indication in direct indication information for NB-IoT (Come back in the next meeting on whether this is also added in Paging-NB):

- etws-Indication;

- cmas-Indication.

5.	Upon receiving the PWS notification from NB-IoT cell, the PWS-capable NB-IoT UE acquires the corresponding PWS message immediately (can come back to clarify further what immediately means)

6.	Clarify in the spec (and at least in 36.300) that for NB-IoT the ETWS, CMAS, PWS requirement may not be met in some scenarios, e.g. when the UE is in eDRX (can come back to further clarify the specific cases)

This paper explores key aspects of supporting PWS for NB-IoT.

- Discussion

### 2.1 PWS message segmentation

It has been agreed that the LTE SIBs for PWS are used as a baseline for designing NB-IoT SIBs for PWS. As shown below, the maximum number of segments for a PWS message transmitted by LTE SIBs is 64.

#### – SystemInformationBlockType12 The IE SystemInformationBlockType12 contains a CMAS notification. SystemInformationBlockType12 information element -- ASN1START SystemInformationBlockType12-r9 ::= SEQUENCE { messageIdentifier-r9 BIT STRING (SIZE (16)), serialNumber-r9 BIT STRING (SIZE (16)), warningMessageSegmentType-r9 ENUMERATED {notLastSegment, lastSegment}, warningMessageSegmentNumber-r9 INTEGER (0..63), warningMessageSegment-r9 OCTET STRING, dataCodingScheme-r9 OCTET STRING (SIZE (1)) OPTIONAL, -- Cond Segment1 lateNonCriticalExtension OCTET STRING OPTIONAL, ..., [[ warningAreaCoordinatesSegment-r15 OCTET STRING OPTIONAL -- Need OR ]] } -- ASN1STOP

As shown below, 3GPP TS 36.331 specifiies that the maximum number of bits for a LTE SIB is 2216 bits.  Thus, the maximum PWS message size supported by LTE is 64 * 2216 bits = 141,824 bits.

## 5.2 System information ### 5.2.1 Introduction #### 5.2.1.1 General <unrelated part omitted> NOTE 1: The physical layer imposes a limit to the maximum size a SIB can take. When DCI format 1C is used the maximum allowed by the physical layer is 1736 bits (217 bytes) while for format 1A the limit is 2216 bits (277 bytes), see TS 36.212 [22] and TS 36.213 [23]. For BL UEs and UEs in CE, the maximum SIB and SI message size is 936 bits, see TS 36.213 [23]. For NB-IoT, the maximum SIB and SI message size is 680 bits, see TS 36.213 [23].

Becaue each NB-IoT SIB cannot exceed 680 bits, if only up to 64 segments are allowed for transmitting a PWS message (same as in LTE),  the maximum PWS message size is reduced to 43,520 bits (64 segments * 680 bits). This is approximately one-third of the maximum PWS message size supported by LTE (141,824 bits).  A government disseminates PWS messages across multiple operators’ networks. If NB-IoT uses the same segmentation limit as LTE, PWS messages exceeding 43,520 bits cannot be transmitted via an NB-IoT network, potentially leaving users on NB-IoT networks uninformed.  To address this, RAN2 should support a similar maximum PWS message size for NB-IoT as for LTE. This would ensure consistent message delivery across different network technologies.

Observation: 	If NB-IoT uses the same segmentation limit as LTE, PWS messages exceeding 43,520 bits cannot be transmitted via an NB-IoT network.

Proposal 1: 	Increase the maximum number of segments supported by NB-IoT for PWS messages to a number larger than 64, e.g., 192. This would enable NB-IoT to carry larger PWS messages, comparable to those supported by LTE.

### 2.2 PWS message reception in RRC_CONNECTED

Unlike LTE, NB-IoT UEs cannot receive broadcast SIBs while operating in the RRC_CONNECTED state. Consequently, to ensure the successful reception of a broadcast PWS message, the network may force NB-IoT devices into the RRC_IDLE state. This transition, however, disrupts ongoing services, impacting user experience.

To ensure seamless PWS reception without service interruption, NB-IoT should support the delivery of PWS messages through dedicated signaling.

Proposal 2: 	Support transmission of PWS messages via dedicated signaling for NB-IoT UEs operating in the RRC_CONNECTED state.

To simplify the implementation, we propose reusing an existing DL RRC message for transmitting PWS messages to NB-IoT UEs in the RRC_CONNECTED state. Given that the PWS message is an upper layer message, the DLInformationTransfer-NB message is a suitable candidate for this purpose.

Proposal 3: 	Utilize the DLInformationTransfer-NB message to transmit PWS messages to NB-IoT UEs operating in the RRC_CONNECTED state.

- Conclusion

We propose the following observation and proposals for RAN2 regarding the transmission of PWS messages via NB-IoT.

Observation: 	If NB-IoT uses the same segmentation limit as LTE, PWS messages exceeding 43,520 bits cannot be transmitted via an NB-IoT network.

Proposal 1: 	Increase the maximum number of segments supported by NB-IoT for PWS messages to a number larger than 64, e.g., 192. This would enable NB-IoT to carry larger PWS messages, comparable to those supported by LTE.

Proposal 2: 	Support transmission of PWS messages via dedicated signaling for NB-IoT UEs operating in the RRC_CONNECTED state.

Proposal 3: 	Utilize the DLInformationTransfer-NB message to transmit PWS messages to NB-IoT UEs operating in the RRC_CONNECTED state.