3GPP TSG RAN WG2 #128	R2-2410314

Orlando, USA, November 18 – 22, 2024

Agenda Item:	8.9.2
Source:	Toyota ITC
Title:	Discussion on Support of Store & Forward

Document for:	Discussion and Decision

# Introduction

The RAN plenary has approved a work item for Internet of Things (IoT) Non-Terrestrial Networks (NTN) [1]. One focus area is the store-and-forward operation, where data can be sent to a satellite for storage if the feeder link is absent, whereof it can off-load the stored data to a ground station when the feeder link is restored.

This document discusses the store-and-forward (S&F) feature in IoT Non-Terrestrial Networks (NTN) from IoT traffic and applications point of view.

# Discussion

The store-and-forward feature lets a satellite operate in two modes: real-time or store-and-forward mode. Real-time mode includes both mobile-originated and mobile-terminated traffic with two-way connections, while store-and-forward mode handles one-way mobile-originated traffic with data transmission from IoT devices to the satellite's storage.

Many contributions have raised concerns about the energy consumption of monitoring paging channels when satellites switch to store-and-forward operating mode. Since there is no incoming mobile-terminated traffic, the User Equipment (UE) should not need to monitor these channels in idle mode. This contribution aims to discuss the concern.

For IoT devices with significant amount of data to send, the store-and-forward feature allows data transmission to satellite’s storage, which is useful for devices with limited storage capacity. In extreme cases, the UE may continuously transmit data and remain in connected mode during the entire store-and-forward period, eliminating the need for paging channel monitoring. The store-and-forward feature is fully utilized in such scenarios. As a result, in these situations, there is no opportunity to conserve energy by skipping paging monitoring.

Observation 1: The store-and-forward feature is fully utilized when the UEs generate more data than they can store, keeping the UEs in connected mode.

When data is transmitted intermittently or in bursts, the UE is transitioned from connected mode to idle mode after data transmission stops. In idle mode, it monitors the paging channel, consuming energy. As transmissions become less frequent, the UE spends more time in idle mode than in connected mode. Ultimately, if no data is sent during store-and-forward mode, energy consumption can be optimized by skipping paging monitoring. A standardized solution for this scenario should at least ensure that paging monitoring continues until all pending mobile-terminated data is delivered.

In [2], it is proposed that when there is no mobile-terminated data for a specific area, the UE should be notified to conserve power by not monitoring paging. Although this approach can help save energy, it is dependent on the area. This means that if there is any mobile-terminated data for the area, all UEs need to wait for the data delivery before they can stop monitoring paging. A paging message may also need several attempts to be successfully received. The coverage area of a satellite can be wide and the user population large, which further adversely impacts the efficiency of this solution.

In [3], it is proposed that the last time the feeder link was available should be provided to the UE for deciding whether to monitor paging. This solution assumes that mobile-terminated data can arrive until the last moment before the feeder link becomes unavailable thereby creating a dependency between data arrival and feeder link availability. The efficiency of this solution depends on the UE decisions. If there are UEs that prematurely stop monitoring paging, the network may keep on paging UEs that do not respond, which unnecessarily consumes network resources and results in loss of mobile-terminated data.

If energy consumption from paging monitoring is a significant concern, the solution should ideally be independent of other UEs, and the UE behavior should be standardized to ensure consistency, e.g., driven by a configuration.

Proposal 1: The skipping of paging monitoring should be independent of other UEs, and the UE behavior should be standardized to ensure consistency and control.

It can be argued that practical use cases do not necessarily involve continuous transmission or complete inactivity, as exemplified above, but analyzing these use cases in detail is complicated by several factors that are beyond the scope of standardization. It should be noted that if paging channel monitoring consumes a significant amount of energy compared to data transmission, it indicates low IoT data transmission rate.

This raises a question: why store and forward minimal data in this manner?

Collecting data and sending it when needed, such as when UE’s storage is getting full, makes more sense. This matches with the first example of transmitting substantial amounts of data, or at least larger blocks of data, and otherwise the UE could be even detached. From the perspective of IoT applications, there should be no difference whether the data is stored in the satellite or within the UE, so long as it can be accommodated within the UE’s own data storage. For example, if an IoT application generates only a single packet during a store-and-forward operating mode, there is no difference when exactly the delay-tolerant data is transmitted, if at all during the store-and-forward operating mode, or even a couple of days later.

It is therefore proposed to agree, for the sake of clarity, that the UE is not required to send data immediately in store-and-forward (or any other) mode when the IoT application generates data. It aims to clarify that the behaviors of radio protocols can be decoupled from the functionalities of IoT applications.

Proposal 2: The UE is not required to send data immediately when the IoT application generates data.

Another traffic-related aspect is the switching of operating mode. Transitioning to store-and-forward mode stops all real-time traffic, while transitioning back to real-time mode allows it. For the real-time UEs, this process is like restarting a node in a cellular system, often causing congestion, especially on contention-based resources, due to simultaneous access by many UEs. Typically, access class barring manages this, but it is slow and signaling-intensive. Switching between operating modes will also occur more often than a node restart, which is a rare event.

To prevent traffic surges during mode switching, a standardized signaling mechanism is proposed to avoid congestion from simultaneous accesses. One simple solution could involve randomization, where real-time UEs access attempts are evenly distributed by delaying them randomly after switching from store-and-forward to real-time mode. Users could, e.g., draw a random number within a specified transmission window, set a timer, and transmit when the timer expires.

Proposal 3: Standardize a signaling mechanism to avoid congestions caused by simultaneous access when transitioning from store-and-forward to real-time operating mode.

# Conclusion

This contribution discusses store-and-forward operation. Observation and proposals are summarized as follows:

Observation 1: The store-and-forward feature is fully utilized when the UEs generate more data than they can store, keeping the UEs in connected mode.

Proposal 1: The skipping of paging monitoring should be independent of other UEs, and the UE behavior should be standardized to ensure consistency and control.

Proposal 2: The UE is not required to send data immediately when the IoT application generates something.

Proposal 3: Standardize a signaling mechanism to avoid congestions caused by simultaneous access when transitioning from store-and-forward to real-time operating mode.

# Reference

- RP-240776, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, March 18-21, 2024.

- R2-2408108, “Further consideration on Store and Forward”, Huawei, HiSilicon, Turkcell, China Southern Power Grid, RAN2#127bis

- R2-2408282, “Discussion on the Store and Forward satellite operation”, HONOR