# 3GPP TSG-RAN WG2 Meeting #128	                                                           R2-2410322

# Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:	8.5.4

Source:	CMCC

Title:	Discussion on SSB adaptation

Document for:	Discussion

Introduction

In the RAN#102 meeting, the new WI of enhancements of network energy savings for NR was set up [1], , one of the objectives is to adapt the common signal/channel transmission, which has been discussed during R18 SI and proved to be beneficial for energy saving, the related objective is as following,

3. Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

In this contribution, we provide our view on SSB adaptation.

Discussion

For SSB adaptation, RAN4 has sent an LS to RAN2 [2]:

RAN4#112bis extensively discussed impact on RRM requirements from SSB adaptation. The following agreements were reached after discussion: Agreements: - RAN4 to discuss whether there are any impacts on at least the following RRM requirements due to SSB adaption: - L1-RSRP for serving cell - L1-SINR - BFD - CBD - SCell activation - TCI state switching - NR intra-frequency L3 measurement - NR inter-frequency L3 measurement However, L3 measurement requirement in RAN4 is defined based on SMTC configuration rather than SSB configuration in legacy. RAN4 would like to ask RAN2 that whether there is any SMTC impact due to SSB adaptation. 2. Actions: To RAN2. ACTION: RAN4 kindly asks RAN2 to answer the SMTC impact question due to SSB adaptation above.

SMTC, SS/PBCH block measurement timing configuration, is to indicated the SSB transmission location in time domain to ensure UE’s measurements. When the SSB is adapted in time domain, the SMTC configuration for the previous SSB configuration may be not valid anymore.

For discussion on SSB adaptation, the following agreements are agreed during RAN1#118[3] and RAN1#118bis meeting [4]:

RAN21#118 meeting Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’)

RAN21#118bis meeting Agreement Conclusion There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19 Agreement For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select Option 3, i.e. Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior). - No spec impact

Based on the agreements in RAN1, SSB adaptation is only applied to RRC_CONNECTED UE. And for UE’s PCell, adaptation of CD-SSB on sync raster is not supported, if only NCD-SSB is adapted while CD-SSB is transmitted as legacy, the NES gain is not clear.

Observation 1: SSB adaptation is only applied to RRC_CONNECTED UE. And for UE’s PCell, adaptation of CD-SSB on sync raster is not supported.

Proposal 1: Do not support SSB adaptation that is only applicable for NCD-SSB/not CD-SSB while CD-SSB is always transmitted in normal periodicity.

It could be seen that, measurement based on adapted SSB is similar to the discussion on OD-SIB1 measurement, if SSB adaptation is mainly applied in SCell and not for RRC_IDLE UE.

Considering that the detail scenario, including whether to support PCell NCD adaptation and to support different UE state is under RAN1 discussion, while OD-SIB1 related L3 measurement frame work is discussed in RAN2, we think RAN2 can postpone the discussion on L3 measurement due to SSB adaptation until RAN1 clarify the applied scenario of SSB adaption. If it is only applied to SCell SSB adaptation, unified solution can be used for OD-SSB and SSB adaptation.

Proposal 2: RAN2 can postpone the discussion on L3 measurement due to SSB adaptation until RAN1 clarify the applied scenario of SSB adaption

Proposal 3: If SSB adaptation is only applied to SCell SSB, unified measurement solution can be used considering OD-SSB and SSB adaptation.

Conclusions

In this contribution, we analyse the adaption SSB, and the following proposals and observation are made:

Observation 1: SSB adaptation is only applied to RRC_CONNECTED UE. And for UE’s PCell, adaptation of CD-SSB on sync raster is not supported.

Proposal 1: Do not support SSB adaptation that is only applicable for NCD-SSB/not CD-SSB while CD-SSB is always transmitted in normal periodicity.

Proposal 2: RAN2 can postpone the discussion on L3 measurement due to SSB adaptation until RAN1 clarify the applied scenario of SSB adaption

Proposal 3: If SSB adaptation is only applied to SCell SSB, unified measurement solution can be used considering OD-SSB and SSB adaptation.

References

- RP-240170 Revised WID for Enhancements of network energy savings for NR; Ericsson;

- R4-2416911; LS on SSB adaptation; Source: RAN4; To: RAN2;

- Report of RAN1#118meeting;

- Report of RAN1#118bis meeting.