3GPP TSG-RAN WG2 Meeting #128	R2-2410434

Orlando, US, 18 – 22 Nov, 2024

Source: 	Huawei, HiSilicon, Turkcell

Title: 	Discussion on Uplink Capacity Enhancements

Agenda Item:	8.8.3

Document for:	Discussion and decision

1	Introduction

The NR NTN WID includes the uplink capacity enhancement as follows [1].

- Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Specify Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH at least for multiplexing 2 or 4 UEs when PUSCH repetitions are used - Specify necessary signalling - Update RF requirements accordingly, if needed - Notes for this objective: - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design

RAN1 discussed this objective for several meetings. In RAN1#118bis, RAN1 has achieved the working assumption of supporting inter-slot OCC for OCC length 2, and for OCC length 4 RAN1 will down select from inter-slot or intra-symbol or a combination of them [2].

Working assumption For the normative phase, - Support OCC length 2 with inter-slot OCC to multiplex up to 2 UEs. - Support OCC length 4 with one of the following OCC techniques - Option 1: Inter-slot with OCC length 4 to multiplex up to 4 UEs. - Option 2: Intra-symbol pre-DFT OCC with OCC length 4 to multiplex up to 4 UEs. - Option 3: Combination of Inter-slot OCC with OCC length 2 and intra-symbol pre-DFT OCC with OCC length 2 to multiplex up to 4 UEs. - Note 1: - At least consider 8 slots, 16 slots, and 20 slots for VoIP with BLER 2% target, with 1 RB, 2 RBs when comparing Option 1, Option 2, and Option 3. Companies can additionally report on 4 slots at least for 2 RBs. - Option 2 assumes TBoMS, FFS Option 3 assumes TBoMS Note 2: as part of the working assumption, it is assumed that there would be separate UE capabilities for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4. Conclusion For TBS calculation and rate matching for OCC with PUSCH, for inter-slot OCC in the working assumption of RAN1#118bis: - for inter-slot OCC for OCC length 2 and for inter-slot OCC for OCC length 4 in option 1 in the working assumption of RAN1#118bis - No change in determination of TBS. - No change for rate matching. Agreement For RV cycling for OCC with PUSCH - For inter-slot OCC for OCC length 2 and for inter-slot OCC for OCC length 4 in option 1 in the working assumption of RAN1#118bis - Same RV value is used in one OCC group (i.e., OCC length applied to N slots). - FFS: RV cycling can be additionally used across OCC groups. Agreement For OCC sequence for OCC with PUSCH: - For OCC length 2, re-use orthogonal sequence [1 1; 1 -1]

In this paper we provide our views on the possible RAN2 impacts.

2	Discussion

In NTN scenario, a satellite usually needs to cover a much larger terrestrial area serving a great number of UEs compared with gNB in terrestrial network. In order to compensate large path-loss suffered by UE in uplink, the maximum number of repetitions for a PUSCH transmission is increased from 16 to 32 in Rel-17. Whereas, the use of repetition increases the transmission time of a UE and is accompanied with high resource utilization in uplink, which will compromise the system capacity. Considering the large number of UEs demanding for uplink transmission and the limited spectrum resources available, it is beneficial to enhance uplink system capacity/throughput.

Orthogonal cover codes (OCC) is proposed to address the issue above. In UL transmission, signals from different UEs can be multiplexed in the same time and frequency domain resource by multiplexing different OCCs on the repeated signals. The orthogonality of OCC sequence can be maintained across multiple UEs, if the power consistence and phase continuity of the repeated signals and channel coefficient are not changed dramatically during the time span of an OCC sequence.

From signaling perspective, the following may need to be configured to the UE:

1) OCC scheme (inter-slot, intra-symbol)

2) OCC length (length 2, length 4)

3) OCC index

On OCC scheme, according to the following working assumption in RAN1#118bis, RAN1 will only consider inter-slot OCC for OCC length 2, and for OCC length 4 RAN1 will down select from inter-slot or intra-symbol or a combination of them. If option 1 is selected, it means RAN1 will only support inter-slot OCC, and OCC scheme does not need to be indicated to the UE.

- Support OCC length 2 with inter-slot OCC to multiplex up to 2 UEs. - Support OCC length 4 with one of the following OCC techniques - Option 1: Inter-slot with OCC length 4 to multiplex up to 4 UEs. - Option 2: Intra-symbol pre-DFT OCC with OCC length 4 to multiplex up to 4 UEs. - Option 3: Combination of Inter-slot OCC with OCC length 2 and intra-symbol pre-DFT OCC with OCC length 2 to multiplex up to 4 UEs.

Observation 1: If RAN1 agrees on Option 1 (inter-slot with OCC length 4 to multiplex up to 4 UEs), OCC scheme does not need to be indicated to the UE, since only one OCC scheme (i.e. inter-slot OCC) will be supported.

On OCC length, we observe that at least OCC length 2 with inter-slot OCC would be supported, and whether to support OCC length 4 depend on RAN1 further discussion. For multiplexing 4 UEs, if option 3 in the above working assumption is adopted, OCC length 4 would be not supported, and there is no need to indicate the OCC length since only OCC length 2 is supported.

Observation 2: If RAN1 agrees on Option 3 (combination of Inter-slot OCC with OCC length 2 and intra-symbol pre-DFT OCC with OCC length 2 to multiplex up to 4 UEs), OCC length does not need to be indicated to the UE, since only OCC length 2 is supported.

Based on the OCC length and OCC scheme, the specific OCC matrix can be generated, and the exact OCC sequence allocated to each UE is represented by each line in the matrix. Therefore, regardless of these options, the OCC index, i.e. the line index of the OCC matrix, should be indicated to the UE anyway.

Observation 3: Regardless of which option is adopted by RAN1, OCC index (i.e. the line index of the OCC matrix) needs to be indicated to the UE.

Proposal 1: At least OCC index (i.e., the line index of the OCC matrix) needs to be indicated to the UE, RAN2 will further consider whether OCC scheme or OCC length is also indicated to the UE depending on RAN1 progress.

The next issue is how to provide/signal the necessary OCC configuration to the UE, considering the UE’s PUSCH transmission can be scheduled by dynamic grant or configured grant:

- For dynamic grant, where the PUSCH transmission is scheduled by DCI, the OCC signaling can be included in DCI and is to be determined by RAN1.

- For configured grant Type 1, where the PUSCH transmission is performed after reception of configuredGrantConfig indicated by RRC signaling, the OCC-related parameters can be included in the rrc-ConfiguredUplinkGrant within configuredGrantConfig.

- For configured grant Type 2, where the PUSCH transmission is activated by a DCI after the reception of RRC signaling including configuredGrantConfig without rrc-ConfiguredUplinkGrant, the OCC-related parameters can be either included in the DCI, or configured by configuredGrantConfig/pusch-config. The down-selection needs to be further discussed by RAN1.

As analyzed above, it is proposed:

Proposal 2: For configured grant Type 1, OCC configuration (i.e. at least OCC index) is provided to the UE via RRC signalling. FFS what OCC configuration should be provided for dynamic grant or configured grant type 2.

As an optional enhancement for Rel-19 NTN UEs, a UE capability needs to be introduced for indicating the OCC support. Since whether the OCC length 4 or intra-symbol OCC are supported is still under discussion, whether separate UE capabilities are used for different OCC length/OCC scheme is pending RAN1 agreement. It is proposed:

Proposal 3: RAN2 introduce UE capability for OCC support. Whether separate UE capabilities are used for different OCC length/OCC scheme is pending RAN1 agreement.

3	Conclusion

This contribution analyses the OCC signalling based on RAN1 progress, and proposes the following:

Observation 1: If RAN1 agrees on Option 1 (inter-slot with OCC length 4 to multiplex up to 4 UEs), OCC scheme does not need to be indicated to the UE, since only one OCC scheme (i.e. inter-slot OCC) will be supported.

Observation 2: If RAN1 agrees on Option 3 (combination of Inter-slot OCC with OCC length 2 and intra-symbol pre-DFT OCC with OCC length 2 to multiplex up to 4 UEs), OCC length does not need to be indicated to the UE, since only OCC length 2 is supported.

Observation 3: Regardless of which option is adopted by RAN1, OCC index (i.e. the line index of the OCC matrix) needs to be indicated to the UE.

Proposal 1: At least OCC index (i.e., the line index of the OCC matrix) needs to be indicated to the UE, RAN2 will further consider whether OCC scheme or OCC length is also indicated to the UE depending on RAN1 progress.

Proposal 2: For configured grant Type 1, OCC configuration (i.e. at least OCC index) is provided to the UE via RRC signalling. FFS what OCC configuration should be provided for dynamic grant or configured grant type 2.

Proposal 3: RAN2 introduce UE capability for OCC support. Whether separate UE capabilities are used for different OCC length/OCC scheme is pending RAN1 agreement.

4	References

- RP-241789, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT

- R1-2409341, Report of RAN1#118bis meeting, ETSI MCC