3GPP TSG-RAN WG2 Meeting #128	R2-2410575

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item:		8.2.3

Source:		Futurewei

Title:		Further discussions on Ambient IoT Paging

Document for:		Discussion and Decision

# Introduction

In meeting #127-bis, RAN2 reached the following agreements regarding the A-IoT paging [1]:

Agreements

- The A-IoT paging message can include information to avoid duplicate response from the device to a reader.  This information should be short and simple.  FFS how to indicate.  Wait for further information and requirements from other WGs and make this decision in normative phase.

- Based on this information the device determines whether to skip sending the response to paging.

- From RAN2 perspective it is feasible to support paging multiple device IDs, however depending on TB size it may not be possible to include multiple device IDs.    RAN2 will determine the need for this after considering aspects related to RAN2 multiplexing and other WGs.

- If multiple device IDs in single paging is supported, if A-IoT paging message contains multiple device IDs, reader can configure either CFRA or CBRA

- For all use cases, device determines the random access type from the paging message.  FFS whether it is explicitly or implicitly.  FFS whether we will down selection for all RA types (2step, 3step, CFRA) or try to unify the design for 2step, 3step

In this contribution, we further discuss information in the A-IoT paging message that can be used by the devices for determining the random access type and for determining their respective availability/unavailability time for energy harvesting.

# Discussions

## Information in A-IoT paging message for determining random access type

In meeting #126, RAN2 agreed on the following [2]:

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion.

For the first case above, where the A-IoT paging message contains an ID of a single A-IoT device, assuming that the A-IoT CN is able to indicate to the reader that the A-IoT Service Request is for a single device, the reader, even if without full visibility to the device ID contained in the A-IoT Service Request (other than that the number of intended device is one), can determine that no contention is needed and hence a CFRA procedure can be and should be used for carrying out the A-IoT service transaction. The reader can schedule radio resource for the CFRA procedure accordingly. Meanwhile, the intended device, having full visibility to the device ID contained in the A-IoT paging message, can determine that a CFRA procedure is to be used, and based thereon, using the scheduled radio resource to respond to the reader in accordance with the CFRA procedure. No additional AS signaling is needed for indicating the type of the random access procedure initiated by the A-IoT paging message in this case. RAN2 can coordinate with other WGs on how to differentiate between a group ID and an individual device ID during the WI phase.

Proposal 1. CFRA procedure is used for carrying out A-IoT service transaction initiated by an A-IoT paging message containing an ID of a single A-IoT device. Other than the number of individual device ID contained in the paging message being one, no additional AS signaling is needed for indicating that the random access procedure initiated by the A-IoT paging message is a CFBR procedure. RAN2 can coordinate with other WGs on how to differentiate between a group ID and an individual device ID during the WI phase.

In the third case above, where no ID is included in the A-IoT paging message, only CBRA procedures should be used. To differentiate between 2-step CBRA procedure and 3-step CBRA procedure, at least one indication bit is needed.

In the second case and the fourth case above, both CFRA and CBRA procedures may be used for carrying out the A-IoT service transactions with the multiple intended devices, in which case an access type indication with up to two bits may be used to differentiate among CFRA, 2-step CBRA, and 3-step CBRA. However, such detailed design can be finalized during the WI phase, with the potential down-selection or other simplifications. In any case, we suggest that a unified design of the access type indication is to be specified for the second, third, and fourth cases above.

Proposal 2. When the A-IoT paging message contains a group ID, multiple IDs, or no ID at all, the reader determines the type of random access procedure to be initiated by the paging message and indicates the random access type in the paging message. The intended devices respond to the A-IoT paging message in accordance with the random access type as indicated.

## Information in A-IoT paging message for controlling availability/unavailability time of devices

In RAN1 meeting #118, RAN1 has reached the following agreements [3]:

Agreement For the study of the potential impact of device unavailability due to charging by energy harvesting, the following directions are captured in TR 38.769: - Direction 1: Reader does not provide information to a device regarding when the device may become available/unavailable. - Direction 2: Reader can provide information to a device based on which the device may become available/unavailable. - Note: The applicability of Direction 1 and/or 2 to different device types 1/2a/2b may be further discussed. - Note: Direction 1 and Direction 2 are not for down-selection.

In Direction 2, the reader provides information in R2D message, based on which the device may become available/unavailable for energy harvesting. For example, if the A-IoT paging message indicates that a device is not an intended device to participate in any access round in the current paging round, then the device can make itself unavailable for communications with the reader, e.g., by transitioning to the OFF state (or the SLEEP state) to harvest energy, until the end of the current paging round. The reader may further include information in the A-IoT paging message to indicate or for unintended devices to estimate how long the current paging round may be, e.g., the number of total access rounds to be scheduled in the current paging round or an estimated duration for the current paging round to be completed.

Proposal 3. If the A-IoT paging message indicates that a device is not an intended device to participate in any access round in the current paging round, the device can make itself unavailable for communications with the reader to harvest energy until the end of the current paging round.

Proposal 4. The reader may further include information in the A-IoT paging message to indicate, or for unintended devices to estimate, how long the current paging round may be, e.g., the number of total access rounds to be scheduled in the current paging round or an estimated duration for the current paging round to be completed.

For another example, if the A-IoT paging message indicates that a device is an intended device to participate in access round #N in the current paging round, then the device makes itself available for communications with the reader (e.g., by being in the ON state) during the period of access round #N while unavailable for communications (e.g., by being in the OFF state or the SLEEP state to harvest energy) during the periods of other access rounds, for which the device is not qualified to contend, until the end of the current paging round. The reader may further include information in the A-IoT paging message to indicate or for the intended device to estimate how long each access round in the current paging round may be so that the intended device may determine its availability time and unavailability time to maximize its ability to harvest energy. When this device wins the contention for an access occasion within access round #N, in Msg3 and/or D2R message(s) that the device sends to the reader in response to command(s), the device includes the energy status indication (ESI) to indicate its predicted energy status at the end of the current paging round, taking into account the energy to be harvested during the remaining unavailability period (e.g., from the end of access round #N to the end of the current paging round). In one of our companion papers submitted to this meeting, we describe, in details, how the ESI may be generated and used and how it may interact with the reader’s controlling of the device’s availability/unavailability time [4].

Proposal 5. The reader may further include information in the A-IoT paging message to indicate or for the intended devices to estimate how long each access round in the current paging round may be so that the intended devices may determine their respective availability time and unavailability time.

Proposal 6. Based on its (estimated) remaining unavailability time, a responding device may determine the ESI to be included in its D2R transmission by considering the energy to be harvested during the remaining unavailability time.

# Conclusions

We propose the following:

Proposal 1. CFRA procedure is used for carrying out A-IoT service transaction initiated by an A-IoT paging message containing an ID of a single A-IoT device. Other than the number of individual device ID contained in the paging message being one, no additional AS signaling is needed for indicating that the random access procedure initiated by the A-IoT paging message is a CFBR procedure. RAN2 can coordinate with other WGs on how to differentiate between a group ID and an individual device ID during the WI phase.

Proposal 2. When the A-IoT paging message contains a group ID, multiple IDs, or no ID at all, the reader determines the type of random access procedure to be initiated by the paging message and indicates the random access type in the paging message. The intended devices respond to the A-IoT paging message in accordance with the random access type as indicated.

Proposal 3. If the A-IoT paging message indicates that a device is not an intended device to participate in any access round in the current paging round, the device can make itself unavailable for communications with the reader to harvest energy until the end of the current paging round.

Proposal 4. The reader may further include information in the A-IoT paging message to indicate, or for unintended devices to estimate, how long the current paging round may be, e.g., the number of total access rounds to be scheduled in the current paging round or an estimated duration for the current paging round to be completed.

Proposal 5. The reader may further include information in the A-IoT paging message to indicate or for the intended devices to estimate how long each access round in the current paging round may be so that the intended devices may determine their respective availability time and unavailability time.

Proposal 6. Based on its (estimated) remaining unavailability time, a responding device may determine the ESI to be included in its D2R transmission by considering the energy to be harvested during the remaining unavailability time.

# References

R2_127bis_ChairNotes_24-10-18_13-00.

R2-2406202, Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan, ETSI MCC.

R1-2407479, Chair notes RAN1#118 (9.4 R19 Ambient IoT) v08 (eom), Ad-Hoc Chair (Huawei).

R2-2410572, Discussions on energy status indication for Ambient IoT, Futurewei.