3GPP TSG RAN WG2 #128			R2-2410602

Orlando, US, November 18th – 22th, 2024

Source:	NTT DOCOMO, INC.

Title:	Discussion on on-demand SIB1

Agenda Item:	8.5.3

Document for: 	Discussion and Decision

- Introduction

At the RAN#105 meeting, the new work item of “Enhancement of network energy savings for NR” was agreed [1]. In this contribution, we provide our views on on-demand SIB1 for idle/inactive mode UEs.

Agreements on OD-SIB1@RAN2#127

Scenarios:

- No consensus for RAN1 case 3 in RAN2.

OD-SIB1 Validity:

- We can rely on legacy UE behaviour to ensure UE has valid SIB1 for the cell (i.e. UE reacquire SIB1 whenever (re)selecting cell).

- Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement).

WUS configuration of NES cell from NES cell:

- Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

MSG3 based OD-SIB1 REQ:

- Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell).

Access restriction for legacy UEs:

- Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

- Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

- Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

- Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

Impacts to RRC connected UEs:

- RAN2 understands the NW can avoid impact on legacy RRC connected UE and R19 RRC connected UE due to on-demand SIB1 (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell).

OD-SIB1 SI conclusion:

- RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1.

Agreements on OD-SIB1@RAN2#127bis

- We will inherit all agreements made during SI phase to WI phase.

- A NES cell can include neighbouring NES cell’s WUS configuration.

- NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

- In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1.

- The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred.

- The legacy UE behaviour can be reused upon on-demand SIB1 acquisition failure, i.e., the NES UE should follow the intraFreqReselection in MIB of NES cell.

- A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

- Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.

- NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

- RAN2 will not start the discussion on the issue when the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, unless we’re asked to do that by other WG, e.g. RAN3.

- Discussion

2.1	Behavior on NES cell

Before a UE request SIB1 to a cell, the UE needs to check if SIB1 is currently being broadcasted or provided on demand, as agreed in the previous meeting [2]. This agreement should be clarified at least for the following two issues: (issue 1) whether legacy UE can use a NES cell where SIB1 is broadcasted (Note: a NES cell means a cell for which UL WUS configuration is available in this context) and (issue 2) how a UE identifies if the cell transmits SIB1 in broadcast mode or on-demand mode.

On issue (1), We would like to note that, regardless of whether legacy UEs can camp on the NES cell with broadcasted SIB1, it should be up to gNB implementation to switch NES cell (including the case where SIB1 is on-demand or broadcasted) to legacy/normal cell that legacy UEs can camp on. In our understanding, such operation can be implemented in a spec transparent manner, and spec never prevent such operation.

Proposal 1:

- Clarify whether or not legacy UE can use (camp on) a cell with SIB1 currently being broadcasted for which SIB1 request (UL WUS) configuration is available.

  - Note: Regardless of the above discussion, it should be up to gNB implementation to switch NES cell (including the case where SIB1 is on-demand or broadcasted) to legacy/normal cell that legacy UEs can camp on.

On top of broadcast of SIB1 on NES cell, we understand that a cell for which SIB1 request configuration is available, can periodically broadcast any other SI. From operators’ perspective, NW should have flexibility to broadcast other SI as well according to situation (e.g., considering the amount of request to SI)

Proposal 2:

- A cell for which SIB1 request (UL WUS) configuration is available can periodically broadcast any other SI message(s) in addition to SIB1.

On issue (2), according to RAN1 agreement, on-demand SIB1 is expected to be transmitted within a certain time window after being requested. In other words, there will also be some duration when SIB1 in on-demand mode would not be transmitted, e.g., outside a configured time window for monitoring on-demand SIB1. Therefore, checking if SIB1 is transmitted in some duration (like LBT) can be a possible way for UE to identify if SIB1 is on-demand mode or broadcast mode.

Another way could be to introduce an explicit signaling via PBCH/MIB or SIB1. It is now discussed in RAN1 whether kssb in PBCH/MIB on NES can be used as NES cell identification. The NES cell identification can be a possible indication in order for a UE to determine whether to transmit UL WUS after the UE obtains UL WUS configuration. Whether to use NES cell identification for this purpose seems to depend on whether legacy UEs can camp on a NES cell with broadcasted SIB1 (mentioned in proposal1) and kssb in PBCH/MIB is used to bar legacy UEs. It is also workable to introduce new RRC parameters (e.g., “broadcasting” or “notBroadcasting”) in SIB1, similar to the legacy parameters for other SI(s).

Proposal 3:

- For identifying whether SIB1 is currently being broadcasted or provided on demand on the NES cell, the following mechanism should be considered:

  - Alt1. New/repurposed parameters in PBCH/MIB

    - E.g., NES cell identification discussed in RAN1

  - Alt2. New parameters in SIB1

    - E.g., “broadcasting” or “notBroadcasting”

  - Alt3. UE check if SIB1 is periodically transmitted in some duration (like LBT)

2.2	UL WUS configuration

As captured in the agreement made at previous meeting, it can be discussed whether RRCRelease message includes UL WUS configuration. Based on the discussion so far, UE requests SIB1 on NES cell based on a criteria of cell reselection. In the legacy spec, there are dedicated parameters related to cell reselection (e.g., frequency priority), so we think it is beneficial that NW can provide UL WUS configuration which can possibly include UE-dedicated parameters that may be determined based on the legacy UE-dedicated cell reselection parameters via RRCRelease message.

Proposal 4:

- UL WUS configuration can be included in the RRCRelease message.

According to the agreements, UL WUS configuration being available to NES cell#1 can be provided by new SIB from any cell A or other NES cells (including NES cell#1 for updating the WUS config.). And in our understanding, one new SIB signaling from one cell can contain multiple WUS configs corresponding to multiple NES cells. Based on the design of WUS config., it is expected that one “WUS config set”, comprising multiple WUS configs that can trigger each corresponding NES cell, can be common across some area. In the legacy SI framework, a UE checks if the stored SI is valid or not by comparing the parameters of “areascope” and “valuetag” in the stored SI and ones provided by SIB1. In our current view, basically the legacy SIB validity rule would work even for UL WUS configuration provided by new SIB. At the same time, it is worth studying if the legacy validity duration of SI (e.g., 3 hours) is enough or not because it is very important factor to deploy OD-SIB1 NES cell operation in real field.

Proposal 5:

- Study whether to enhance or follow the legacy SIB validity for UL WUS configuration provided by new SIB.

- Conclusion

In this contribution, we provided the following observations and proposals of on-demand SIB1 operation for idle/inactive mode UEs network energy saving.

Proposal 1:

- Clarify whether or not legacy UE can use (camp on) a cell with SIB1 currently being broadcasted for which SIB1 request (UL WUS) configuration is available.

  - Note: Regardless of the above discussion, it should be up to gNB implementation to switch NES cell (including the case where SIB1 is on-demand or broadcasted) to legacy/normal cell that legacy UEs can camp on.

Proposal 2:

- A cell for which SIB1 request (UL WUS) configuration is available can periodically broadcast any other SI message(s) in addition to SIB1.

Proposal 3:

- For identifying whether SIB1 is currently being broadcasted or provided on demand on the NES cell, the following mechanism should be considered:

  - Alt1. New/repurposed parameters in PBCH/MIB

    - E.g., NES cell identification discussed in RAN1

  - Alt2. New parameters in SIB1

    - E.g., “broadcasting” or “notBroadcasting”

  - Alt3. UE check if SIB1 is periodically transmitted in some duration (like LBT)

Proposal 4:

- UL WUS configuration can be included in the RRCRelease message.

Proposal 5:

- Study whether to enhance or follow the legacy SIB validity for UL WUS configuration provided by new SIB.

References

- RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson

- 3GPP, RAN2#127bis meeting, Chairman’s notes