3GPP TSG-RAN WG2 Meeting #128	R2-2410626

Orlando, USA, Nov. 18th – 22nd , 2024

Agenda item:	8.1.2.2

Title:	On LCM for UE-sided model for Beam Management

Source:	ZTE Corporation

Document for:	Discussion and Decision

Introduction

In RAN2#127bis[1], following agreement were reached.

Agreements for BM

- UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality.   We should aim to align the design on how the applicable functionality are signaled.   FFS on the applicability reporting content.

- FFS if inference configuration can be signalled in step3.

- UE can report to the network when an applicable AI functionality becomes non-applicable.  FFS how this is signaled (e.g. explicitly/implicitly).  Consider different scenarios, whether it is regarding an active functionality)

- Data collection initiation and configuration for data collection is under network control.  FFS how the NW determines whether data collection should be initiated (e.g. via UE requests (UE directly or UE server)

- For the purpose of discussion of AI/ML BM LCM operations, existing procedures and terminologies from the CSI Framework should be used, including those defined for aperiodic, semipersistent on PUCCH, semipersistent on PUSCH, and periodic reporting configurations (as/if defined in RAN1 pending response LS from RAN1).

- For now, RAN2 will not define terminology specific to the activation or deactivation for AI/ML models.  Can come back to this discussion later.

This contribution provides considerations on the FFS issues.

Discussion

Signaling of functionality applicability reporting

It was agreed that both UAI and RRCReconfigurationComplete message can be used to report applicability functionality. In addition, we think RRCResume/RRCResumeComplete message can also be used for applicability reporting which can bring up some benefits to quick activation AI/ML based feature.

During the RRC resume procedure, network can request applicability reporting based on UE capability. The UE can response in RRCResumeComplete message immediately. This procedure is similar to RRCReconfiguration/RRCReconfigurationComplete. To report applicable functionality in RRCResumeComplete is beneficial to have fast configuration and activation of AI/ML functionalities during RRC resume procedure.

One difference between RRCResume and RRCReconfiguration message is that there is no otherConfig IE in RRCResume message. But the network can transmit another RRCReconfiguration which includes otherConfig if the network wants the UE to do UAI reporting.

Proposal 1: RRCResume/RRCResumeComplete can be used to report applicable functionalities.

On the FFS issue that whether inference configuration can be signaled in step3, and FFS on the applicability reporting content, we think these issues is under RAN1’s ongoing discussion about the content of step 3 and step 4.

There are 3 options under discussion in RAN1[2]. Following are the summary of these options:

- Option 1:In Step 3, NW configures one or more CSI-ReportConfig for inference configuration, where the associated ID may be configured in CSI framework as working assumption applied. In Step 4, UE reports applicability(ies) of the above CSI-ReportConfig.

- Option 2:In Step 3, NW configures one set or multiple sets of inference related parameters. And FFS on the set of inference related parameters. In Step 4, UE reports applicability of the above one or multiple sets of inference related parameters, where the associated ID information may be associated.

- Option 3:In Step 3, The associated ID(s) may be provided to UE, e.g., a new RRC parameter. In Step 4, UE reports by UAI which may include the applicable one or multiple sets of inference related parameters and the associated ID(s).

As we can see, there is no consensus among three options on what to be included in step 3 and step 4. So we suggest RAN2 hold these discussion and wait for RAN1’s input.

Proposal 2: RAN2 to wait for RAN1’s input before decide whether inference configuration can be signaled in step 3 and applicability reporting content.

On Reporting for applicability change

Regarding to reporting for applicability change of the functionality, it was agreed that UE can report to network when an applicable functionality becomes non-applicable.

First of all, we understand the reporting for applicability change is only applied to functionalities that has been reported as applicable to network. The applicable functionality reporting can be either initial reporting (i.e. step 4) or proactive reporting via UAI message. In either case, the UE can report to network in case applicable functionality becomes non-applicable.

Proposal 3: RAN2 to confirm that reporting for applicability change of a functionality is triggered if the functionality that has reported as applicable to the NW before.

On whether to signal non-applicable functionality explicitly or implicitly in applicability change reporting:

As proposed by companies in last meeting, in explicit signaling, the information of non-applicable functionality is included in the reporting. While in implicit signaling, the reporting only includes the information of applicable functionalities. The network deduces which functionality becomes non-applicable by the presence in the reporting. E.g. a former applicable functionality is not present in the reporting, the network regards it as non-applicable.

We think which kind of signaling is adopted depends on whether additional information of the non-applicable functionality is needed. The additional information may be the cause of why applicable functionality beacome non-applicable. For example, the UE may indicate training data is needed for the functionality reported as non-applicability in the applicability change reporting due to some reason, for example, the model need to be refined.

If such additional information is adopted by RAN2, it is naturally to adopt explicit signaling. Otherwise, both explicit or implicit are workable.

Proposal 4: If additional information is needed for the functionality that is reported as non-applicable , e.g. to include the cause of non-applicable, explicit signaling of the non-applicable functionality is adopt in the reporting.

The reason that an applicable functionality becomes inapplicable may include UE side additional condition changes, and UE side hardware capacity (battery capacity, memory...) changes. These changes can happen for both active and deactivated functionalities. Thus both active and deactivated functionality may become inapplicable after reporting applicable.

Observation 1: Both active and deactivated functionality may become inapplicable if UE side additional condition and UE side hardware capacity changes after applicability reporting.

For active functionality, it is natural for the UE to report non-applicable to network. Thus the network can deactivate the functionality to save UE battery/memory, or to avoid performance degradation.

For deactivated functionality, we think it is also meaningful to report non-applicable. Otherwise, the network may activate the functionality later because the network still think the functionality is applicable. If UE reports non-applicable for such functionality, the network may reconfigure the UE and avoid activate the functionality.

Proposal 5: The reporting for applicability change shall be triggered when either active or deactivated functionality becomes inapplicable.

Data Collection and model training

On the request for resources used data collection for model training of UE side model, RAN2 has agreed that data collection initiation and configuration for data collection is under network control.

On which network entity initiates the data collection procedure, or initiate the request to network for resources used for data collection, there are 2 options on the table.

- Option 1: UE sends message directly to serving gNB to trigger configuration for data collection for model training.

- Option 2: UE server request to network to initiate configuration for data collection for model training.

- In our understanding, the data collection configuration is per UE configuration. Thus it is more preferred the request is initiated by the UE directly. Thus the serving gNB can configure proper configuration according to UE’s capability. Further whether the request can be satisfied also depends on the resource usage of each individual UE’s serving gNB. So it is more straightforward to adopt option 1.

- And even if UE server wants to be part of this process, it can communicate with UE and trigger UE to initiate request. The communication between UE server and UE should be transparent to 3GPP.

Proposal 6: RAN2 to adopt option 1, i.e. UE sends RRC message directly to serving gNB to request configuration for data collection for UE side model training.

- On the network control on the initiation and configuration of data collection:

The network should be able to indicate whether UE is allowed to initiate the request for data collection.

Considering transmission reference signals used for data collection to UE consume network resources. Whether the request can be satisfied depends on current available network resources. Thus the network should be able to decide whether to accept the UE request.

Some companies proposed that UE can indicate the time duration for data collection. Similarly, the network should be able to decide when to start and stop the transmission of reference signals for data collection.

Proposal 7: RAN 2 to consider following methods for network control of the initiation and configuration for data collection:

- The network can configure whether UE is allowed to initiate request for data collection.

- The network can decide whether to accept UE’s request for data collection.

- The network can decide when to start/stop the configuration for data collection.

  - Applicability reporting during mobility aspect

There are 3 options regarding how to handle applicability reporting and inference configuration during mobility.

Option 1: Target cell requests applicability reporting during or after handover.

Figure 1: mobility scheme option 1

As illustrated in figure 1, target cell includes information for applicability reporting, e.g. functionality information, and network-side additional condition of the target cell in the RRCReconfiguration message prepared for the UE. The UE reports applicability of the functionality either via RRCReconfigurationComplete or UAI message after it accesses the target cell. Then the target cell can configure/activate the functionality accordingly.

This option is straightforward without additional impact on specification. So it can be regarded as a baseline solution.

Option 2: Source cell transfers source cell’s applicability information to target cell to assist configuration in target cell.

Figure 2: mobility scheme option 2

The source cell transfers the applicability information gathered for source cell to the target cell via handover preparation procedure. The applicability information includes applicable network-side additional conditions of the source cell.

Based on the source cell’s applicability information, the target cell deduces whether the functionality is also applicable in target cell by comparing the network-side additional condition of source cell and the target cell. Although network-side additional conditions, i.e. Associated ID, is unique only within serving cell level, this option is still feasible if by implementation source cell and neighbor cell have the knowledge on whether network-side additional condition represented by source cell’s Associated ID and the target cell’s Associated ID are the same.

The advantage of this option is that the target cell can configure and activate functionality in the handover command. And the applicability reporting procedure in the target cell may be saved.

Note the option 2 is also applicable to the mobility schemes that source cell prepares multiple candidate configurations in advance, e.g. LTM and CHO. The source can transfer source cell’s applicable network-side additional conditions to candidate cells, and candidate cell may prepare candidate configuration including inference configuration. Thus when UE is switched to target cell, the corresponding inference configuration for the functionality can be activated without additional signaling procedure between UE and target cell.

Option 3: Source cell requests applicability reporting for target cell with target cell’s network-side additional conditions.

Figure 3: mobility scheme option 3

In this option, the source cell requests functionality applicability for the target cell before handover occurs.

Althougth it is still FFS in RAN1 on whether the assumption on Associated ID can be applicable for multiple cells, i.e. multiple cell level Associated ID, the network side additional of source cell and target cell may still be different. So it is meaningful to discuss this option no matter the Associated ID is cell level or multiple cell level.

Before applicability reporting, RAN3 signaling is needed to exchange network-side additional conditions between neighbor cells.

The source cell request the applicability with target cell’s network-side additional condition(s), in the same way as request applicability for source cell’s network-side additional condition.

Then the source can transfer the gathered applicability information to target cell during handover request procedure.

The target cell prepares the configuration based on the applicability information and sends the configuration to UE via RRCReconfiguration message.

Note that this solution is also useful for CHO and LTM where candidate configuration is prepared in advance. And the same benefit as in option 2 can be achieved.

Proposal 8: RAN2 to consider following options regarding the applicability reporting and inference configuration during mobility:

- Option 1: Target cell requests applicability reporting during or after handover.

- Option 2: Source cell transfers source cell’s applicability information to target cell to assist inference configuration in target cell.

- Option 3: Source cell requests applicability reporting for target cell with target cell’s network-side additional conditions.

If option 3 is adopted, RAN2 needs to decides whether UAI message can be used to convey applicability reporting for a functionality in neighbor cell. In option 3, the functionality information, neighbor cell’s network side additional conditions, and part or full inference configuration may be provided to UE in step 3. Applicability reporting for the neighbor cell will be conveyed in UAI message. We don’t think it is a problem to use UAI for applicability reporting for neighbor cell.

Proposal 9: If option 3 is adopted, RAN2 to decide whether UAI message can be used to convey applicability reporting for neighbor cell.

Conclusion

On Signaling of functionality applicability reporting

Proposal 1: RRCResume/RRCResumeComplete can be used to report applicable functionalities.

Proposal 2: RAN2 to wait for RAN1’s input before decide whether inference configuration can be signaled in step 3 and applicability reporting content.

On reporting for applicability change

Proposal 3: RAN2 to confirm that reporting for applicability change of a functionality is triggered if the functionality that has reported as applicable to the NW before.

Proposal 4: If additional information is needed for the functionality that is reported as non-applicable , e.g. to include the cause of non-applicable, explicit signaling of the non-applicable functionality is adopt in the reporting.

Observation 1: Both active and deactivated functionality may become inapplicable if UE side additional condition and UE side hardware capacity changes after applicability reporting.

Proposal 5: The reporting for applicability change shall be triggered when either active or deactivated functionality becomes inapplicable.

Data Collection and model training

Proposal 6: RAN2 to adopt option 1, i.e. UE sends RRC message directly to serving gNB to request configuration for data collection for UE side model training.

Proposal 7: RAN 2 to consider following methods for network control of the initiation and configuration for data collection:

- The network can configure whether UE is allowed to initiate request for data collection.

- The network can decide whether to accept UE’s request for data collection.

- The network can decide when to start/stop the configuration for data collection.

Applicability reporting during mobility aspect

Proposal 8: RAN2 to consider following options regarding the applicability reporting and inference configuration during mobility:

- Option 1: Target cell requests applicability reporting during or after handover.

- Option 2: Source cell transfers source cell’s applicability information to target cell to assist inference configuration in target cell.

- Option 3: Source cell requests applicability reporting for target cell with target cell’s network-side additional conditions.

Proposal 9: If option 3 is adopted, RAN2 to decide whether UAI message can be used to convey applicability reporting for neighbor cell.

References

- RAN2#127bis chairman notes

- R1-2409222 Session notes for 9.1 (Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface), Ad-hoc Chair (CMCC), RAN1#118bis