3GPP RAN WG2 Meeting #128	R2-2410685

# Orlando, USA, Nov 18th – 22nd, 2024

Agenda Item:	8.7.5

Source:	HONOR

Title:	Discussion on RLC enhancements

Document for:	Discussion and Decision

1. Introduction

In RAN2#127bis, the following agreements were agreed for enhancing RLC retransmissions [1]:

Agreements on RLC timely retransmissions - RAN2 confirm that existing mechanisms are insufficient to resolve the timely RLC retransmission problem and RLC enhancements for timely RLC retransmission are investigated in Rel-19. - Exclude enhanced status reporting. - Focus the discussion on autonomous retransmission and polling enhancements, e.g. we need to understand how each option affects the capacity and packet delay Agreements on avoiding unnecessary retransmissions - RAN2 confirm the previous baseline assumption: the RLC receiving window always advances to any given RLC SN before the transmitting window does. - RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e. - TX side stops transmissions of an outdated SDU - RX side abandons the SDU based on a local timer - Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified - FFS if some C-PDU handling is needed to avoid C-PDU discard - FFS if some indication is sent from Tx to Rx. The assumption is this is not a full status report, but something simple (if needed)

In this contribution, we discuss the potential solutions related to RLC enhancements.

2. Discussion

2.1 Avoiding unnecessary RLC retransmissions

How the TX side determines the outdated SDU

In the current specification, the SDU discarded mechanism is as following:

TS38.323 v18.3.0 5.3 SDU discard If the corresponding PDCP Data PDU has already been submitted to lower layers, the discard is indicated to lower layers. TS38.322 v18.1.0 5.4 SDU discard procedures When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU.

If the discarded indication of a RLC SDU is received from PDCP, the RLC will not discard the RLC SDU or a segment thereof what has been submitted to the lower layers for avoiding SN gap. However, the resources will be wasted to retransmit the outdated SDUs. Therefore, RAN2 discusses the enhancements on avoiding unnecessary RLC retransmission. It is agreed that TX side stops transmissions of an outdated SDU. The current indication can be reused for the TX side to determine which SDUs are outdated. When receiving the discarded indication, the RLC discards the SDU regardless of whether it is submitted to lower layer. To differentiate from existing handling, the network configures an enabled or disabled indication for discarding.

Proposal 1: The Network configures an indication to enable the RLC to stop transmissions of an outdated SDU notified by the PDCP regardless of whether the SDU has been submitted to the lower layer.

How the RX side determines the outdated SDU by the local timer

In the last meeting, it was agreed to use local timer to determine abandoned SDU in RX side. In the current procedure, there is a reordering timer in PDCP when the reordering submission is configured. However, this timer is only applied for reordering submission, which is not suitable for the RLC to determine the abandoned SDUs. Therefore, it is needed to introduce a new timer in the RLC for abandoning the SDUs.

There is a state variable RX_NEXT to hold the lowest value of the SN of incompletely received RLC SDU in the RX window. If an SDU with SNX lager than RX_NEXT has been received, the SN gap occurs in RX window. In such scenario, the SDUs with the SN in the SN gap may still retransmit in the lower layer. The new timer can be started associated with SNx of received SDU in the RX window. When the new timer expires, the SDUs with the SN smaller than SNX can be abandoned for the large duration to transmit a complete SDU.

Proposal 2: Introduce a new timer to abandon SDUs in the RLC RX side:

- When there is a SN gap in RX side, start the new timer and associate the SN of the received SDU with the new timer.

- When the new timer expires, abandon the incompletely received SDUs with the SN smaller than or equal to the SN associated with the new timer within the RX window.

The combination of TX side and RX side

In the last meeting, RAN2 applied a “combined” approach for avoiding unnecessary RLC retransmissions. It seems that the TX side and RX side abandon the SDUs independently. However, there are some issues for unsynchronized discarding between the TX side and the RX side.

For the first scenario, the RX side abandons an SDU, but the TX side still maintains that SDU, it may be confused that whether the TX side will retransmit the SDU. In the following, if an SR is triggered by some conditions (current one or the new one), the abandoned SDUs may be marked with “dropped” or “ACK” in the SR. According to current specification, the SDU will not be retransmitted if it is assumed as “ACK”. If an SDU is marked as "dropped", it should not be retransmitted regardless of whether the TX side has discarded it. This makes the TX side and the RX side have the same understanding of the SDU, such as this SDU is in ‘non-retransmitted’ state. Therefore, if an SDU is abandoned in RX side, it will not be retransmitted in TX side. And this can be achieved by the “ACK” indication in the SR.

Proposal 3: When the RX side informs the Tx side about the abandoned SDUs, the RX side indicates the abandoned SDUs as “ACK” in the SR, and no need to introduce a new “dropped” state.

Proposal 4: The TX side will not retransmit the SDU which is indicated as “ACK” in the SR, same as the existing handling for the SR.

In the last meeting, RAN2 confirms that the RLC receiving window always advances to any given RLC SN before the transmitting window does. For the first scenario, with the P1 and P2, there is no issue for the RX side to move the RX window based on the abandoned SDUs. Even so, it is needed to confirm that RX side can update the RX window based on the abandoned SDUs before informing the TX side about the abandoned SDUs.

Proposal 5: The RX side can update the RX window based on the abandoned SDUs before informing the TX side about the abandoned SDUs.

For timely advancing TX side window based abandoned SDUs, there may be a solution that the RX side immediately triggers the SR if the RX side has abandoned the SDUs. However, this may cause the earlier triggering of SR, and further the earlier RLF. Current SR triggering based on t-Reassembly and polling works well for the RX side to inform abandoned SDUs.

Proposal 6: No need to introduce the new SR triggering condition for the Rx side to inform Tx side about the abandoned SDUs.

For another scenario, the SDUs marked as “NACK” in the SR may be discarded in TX side. Since the RX side have no idea about this, these SDUs will be maintained in TX side window and RX side window. If the PDU set is discarded, TX side and RX side may keep multiple unnecessary SDUs in the window. Based on the local timer for abandoned SDUs to move RX side window is insufficient for scenario of PDU set discarded. For timely moving the RX side window and the TX side window, TX side can inform RX side about discarded SDUs when the discarded SDU is associated with “NACK” indication in the SR.

Proposal 7: The TX side informs the RX side about discarded SDUs when the discarded SDU is associated with “NACK” indication in the SR.

2.2 Timely RLC retransmission

In the RAN2#127bis meeting, it was agreed to focus the discussion on autonomous retransmission and polling enhancements, each of which has its own pros and cons.

Retransmission based on enhanced polling introduces the smaller values (e.g., t-PollRetransmit, pollbyte, pollPDU) or new condition(s) considering delay-critical data to trigger poll frequently, which essence is to request status reports timely. This enhancement is easy to implement and has less impact on the standard. However, the gain from reducing packet delay by obtaining status reports timely is limited, as most of the packet delay is caused by the waiting time for the RX to send the status reports. Therefore, the retransmission based on enhanced polling cannot achieve the goal of timely retransmission for short packet delay budgets applicable to XR traffic.

Observation 1: Only polling enhancement could not achieve timely retransmission well.

Autonomous retransmission refers to the TX retransmission of the RLC SDU (segment) without Status Report when the specific condition(s) is met. Compared to enhanced polling, this scheme can completely avoid the delay for status reports and achieve timely retransmission. However, without status reports may lead to unnecessary retransmissions (ACK SDUs).

Observation 2: The unnecessary retransmission is introduced in autonomous retransmission.

From the above analysis, both methods have their own drawbacks, including limited gains or introducing system capacity issues. The major motivation on timely RLC retransmission is to reduce RLC ARQ retransmission delay. From this perspective, autonomous retransmission can minimize retransmission delay to the greatest extent. If status report of the SDU is not received within a long period of time, the autonomous retransmission should be performed. However, the subsequent operation of the SDU after autonomous retransmission needs to be based on status report. Therefore, further optimizing on autonomous retransmission can be considered by autonomous retransmission with polling enhancement.

The enhancement for timely RLC retransmission based on autonomous retransmission is considered for timely RLC retransmissions. The enhancement for timely RLC retransmission primarily relies on autonomous retransmission. When the remaining time of the SDU meets a certain threshold, the autonomous retransmission is triggered. The retransmitted SDU carries a poll indication, which is used to query the status of the SDU, allowing the RX to promptly provide a status report for subsequent actions in TX. If the status of the SDU is ACK, the retransmission will not continue. If the status of the SDU is NACK, retransmission continues based on the status report until the SDU is successfully received or discarded due to outdated.

Timely feedback of status reports allows TX to retransmit SDU within a limited time, improving the complete reception rate of the SDU. In some cases, timely status reports can also help TX to advance the window, potentially avoiding the retransmission of other nearly expired ACK SDUs. This autonomous scheme not only reduces the delay in waiting for status report but also controls unnecessary retransmissions, making it an optional timely retransmission solution.

Proposal 8：Enhance autonomous retransmission of the SDU with poll indication.

When the remaining time of the RLC SDU is less than a threshold, autonomous retransmission will be triggered. For autonomous retransmission, the existing delay critical indication from PDCP to RLC can be reused. If a PDCP SDU has less remaining time, and if the corresponding PDCP Data PDU has already been submitted to lower layers, the delay-critical indication for the PDCP Data PDU is provided to RLC. This indication means the corresponding RLC SDU is about to be outdated, but the indication is introduced for DSR, which may be too early for autonomous retransmission triggering. Therefore, the retransmission triggering threshold is different from the DSR triggering threshold and may be lower than it. When the remaining time of the RLC SDU is less than the threshold configured by network, RLC autonomous retransmission is triggered.

There are two methods for configuring the trigger threshold:

Method 1: Configuring the trigger threshold in the PDCP entity. The advantage is that it reuses the existing timer. When the remaining time of the SDU is less than this threshold, the PDCP indicates the RLC to perform autonomous retransmission.

Method 2: Configuring the trigger threshold in the RLC entity. This solution requires introducing a new timer in the RLC layer, which starts when the RLC receives the SDU from the upper layer. When the remaining time of the SDU is less than this threshold, the RLC performs autonomous retransmission without the inter-layer interaction (PDCP indication).

Proposal 9: Introduce an independent threshold for autonomous retransmission. If the remaining time of the RLC SDU becomes below the threshold, trigger the RLC SDU retransmission with poll indication.

In congestion scenarios, autonomous retransmission may have a negative impact on the system and exacerbate congestion. To solve this issue, the network can evaluate the channel quality and congestion status to determine whether to enable autonomous retransmission. Therefore, the introduction of activation/deactivation indicators in autonomous retransmission is used for network to control retransmission and avoid exacerbating congestion. This indication can activate autonomous retransmission for only certain UEs/QoS flows/DRBs/SDUs, or activate NACK SDU retransmission based on the reception status (simpler and more timely than status reports). Only the activated UEs/QoS flows/DRBs/SDUs will trigger autonomous retransmission when the condition in Proposal 9 is met.

Proposal 10: RAN2 considers introducing the indication for autonomous retransmission activation/deactivation.

If the TX side receives a status report for an SDU, the SDU will be retransmitted if the received status associated with the SDU is “NACK”. In this case, autonomous retransmission is not necessary because the retransmission opportunity has already been obtained for such SDU. Especially when the duration between autonomous retransmission and retransmission based on SR is too close. Autonomous retransmission is more applicable to the SDU whose reception status has never been received, i.e. the TX side has never received a status report containing the SN of that SDU. Therefore, autonomous retransmission can be used to give the retransmission opportunity for an SDU which without any reception status and is about to become outdated.

When the remaining time of the SDU is less than the trigger threshold, if the autonomous retransmission is activated and the status report for the SDU is not received after the initial transmission, the RLC will perform the autonomous retransmission of the SDU. If the autonomous retransmission is deactivated by network, the RLC will not perform the autonomous retransmission of the SDU.

Proposal 11: The TX side can perform autonomous retransmission of an SDU with the remaining time lower than the threshold if no status report containing the SN of the SDU has been received.

3. Conclusions

- Avoiding unnecessary RLC retransmissions

Proposal 1: The Network configures an indication to enable the RLC to stop transmissions of an outdated SDU notified by the PDCP regardless of whether the SDU has been submitted to the lower layer.

Proposal 2: Introduce a new timer to abandon SDUs in the RLC RX side:

- When there is a SN gap in RX side, start the new timer and associate the SN of the received SDU with the new timer.

- When the new timer expires, abandon the incompletely received SDUs with the SN smaller than or equal to the SN associated with the new timer within the RX window.

Proposal 3: When the RX side informs the TX side about the abandoned SDUs, the RX side indicates the abandoned SDUs as “ACK” in the SR, and no need to introduce a new “dropped” state.

Proposal 4: The TX side will not retransmit the SDU which is indicated as “ACK” in the SR, same as the existing handling for the SR.

Proposal 5: The RX side can update the RX window based on the abandoned SDUs before informing the TX side about the abandoned SDUs.

Proposal 6: No need to introduce the new SR triggering condition for the RX side to inform TX side about the abandoned SDUs.

Proposal 7: The TX side informs the RX side about discarded SDUs when the discarded SDU is associated with “NACK” indication in the SR.

- Timely RLC retransmission

Observation 1: Only polling enhancement could not achieve timely retransmission well.

Observation 2: The unnecessary retransmission is introduced in autonomous retransmission.

Proposal 8：Enhance autonomous retransmission of the SDU with poll indication.

Proposal 9: Introduce an independent threshold for autonomous retransmission. If the remaining time of the RLC SDU becomes below the threshold, trigger the RLC SDU retransmission with poll indication.

Proposal 10: RAN2 considers introducing the indication for autonomous retransmission activation/deactivation.

Proposal 11: The TX side can perform autonomous retransmission of an SDU with remaining time lower than the threshold if no status report containing the SN of the SDU has been received.

4. References

[1]	RAN2 #127bis Chairman Notes