3GPP TSG-RAN WG2 Meeting #128	R2-2410691

Orlando, USA, Nov.  18th – 22nd , 2024

Agenda Item:	8.8.2

Source:	HONOR

Title: 	Discussions on downlink coverage enhancement

Document for:	Discussion and decision

1	Introduction

Besides, during the RAN#104 meeting, the WID was updated based on the discussion related to SSB periodicity extension as following modification [1].

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] … - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

During the previous meetings for Rel-19 NTN, RAN2 discussed the downlink coverage enhancement in NTN system and made some agreements as follows.

Agreements:

1.	If it turns out that there is a need to bar UEs not supporting DL-CE, Rel-19 UEs not supporting DL-CE can be barred from accessing a cell operating with DL-CE using the existing NTN bar bit, in the same way as pre-Rel19 NTN UEs (this is an extension of the previous agreement to include also Rel-19 UE not supporting DL-CE)

2.	If it turns out that there is a need to bar UEs not supporting DL-CE, then we need to introduce a barring mechanism to control access of UEs supporting Rel19 NTN DL-CE. FFS on the details. (This also implies that UEs supporting Rel19 NTN DL-CE will not consider the existing NTN barring bit)

3.	(also depending on the details of the RAN1 solution) we can further consider methods to allow UEs not supporting DL CE to down-prioritize or prevent re-selection to cells operating with DL CE.

In this contribution, we would further discuss downlink coverage enhancement based on the previous agreements and provide proposals accordingly.

- Discussion

To improve the DL coverage enhancements, RAN2 continues discussing the system level enhancement. However, it is hard for RAN2 to discuss the solution in detail before receiving RAN1’s answers to the questions in the LS [2]. Based on our observation, RAN1 could not align with the basic approach in the last meeting since some companies argued that SSB periodicity extension is one of the promising solutions which others doubt the backward compatibility. Companies brought the discussion related to the system level enhancement in RAN#104 meeting and provided their thought that SSB periodicity extension. As shown in the updated WID, it is clarified that the restriction for SSB channel enhancement is not applied in the system level enhancement and RAN1 is asked to evaluate the impact to backward compatibility for potential extension of the SSB periodicity.

Observation 1: Potential extension of the SSB periodicity would be evaluated by RAN1 considering on impact to backward compatibility.

Based on the agreement in RAN#104 meeting, RAN2 could narrow the discussion down to the SSB periodicity extension. That is, we could further discuss the use cases, scenarios and identify the potential issues caused by SSB periodicity extension. Although the mapping relations between satellite beam, NR beam and cell are relative to RAN1 discussion, it is reasonable to have the assumption that one NR beam is composed of one or multiple satellite beams. As stated in the LS [2], the satellite beam is not visible to UEs in the current 5G system. The state transition of the satellite beam directly impacts the NR beam which is visible to UE which results in potential extension of the SSB periodicity. Thus, we could focus on the discussion about the impact on the NR beam instead of the satellite beam.

Proposal 1: RAN2 assumes one NR beam is composed of one or multiple satellite beams. Final decision is up-to RAN1.

Proposal 2: RAN2 assumes the beam level refers to the NR beam.

Based on the NTN deployment, one cell could include one or multiple NR beams. We don’t see the necessity to limit the NW implement to one cell only has one NR beam. Thus, we propose the way-forward that we start with the beam level enhancement and further discuss the enhancement when there is only one beam within the cell.

Proposal 3: RAN2 assumes that beam level mechanism is the start point for the Rel-19 DL coverage enhancements.

As mentioned in the WID, the issues/impacts could be cell search complexity, initial cell selection, latency, and success rate. For RRC_IDLE and RRC_INACTIVE UEs, the initial access is one of the most severe problems caused by the extension. Considering the limited number of simultaneously active beams, it is hard to keep one satellite beam active for a long time due to limited power and limited feeder link bandwidth. That is, it is also hard for RRC_CONNECTED UEs receiving long-term service from one specific NR beam/cell. Thus, enhancements for RRC_CONNECTED UEs may be needed due to the satellite beam state transition.

Considering the DL CE mechanism has impacts on both RRC_IDLE/INACTIVE UEs and RRC_CONNECTED UEs, the broadcasted SIB could be the baseline approach to deliver the information related to the cell/beam operating with DL CE.

Proposal 4: Information related to the cell/beam operating with DL CE could be broadcasted to the UE by SIB.

Observation 2: In addition to RRC_IDLE/INACTIVE UEs, the SSB periodicity extension also has impacts on RRC_CONNECTED.

In legacy, mobility management is important for the cellular network to provide consecutive service when the UE moves around. In another word, the mobility occurs at least when the UE prepare or execute the switch from one beam/cell to another. In the current specification, the mobility includes the mobility in all RRC states (e.g., handover, cell selection and cell reselection, etc.), measurement, radio link failure, beam failure detection and recovery and so on. In the scenario of DL coverage enhancement, the mobility occurs not only when the UE or NW node move around, but also when the beam hopping. From perspective of both CONNECTED and IDLE/INACTIVE mode mobility, mobility management need to perform measurement, cell selection, cell reselection, handover, radio link failure, beam failure detection and recovery and so on. Thus, we propose that:

Observation 3: There are potential impacts on mobility related aspects (e.g., measurement, handover, etc.) and failure handling cause by SSB periodicity extension.

Since NW is aware of whether the beam/cell is operating with DL CE, NW could send the beam/cell switch command to the suitable beam/cell to the RRC_CONNECTED UE. However, for the conditional handover which is triggered by the UE based on the measurement results and the Ephemeris, the UE may execute the handover to the beam/cell which is not available for a while which may result in unnecessary failure. Thus, we propose that

Proposal 5: RAN2 to study the impacts on conditional handover considering the beam/cell operating with DL CE.

The RRC_IDLE and RRC_INACTIVE UE shall perform measurements for cell selection and reselection purposes and try to select a suitable cell based on the measurements and the NW-configured criteria. Cell Reselection evaluation process composes of several steps like reselection priorities handling, measurement rules for cell re-selection, cell ranking/selection and so on. The SSB periodicity extension would prolong the time of performing measurement and the initial access. Thus, we propose:

Proposal 6: RAN2 to study the impacts on the measurement and evaluation for the cell selection/reselection handover considering the beam/cell operating with DL CE.

As for the pre-R19 UEs, RAN2 concluded that the existing NTN bar bit can be used during RAN2#127 meeting. However, there is still an FFS for the necessity of the barring.

Agreements: - If there is a need to bar pre-Rel19 NTN UEs from accessing a cell operating with DL coverage enhancement (e.g. because of extreme SSB periodicity) the existing NTN bar bit can be used. FFS about the behaviour for Rel-19 UEs not supporting DL coverage enhancement when the existing NTN bar bit is set.With regard to link level enhancement, RAN2 waits for RAN1 agreement on the DL channels to enhance before starting any RAN2 work.

In the current specification [3], the maximum value of the SSB periodicity is 160ms, which is shown as following.

During RAN1#118 meeting, RAN1 concluded that the default value[s] with extended periodicity assumed by NTN UE for initial access can be up to 640ms. During RAN1#118bis meeting, RAN1 down selected the periodicity of the SSB from the list {40ms, 80 ms, 160 ms, 320ms, 640ms} as follows:

Agreement for RAN1#118 For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is at least 160ms. - FFS: whether 320ms can be supported as the maximum of the additional default value instead of or in addition to 160ms.

Agreement for RAN1#118 bis As part of the NTN DL coverage enhancements at both system level and link level, RAN1 to consider: - Extending the periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - Default value[s] with extended periodicity assumed by NTN UE for initial access can be: - One [or more] values from the list {40ms, 80 ms, 160 ms, 320ms, 640ms} - Potential enhancements for transmitting the DL common channels using a wider beam footprint, while DL/UL dedicated channels (incl. PRACH) may be transmitted using a narrower beam footprint - Link-level enhancements for the following channels: - PDCCH - PDSCH with Msg 4 - PDSCH with SIB1/SIB19. - Note: link-level enhancements for PDSCH with SIB1/SIB19 may be applicable to other SIBs, without additional specification impact. - Note: the above does not imply that all the channels above will be enhanced, but all of them should be considered based on this agreement

Obviously, the pre-R19 UE could not cope with the extended periodicity value as 320ms or 640ms. If the maximum of the additional default value (apart from the existing 20ms value) is extended to 320ms, the pre-Rel19 UEs need to be barred. In addition, RAN1 is considering the wider beam only transmitting the DL common channels, the pre-Rel19 UEs could not work in the cells operating with DL CE. RAN2 could confirm the necessity of the barring.

Proposal 7: RAN2 to confirm the need of barring pre-Rel19 NTN UEs from accessing a cell operating with DL coverage enhancement (e.g., for the cell with SSB periodicity larger than 160ms and/or wider/narrower beam footprint mechanism).

As can be seen from the RAN1’s agreement above, there may be another potential enhancement for transmitting the DL common channels using a wider beam footprint, while DL/UL dedicated channels (incl. PRACH) may be transmitted using a narrower beam footprint. Although, the satellite beam footprint is not visible to the UE, NR beam which is composed of satellite beam(s) could be divided to two transmission mode: only DL common channels, DL/UL dedicated channels. That is, the N2 state with only DL common channel could be applied in the SSB of the wider beam footprint. UE may be not able/supposed to initiate random access to these NR beams with wider beam footprint.

If the R19 UE not supporting DL coverage enhancement is not barred based on the SIB broadcasted by the N2 mode beam mentioned above, the UE could attempt random access ending up with failure. Thus, if RAN1 considers the beam with only DL common channel, further enhancement needs to be done.

Proposal 8: RAN2 to study the potential impacts (e.g., access control, DL only indication)brought by the beam with only DL common channel.

3	Conclusions

In this contribution, we discussed the issues related to DL coverage enhancements in NTN. Based on the discussion, the following proposals are concluded:

Observation 1: Potential extension of the SSB periodicity would be evaluated by RAN1 considering on impact to backward compatibility.

Proposal 1: RAN2 assumes one NR beam is composed of one or multiple satellite beams. Final decision is up-to RAN1.

Proposal 2: RAN2 assumes the beam level refers to the NR beam.

Proposal 3: RAN2 assumes that beam level mechanism is the start point for the Rel-19 DL coverage enhancements.

Proposal 4: Information related to the cell/beam operating with DL CE could be broadcasted to the UE by SIB.

Observation 2: In addition to RRC_IDLE/INACTIVE UEs, the SSB periodicity extension also has impacts on RRC_CONNECTED.

Observation 3: There are potential impacts on mobility related aspects (e.g., measurement, handover, etc.) and failure handling cause by SSB periodicity extension.

Proposal 5: RAN2 to study the impacts on conditional handover considering the beam/cell operating with DL CE.

Proposal 6: RAN2 to study the impacts on the measurement and evaluation for the cell selection/reselection handover considering the beam/cell operating with DL CE.

Proposal 7: RAN2 to confirm the need of barring pre-Rel19 NTN UEs from accessing a cell operating with DL coverage enhancement (e.g., for the cell with SSB periodicity larger than 160ms and/or wider/narrower beam footprint mechanism).

Proposal 8: RAN2 to study the potential impacts (e.g., access control, DL only indication)brought by the beam with only DL common channel.

4	References

- RP-240775 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3

- R2-2405767 LS on DL coverage enhancements, Release 19, Source:	RAN2, To: RAN1.

- TS 38.331 Radio Resource Control (RRC); Protocol specification, V18.1.0 (2024-05)