3GPP TSG RAN WG2 Meeting #128	 R2-2410710

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:	8.6.3

Source: 	ITL

Title: 	Discussion on event triggered L1 measurement reporting

Document for:	Discussion and decision

Introduction

In the previous RAN2 meetings [1], agreements related to measurement enhancements are as follows:

Agreements on L1 event triggered MR - MR can be sent when the leaving condition is met, based on NW configuration. - Event triggered periodic MR can be supported, based on NW configuration. - For measurement resource configuration, R18 LTM CSI resource configuration is reused if possible. If CSI-RS resource only IE needs to be defined, we can revisit it in the stage 3. - For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused if possible. We can revisit it in the stage 3 if needed. - For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused if possible. We can revisit it in the stage 3 if needed. - The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1. - TTT operates only based on a timer (like TTT used in L3 event triggered MR). - Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5). - Basic information included in MR MAC CE: - Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id) - Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams - Triggered event information (e.g., ReportConfigID) - MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not). - N is configurable by NW. - Additional information included in MR MAC CE: - The information and quantity of current beam, based on NW configuration. - The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE. - NW can configure a dedicated SR configuration for MR MAC CE transmission.

In this contribution, we discussed remaining issues for event triggered L1 measurement reporting.

Discussion

Time to trigger (TTT)

In RAN2#127bis, L3 event triggered measurement report like TTT operation is agreed and an issue remained regarding whether the TTT operation is applied per beam or per cell.

- Per beam: The TTT is applied independently to each beam within a specific candidate cell. If the signal strength of any beam remains above the threshold during the TTT period, an L1 measurement result report is triggered.

- Per cell: The TTT is applied integratively to all beams indicated by a specific candidate cell. Due to the lack of filtering on L1 measurement results, there are significant fluctuations, and situations where the beam level falls below the threshold during the TTT period occur frequently. Therefore, by applying TTT on a per-cell and not resetting it as long as at least one beam meets the threshold, delays in reporting L1 measurement results can be minimized.

Firstly, we think it is important to consider the purpose of introducing TTT. In our understanding, TTT was introduced to enhance the reliability of measurement results.

When TTT operates per cell, it can lead to situations where the reliability of measurement results is reduced. For example, if TTT is set to 2 seconds and Beam 'A' has a signal strength above the threshold from 0 to 1 second, while Beam 'B' has a signal strength above the threshold from 1 to 2 seconds, an L1 measurement result report will be triggered. In this case, the purpose of introducing TTT is lost. Besides, it becomes unclear which beam should be reported when an L1 measurement result report is triggered. For example, even though the current signal strength is weak, if it has any effect on TTT operation, should it be included in the L1 measurement result reporting?

To address fluctuations in L1 measurement results, which are the reason for introducing TTT operation per cell, we think that sufficient control can be achieved through the configuration of LTM event evaluation parameters (Offset and threshold).

We prefer that TTT operates on a per-beam basis for the above reasons.

Proposal 1: TTT operation is applied per beam for the LTM event evaluation

An issue remains regarding whether to reset the TTT operation when the current beam changes.

Changing the current beam indicates that the new beam has a better quality than the previous one. In our understanding, since the new current beam can still satisfy the event conditions, we think there is no reason to reset the TTT. In other words, if the newly changed current beam does not satisfy the event conditions, the TTT is stopped, and there is no need to send the L1 measurement result. Conversely, if the newly changed current beam satisfies the event conditions, it indicates sufficient reason to proceed with a cell change via LTM.

Therefore, we prefer to maintain the TTT without resetting it if there is sufficient reason for a cell change (i.e., the current beam still satisfies the event conditions).

Proposal 2: When the current beam changes, the TTT operation is not reset

Measurement RS type alignment

In RAN2#127bis, RAN2 confirm the working assumption that same RS type should be used for both serving and neighbouring cell for event evaluation.

An issue was raised regarding the inability to always keep the RS types of the candidate cell and serving cell the same, as the current beam can change dynamically (CSI-RS or SSB).

We show our views on the options discussed as follows:

- Option 1: NW configures the both types of RS (SSB and CSI-RS) for the new beam(s), UE could select the same type of beam when UE evaluate the L1 event measurement.

- Option 2: Only one type of RS is configured for the new beam as a NW restriction, and UE perform the L1 measurement when RS type of the current beam is same with the new beam for event L1 measurement.

- Option 3: NW provides the offset which can be applied one type of RS to evaluate the different type of RS. For example, If the current beam is SSB and the measured beam for the candidate cell is CSI-RS, the UE applies the configured the offset to the measured CSI-RS value and then compare it with the current beam quality i.e. SSB measurement.

- Option 4: Rely on network configuration, i.e. same RS type is configured.

- Option 5: Which type to be used in measurement will be configured by NW. Then the UE only consider the configured RS type in the measurement.

- Option 6: The RS for current beam of serving cell is same as or associated with QCL RS provided in the indicated TCI-state, i.e. using the same type of RS as the candidate beam.

We think that configuring only one type of RS for the candidate cell (Option 2 and Option 4) and using only one type of RS for L1 measurement (Option 5) are too restrictive, especially when different types of RS could be measured or configured.

Option 1 and Option 6 are methods that do not restrict network configurations and can operate flexibly. Each of these methods can be used in the following situations.

- Option 1: When both types of RS are configured for the candidate cell, the UE determines the RS type of the candidate cell to use for measurement based on the current beam.

- Option 6: When only one type of RS is configured for the candidate cell, the UE determines the beam to use for measurement based on the RS type configured for the candidate cell (i.e., the indicated beam in the TCI state ID or the QCL RS beam associated with the TCI state ID).

In our understanding, both CSI-RS and SSB RS types can always be configured together in the measurement resource configuration. Therefore, we think that if both types are always configured, it is appropriate to use Option 1; otherwise, Option 6 can be used.

Proposal 3: RAN2 kindly asked to discuss following aspects to use same RS type for both serving and neighbouring cell for event evaluation

- If both CSI-RS and SSB RS types are always configured in the measurement resource configuration, Option 1 is used

- If only one type of RS is configured in the measurement resource configuration, Option 6 is used

Conclusion

In this contribution, we discussed event triggered L1 measurement reporting for LTM. According to discussion in section 2, we have the following proposals:

Proposal 1: TTT operation is applied per beam for the LTM event evaluation

Proposal 2: When the current beam changes, the TTT operation is not reset

Proposal 3: RAN2 kindly asked to discuss following aspects to use same RS type for both serving and neighbouring cell for event evaluation

- If both CSI-RS and SSB RS types are always configured in the measurement resource configuration, Option 1 is used

- If only one type of RS is configured in the measurement resource configuration, Option 6 is used

Reference

RAM2#127bis meeting report