3GPP TSG-RAN WG2 Meeting #128	R2-2410715

Orlando, USA, Nov. 18th – 22th, 2024

Agenda item:		8.8.2 Downlink coverage enhancements

Title:		Discussion on downlink coverage enhancements in NR NTN

Source:		ETRI, Korea University

Document for:		Discussion

# 1. Introduction

During the 103th meeting of RAN, a new WI on NTN for NR phase 3 was approved, including the following objective:

1. Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - Notes for this objective: - SSB channel enhancement is not considered - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

The first discussion regarding the new WI took place during 125bisth meeting of RAN WG2 and then, the following agreements were made for downlink coverage enhancements in NTN.

Agreements: 1. With regard to link level enhancement, RAN2 waits for RAN1 agreement on the DL channels to enhance before starting any RAN2 work. 2. We will continue the discussion on RAN2 aspects of DL coverage enhancements (e.g. cell level / beam level DTX/DRX mechanism, etc.) in the next meetings, trying to identify questions to RAN1 for aspects where we need their input

During the 126th meeting of RAN WG2, the following agreements were made:

Agreements: 1. Based on the solution being investigated in RAN1, RAN2 will further discuss whether/how legacy UEs might operate in a cell supporting DL coverage enhancements. 2. RAN2 assumes that both EFC and EMC are supported

During the 127th meeting of RAN WG2, the following agreements were additionally made:

Agreements: 1. From RAN2 point of view, if the SSB periodicity is no larger than 160ms, there is no RAN2 impact on SSB configuration (there might still be impacts on DTX aspects) 2. From RAN2 point of view, If the SSB periodicity is larger than 160ms, for example ssb-PeriodicityServingCell, measurement gap periodicity, SMTC configuration, ssb-Periodicity-r17 for NonCellDefiningSSB-r17 may need to be extended. And the field description of nAndPagingFrameOffset may need to be enhanced to consider the SSB periodicity higher than 160ms. 3. RAN2 can further consider SMTC impacts due to beam-hopping / larger SSB periodicity 4. If there is a need to bar pre-Rel19 NTN UEs from accessing a cell operating with DL coverage enhancement (e.g. because of extreme SSB periodicity) the existing NTN bar bit can be used. FFS about the behaviour for Rel-19 UEs not supporting DL coverage enhancement when the existing NTN bar bit is set.

During the 127bisth meeting of RAN WG2, the following agreements were additionally made:

Agreements: 1. If it turns out that there is a need to bar UEs not supporting DL-CE, Rel-19 UEs not supporting DL-CE can be barred from accessing a cell operating with DL-CE using the existing NTN bar bit, in the same way as pre-Rel19 NTN UEs (this is an extension of the previous agreement to include also Rel-19 UE not supporting DL-CE) 2. If it turns out that there is a need to bar UEs supporting DL-CE, then we need to introduce a barring mechanism to control access of UEs supporting Rel19 NTN DL-CE. FFS on the details. (This also implies that UEs supporting Rel19 NTN DL-CE will not consider the existing NTN barring bit) 3. (also depending on the details of the RAN1 solution) we can further consider methods to allow UEs not supporting DL CE to down-prioritize or prevent re-selection to cells operating with DL CE.

In this paper, we would like to provide our views on the highlighted aspects as well as remaining issues on downlink coverage enhancements in NTN.

# 2. Discussion

2.1	Barring Mechanism

During the 127th bis meeting, it was agreed that, if there is a need to bar pre-Rel-19 NTN UEs and Rel-19 UEs not supporting DL-CE from accessing a cell operating with DL-CE, the existing NTN bar bit can be used. It was also agreed that, if there is a need to bar UEs supporting DL-CE, a new barring mechanism should be introduced to control access for UEs supporting Rel-19 NTN DL-CE. However, the specifics of this mechanism are still under discussion (FFS). In this regard, the following explicit and implicit solutions are possible for consideration:

- Explicit solution: Define a cellBarredNTN-DL-CE field with values {bared, notBared}.

  - The value "barred" indicates that the cell is barred for UEs supporting DL-CE, while "notBarred" indicates that the cell is accessible to UEs supporting DL-CE. If not present, the UE assumes the cell is not allowed for connectivity to UEs supporting DL-CE.

  - When both cellBarred and cellBarredNTN are set to "barred," UEs supporting DL-CE ignore the "barred" value.

- Implicit solution: Use a SIB to convey system information related to NTN DL-CE cell configurations (e.g., DTX/DRX pattern, SSB periodicity extension, etc.).

  - When both cellBarred and cellBarredNTN are set to "barred," UEs supporting DL-CE check for the presence of a SIB conveying system information related to NTN DL-CE cell configuration. If the SIB including information related to NTN DL-CE is present, the cell is allowed for connectivity to UEs supporting DL-CE; if absent, the UE assumes the cell is not allowed for connectivity to UEs supporting DL-CE.

ETRI prefers the explicit barring mechanism, as it provides a simple and clear solution, whereas the implicit solution may impose additional burdens on the UE to monitor other SIBs in addition to SIB1.

Observation 1: An explicit barring mechanism provides a simple and clear solution, while the implicit mechanism may impose additional burdens on UEs by requiring them to monitor SIBs beyond SIB1.

Proposal 1: RAN2 to consider an explicit barring mechanism to control access for UEs supporting Rel-19 DL-CE.

- Define a cellBarredNTN-DL-CE field with values {barred, notBarred}.

  - The value "barred" indicates that the cell is barred for connectivity to UEs supporting DL-CE, while "notBarred" means that the cell is allowed for connectivity to UEs supporting DL-CE. If not present, the UE assumes the cell is not allowed for connectivity to UEs supporting DL-CE.

  - When both cellBarred and cellBarredNTN are set to "barred," UEs supporting DL-CE ignore the "barred" value.

2.2	Cell/Beam DTX(/DRX)

According to the reply LS from RAN1, it was noted that the beam footprint status — categorized as "off," "common messages only," and "active traffic" — in one of the RAN1 agreements is defined solely for system-level evaluation methodology purposes. RAN2 has not yet received detailed information on beam hopping mechanisms within the scope of RAN1's work. To facilitate rapid progress under the same working assumptions between RAN1 and RAN2, RAN2 should provide its preferred working assumptions for the beam hopping mechanism to RAN1 while awaiting RAN1's response. In this context, ETRI believes that RAN2 should begin discussing the cell/beam DTX(/DRX) mechanism to support beam hopping operations while awaiting answers from RAN1.

Proposal 2: RAN2 to begin discussing the cell/beam DTX(/DRX) mechanism to support beam hopping operations while awaiting answers from RAN1.

The following beam states are considered in RAN1 for system level study:

- N1 beam footprints are in state of “off”

  - These beam footprints are not served by any signal (no satellite service in this area)

- N2 beam footprints are in state of “common messages only”

  - These beam footprints do not have any active user traffic, and are served the necessary information for cell discovery and initial access.

  - Optionally, companies may consider user arrival (e.g. RACH access) in this type of cell, and should describe how this is taken into account in the analytical evaluation

- N3 beam footprints are in state of “active traffic”

  - These beam footprints have X active (e.g. VoNR) users each.

  - These beam footprints also provide the necessary information for cell discovery and initial access processes.

- N1 + N2 + N3 = “Total number of beam footprints “

Since Rel-18 NES is considered as baseline in the system level study according to the objectives of this WI, we have to confirm whether or not the cell DTX(/DRX) mechanism introduced in Rel-18 NES can be directly applied for the support of the aforementioned beam states as it is. To access the applicability of the DTX/DRX mechanism in Rel-19 NES, it is imperative to look into the differences between Rel-18 NES and beam hopping in NTN. The main features of cell DTX(/DRX) mechanisms in Rel-18 NES are outlined below:

- Cell-level DTX(/DRX) operation

- DTX/DRX can be configured independently.

- It is only applicable to RRC connected UE.

- During cell DTX non-active duration, UE does not monitor PDCCH in determined cases and SPS occasions.

- During cell DRX non-active duration, UE does not have transmission on CG resources and SR transmission.

- It has no impact on transmission of RACH, paging, SIBs and SSB.

On the other hand, the main features of beam hopping operation for downlink coverage enhancements in NTN are as follows:

- Possible NR beam-level hopping operation

  - For instance, in case of multiple NR beams per one NR cell within one satellite

- Beam hopping is applied for both transmission and reception.

- It is appliable to RRC connected as well as Idle mode UEs

- In state N1, UE does not have any transmission and reception.

Considering these different features between Rel-18 NES and beam hopping in NTN, we believe that new DTX(/DRX) mechanisms similar to those in Rel-18 NES should be considered for downlink enhancements in NTN. Next, while the existing DTX(/DRX) mechanisms in Rel-18 NES are operated at the cell level, meaning that all UEs within a single cell are subject to the same DTX(/DRX) configurations in their UE behaviour, the support of beam hopping in NTN requires independent configuration of DTX(/DRX) operations for each physical satellite beam. In scenarios where a single NR cell comprises multiple NR beams, with each NR beam mapped to a distinct physical satellite beam, cell-level DTX(/DRX) mechanisms cannot adequately support the independent DTX(/DRX) operation of each physical satellite beam.

Proposal 3: RAN2 to consider NR beam-level DTX(/DRX) mechanisms for downlink coverage enhancement in NTN.

In NTN, DTX(/DRX)-based beam hopping operation may require additional timing adjustments between the UE and gNB for various signaling procedures (e.g., RACH procedure, SMTC measurement) based on information about the UE location and beam hopping configurations. Therefore, ETRI believes that beam hopping configuration information should be provided to the UE. Considering that beam hopping can be applied to both RRC-connected and idle UEs, these DTX/DRX configurations for beam hopping can be signaled via system information. In practical implementation, while DTX/DRX operation for the N2 beam state could be semi-statically configured, in the N3 beam state, where RRC-connected UEs are present, it could be dynamically configured via DCI.

Observation 2: DTX(/DRX)-based beam hopping operation may require additional timing and operation duration adjustments between the UE and gNB for various signaling procedures (e.g., RACH procedure, SMTC measurement) based on information about the UE location and beam hopping configurations.

Proposal 4: RAN2 to consider signaling DTX(/DRX) configurations for downlink coverage enhancements via system information (at least for DTX(/DRX) configuration for N1 & N2 beam states).

Another issue is whether DRX can be supported for downlink coverage enhancement in NR-NTN. If DRX is not supported, flexibility in certain operations, such as RACH, may increase, as the UE could transmit an uplink signal at any time (even during cell DTX inactive duration). However, this may also increase the impacts on the existing timing and operation durations for some operations. Therefore, before deciding on DRX support, RAN2 should study the impacts on the existing operations, such as RACH, when DRX is not supported.

Proposal 5: Before deciding on DRX support, RAN2 should study the impacts on the existing operations, such as RACH, when DRX is not supported.

Furthermore, if DRX is supported, considering the purpose of implementing the beam hopping technique, such as SWaP-C limitations on satellite payload, simultaneous beam hopping in both downlink and uplink is likely to be of particular interest.

Proposal 6: if DRX is supported, as an initial consideration, simultaneous DTX/DRX operation can be considered for downlink coverage enhancement in NTN.

According to the agreement of RAN1, it is noted that beam hopping operation in NTN involves three states, which differs from the two states defined in Rel-18 NES. Therefore, the following two active durations need to be defined to accommodate the three types of beam states.

- N2 active duration (start and end time) and periodicity

  - UE behaviour

    - Reception of paging, SIBs and SSB and transmission of RACH

    - No monitor of PDCCH in determined cases and SPS occasions (similar to Rel-18 NES)

    - No transmission of CG resources and SR transmission (similar to Rel-19 NES)

- N3 active duration (Start and end time) and periodicity

  - UE behaviour

    - Similar to those in non-active duration of Rel-18 NES

- N2 and N3 non-active duration

  - UE behaviour: No transmission and Reception

Proposal 7: RAN2 to consider active durations and non-active durations for N1, N2 and N3 in DTX/DRX operation for downlink coverage enhancement in NTN:

  - N2 or N3 beam active duration

  - Non-active duration (N1 active duration)

The above active and non-active duration can be defined with the start/end time, periodicity and offset value from reference time. However, the detailed values for these active and non-active durations should be determined from the study results of RAN1, considering the impacts on the performance of receiving paging, SIBs and SSB as well as the transmission of RACH.

Proposal 8: RAN2 to wait for the study results from RAN1 for the detailed designs of DTX(/DRX) mechanisms (e.g. active durations and periodicities) under consideration of the impacts of performance related to receiving paging, SIBs and SSB as well as RACH procedure.

# 3. Conclusions

Observation 1: An explicit barring mechanism provides a simple and clear solution, while the implicit mechanism may impose additional burdens on UEs by requiring them to monitor SIBs beyond SIB1.

Proposal 1: RAN2 to consider an explicit barring mechanism to control access for UEs supporting Rel-10 DL-CE.

- Define a cellBarredNTN-DL-CE field with values {barred, notBarred}.

  - The value "barred" indicates that the cell is barred for connectivity to UEs supporting DL-CE, while "notBarred" means that the cell is allowed for connectivity to UEs supporting DL-CE. If not present, the UE assumes the cell is not allowed for connectivity to UEs supporting DL-CE.

  - When both cellBarred and cellBarredNTN are set to "barred," UEs supporting DL-CE ignore the "barred" value.

Proposal 2: RAN2 to begin discussing the cell/beam DTX(/DRX) mechanism to support beam hopping operations while awaiting answers from RAN1.

Proposal 3: RAN2 to consider NR beam-level DTX(/DRX) mechanisms for downlink coverage enhancement in NTN.

Observation 2: DTX(/DRX)-based beam hopping operation may require additional timing and operation duration adjustments between the UE and gNB for various signaling procedures (e.g., RACH procedure, SMTC measurement) based on information about the UE location and beam hopping configurations.

Proposal 4: RAN2 to consider signaling DTX(/DRX) configurations for downlink coverage enhancements via system information (at least for DTX(/DRX) configuration for N1 & N2 beam states).

Proposal 5: Before deciding on DRX support, RAN2 should study impacts on existing operations, such as RACH, when DRX is not supported.

Proposal 6: if DRX is supported, as an initial consideration, simultaneous DTX/DRX operation can be considered for downlink coverage enhancement in NTN.

Proposal 7: RAN2 to consider active durations and non-active durations for N1, N2 and N3 in DTX/DRX operation for downlink coverage enhancement in NTN:

  - N2 or N3 beam active duration

  - Non-active duration (N1 active duration)

Proposal 8: RAN2 to wait for the study results from RAN1 for the detailed designs of DTX(/DRX) mechanisms (e.g. active durations and periodicities) under consideration of the impacts of performance related to receiving paging, SIBs and SSB as well as RACH procedure.