3GPP TSG-RAN WG2 Meeting #128	R2-2410733

Orlando, USA, 18th – 22nd November 2024

Source:                Qualcomm Incorporated

Title:	MRO enhancement for SON and MDT

Agenda Item:	8.10.2

Document for:	Discussion and Decision

# Introduction & Background

- MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]:

- Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3]

- Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

# Discussion

## 2.1 MRO for LTM

### RACH information in RLF Report

In the previous meetings, some companies proposed to discuss RACH-less LTM to RACH-based LTM “fall back” scenarios. There could be two cases that the RACH-less LTM is not successful:

Case 1: The UE doesn’t receive a valid TA from LTM CSC or if the UE-based TA measurement is not successful, UE doesn’t even start the RACH-less LTM cell switch procedure and directly attempts a RACH-based LTM procedure.

Case 2: The RACH-less LTM is initiated (because of UE having a valid TA either received from LTM CSC or via UE-based measurement) but the RACH-less LTM cell switch execution is not successful e.g., because UE couldn’t determine that the network has successfully received its first UL data. In such a case, T304 could expire.

Upon T304 expiry, UE performs cell selection and if the selected cell is an LTM candidate cell and if network configured the UE to try LTM after LTM execution failure, then the UE attempts RACH-based LTM execution once, otherwise re-establishment is performed. Whatever the LTM recovery is configured or not, is executed successfully or not, it is beneficial the UE indicates to gNB if RACH-less LTM is attempted or not before RACH-based LTM failure so that gNB will know whether TA configuration is not proper or UL transmission is not good.

- UE indicates to gNB in RLF Report if RACH-less LTM is attempted or not before RACH-based LTM failure.

### Similarly, in successful case, it is also possible that RACH-less LTM failure and then UE fall back to RACH-based LTM and success. In this case, it is also beneficial to let gNB know if RACH-less LTM is attempted or not before LTM successful RACH-based LTM recovery.

- UE indicates to gNB in SHR if RACH-less LTM is attempted or not before RACH-based LTM successful RACH-based LTM recovery.

### MHR enhancements for LTM

A UE supporting LTM might undergo both LTM and normal L3 HO scenarios depending on the gNB implementation. A gNB might do a back-to-back L3 HO and LTM due to L3 execution conditions being met in gNB-CU and LTM execution condition met in gNB-DU.

Therefore, it useful for the gNB to know whether the UE’s visited cells were due to LTM or L3 HO. Without such knowledge, the gNB might unnecessarily be trying to optimize ping-pong handovers even though they happened due to different reasons (i.e., L3 HO and LTM). It is beneficial to optimize the MHRto include a HO-type (i.e., LTM or L3 HO) similar to how it has been agreed by RAN2 for RLF Report.

- Enhance MHR to include a “HO-type” (i.e., LTM or L3 HO).

## 2.3 MRO for CHO with candidate SCG

On MRO for CHO with candidate SCG, there could be some scenarios to be considered as following.

Scenario 1: UE encounters an RLF or SCG failure before the CHO with candidate SCG execution and execution conditions of no candidate PCell and no candidate PSCells were met before the RLF or SCG failure

Scenario 2: UE encounters an RLF or SCG failure before the CHO with candidate SCG execution and one execution condition was “blocking” the whole execution e.g.,

- A candidate PCell (A1 in the example below) satisfies the execution condition but none of its associated candidate PSCells (i.e., B1 or B2) satisfy the corresponding execution condition.

- A candidate PSCell (B3 in the example below) satisfies the execution condition but the associated candidate PCell (i.e., A2) doesn’t satisfy the corresponding execution condition.

In scenarios 1 and 2, it might be useful for the network to know why the CHO with candidate SCG execution e.g., it didn’t happen because of one “blocking” execution condition (even though the other execution condition was met) or because none of the execution conditions were met

Scenario 3: UE encounters an RLF or SCG failure before the CHO with candidate SCG execution and non-associated PCells and PSCells met the execution condition

In the example below, Execution condition of candidate PCell A1 and candidate PSCell B3 is met but the CHO with candidate SCG execution didn’t happen because there is no (A1, B3) associated configuration.

It would be beneficial if the network can know what execution conditions were met (i.e., A1 and B3) so that the network can fine-tune the CHO with candidate SCG configuration eg., by including a (A1, B3) configuration or (A2, B3) confguration to avoid RLF or SCG failures in the future.

In last RAN2 meeting, RAN2 agreed the following for CHO with candidate SCG,

UE includes following information in RLF report:

- b.	Time information regarding condition fulfilment for CHO with candidate SCGs. Details are FFS. We consider both the case when both CHO condition and associated CPC condition are fulfilled, and the case when CHO (or CPC) is fulfilled but CPC (or CHO) conditions are not fulfilled.

- c.	Measurement results of PCells and PSCells.

Considering the above example scenarios and RAN2 agreement, we propose additionally the following:

- Enhance RLF Report and SCGFailureInformation with:

    - The type of fulfilled execution condition(s) (i.e., CHO execution condition or CPC execution condition or none)

    - Identifier of candidate PCell(s) which met the configured CHO execution conditions before the RLF is encountered

    - Identifier of candidate PSCell(s) which met the configured CPC execution conditions before the RLF is encountered

    - Time elapsed between the fulfilled execution and RLF for CHO and/or CPC.

    - Time elapsed between the two fulfilled execution conditions (CHO and CPAC)

### 2.2.4 CHO Recovery after CHO with candidate SCG execution failure

When initial CHO execution attempt fails or HO fails, the UE performs cell selection, and if the selected cell is a CHO candidate and if network configured the UE to try CHO after handover/CHO failure, then the UE attempts CHO execution once, otherwise re-establishment is performed.

Suppose a UE receives a CHO with candidate SCG config, both the execution conditions are met but the CHO execution fails (e.g., RACH in target cell fails). UE performs cell selection, and the selected cell is also a CHO candidate cell.

As per the below text from 38.331, UE sets the choCellId (CHO recovery cell ID) in RLF Report only in case CHO recovery is attempted after a legacy CHO failure and not in case of CHO recovery is attempted after a CHO with candidate SCG execution failure.

1>  if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG, except for an LTM cell switch procedure following cell selection performed while timer T311 was running, as specified in 5.3.7.3, or mobility from NR failure, and

1>  if attemptCondReconfig is configured; and

1>  if the selected cell is not configured with CondEventT1, or the selected cell is configured with CondEventT1 and leaving condition has not been fulfilled; and

1>  if the selected cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig and the condExecutionCondPSCell is not configured for the corresponding condReconfigId in the MCG VarConditionalReconfig:

2>  if the UE supports RLF-Report for conditional handover, set the choCellId in the VarRLF-Report to the global cell identity, if available, otherwise to the physical cell identity and carrier frequency of the selected cell;

2>  apply the stored condRRCReconfig associated to the selected cell and perform actions as specified in 5.3.5.3;

NOTE 1:  It is left to network implementation to how to avoid keystream reuse in case of CHO based recovery after a failed handover without key change.

We therefore have the following proposal:

- UE should log the CHO recovery cell ID in RLF Report even when UE attempts CHO recovery after a CHO with candidate SCG execution failure.

Consider Scenario 2 again where the UE encounters an RLF before the CHO with candidate SCG execution because candidate PSCell (B3 in the example below) satisfies the execution condition but the associated candidate PCell (i.e., A2) doesn’t satisfy the corresponding execution condition.

Suppose the UE attempts CHO recovery and observed that both PCell A2 and PCell A3 are suitable cells, the UE should be able to prioritize selecting PCell A2 for CHO recovery because its associated PSCell’s (B3) execution condition was met before rather than selecting PCell A3 for CHO recovery because its associated PCell’s (B4) execution condition was not met before.

We therefore have the following proposal:

- In case the UE attempts CHO recovery after a CHO with candidate SCG execution failure and observes that there are 2 or more suitable cells, UE may prioritize selecting a cell for CHO recovery based on whether its associated PSCell’s execution condition was met before or not.

# Conclusion

LTM,

- UE indicates to gNB in RLF Report if RACH-less LTM is attempted or not before RACH-based LTM failure.

- UE indicates to gNB in SHR if RACH-less LTM is attempted or not before RACH-based LTM successful RACH-based LTM recovery.

- Enhance MHR to include a “HO-type” (i.e., LTM or L3 HO).

CHO with candidate SCG,

- Enhance RLF Report and SCGFailureInformation with:

    - The type of fulfilled execution condition(s) (i.e., CHO execution condition or CPC execution condition or none)

    - Identifier of candidate PCell(s) which met the configured CHO execution conditions before the RLF is encountered

    - Identifier of candidate PSCell(s) which met the configured CPC execution conditions before the RLF is encountered

    - Time elapsed between the fulfilled execution and RLF for CHO and/or CPC.

    - Time elapsed between the two fulfilled execution conditions (CHO and CPAC)

- UE should log the CHO recovery cell ID in RLF Report even when UE attempts CHO recovery after a CHO with candidate SCG execution failure.

- In case the UE attempts CHO recovery after a CHO with candidate SCG execution failure and observes that there are 2 or more suitable cells, UE may prioritize selecting a cell for CHO recovery based on whether its associated PSCell’s execution condition was met before or not.

# Reference