3GPP TSG-RAN WG2 Meeting #128	R2-2410774

Orlando, USA, Nov 18th – Nov 22nd, 2024

Agenda item:	8.3.2

Source:	Qualcomm Incorporated

Title:	Simulation results for RRM measurement prediction

WID/SID:	FS_NR_AIML_Mob – Release 19

Document for:	Discussion and Decision

# Introduction

In the last RAN2 meeting (RAN2 #127-bis [1]), very good progress was achieved on the following topics. For the agreements made during the meeting, please see the Chair notes [1].

- Measurement Event prediction.

- Simulation results for RRM measurement prediction.

  - Companies provided simulation results for the RRM measurement prediction study, for the previously agreed scenarios. Based on the simulation results, some conclusions and observations were agreed during the meeting.

Table 1 summarizes the agreed prioritization of the evaluation scenarios for the RRM measurement prediction study.

Table 1: Prioritization of RAN2 AI/ML RRM measurement prediction study evaluation scenarios

| Case number | Prioritization | Evaluation scenario combination | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

In this contribution, we discuss our simulation results for the following scenarios from the above table:

- FR2-to-FR2 intra-frequency, intra-cell, temporal domain case A (Case number 4 in the table above),

- FR1-to-FR1 intra-frequency, intra-cell, temporal domain case B (Case number 2 in the table above).

- FR1 to FR1 inter-frequency (frequency domain) (Case number 3 in the table above).

# Simulation assumptions

In this section, we discuss assumptions applicable to both FR1 and FR2 data generation simulations, detailing the methodology for UE trajectory generation, soft line-of-sight (LoS) updates, and shadow fading updates.

## UE trajectory generation formulation

This section details the chosen model for generating random trajectories for mobile UEs in data generation simulations. The chosen model is based on Option 2 in TR 38.343 [2], Section 6.3.1, with modifications based on Option 4 in [2] and a minor modification for extended mobility.

A UE has initial position (x0,y0, z0), constant forward velocity s, and initial azimuthal velocity angle ϕv,0. Temporal granularity tgran is defined as the smallest unit of time considered in random trajectory generation and mean temporal step size tstep is defined as the mean travel time for walk steps along the random trajectory. tstep is chosen such that it will be a whole-number multiple of the temporal granularity:

tstep=Nsubstepstgran, where

- Nsubsteps∈Z+ is the mean number of substeps per walk step.

- Maximum walk time tmax≫tstep is likewise chose to be a whole-number multiple of tgran.

- Distance granularity dgran is computed as dgran=stgran.

For the purposes of random trajectory generation for urban macro and urban micro simulations, a geometric sector SNb is defined as a geometric region consisting of the intersection of two regions: the first region consists of one third of the hexagonal cell surrounding a gNB and symmetric with respect to the gNB’s azimuthal orientation ϕNb, and the second region consists of the area outside a circle with radius dNb centered on the gNB. Figure 1 provides an example of a geometric sector.

Figure 1 Geometric sector defined for UE mobility constraints

The overall valid UE trajectory region Svalid is the geometric region in which a UE trajectory may be generated and consists of the union of all geometric sectors corresponding to each gNB in the simulation:

Svalid=i=1NNbSi

Where NNb is the number of gNBs in the simulation. Figure 2 visualizes Svalid for a 2-tier UMa simulation with ISD 200.

Figure 2 Geometric sectors for every gNB in a 2-tier Urban Macro simulation. Each geometric sector is labelled with the MAC ID of its corresponding gNB. The valid UE trajectory region Svalid consists of the union of all sectors.

Given the above setup, the following algorithm is used to generate a random trajectory for a particular UE:

- Initialize total sub-step counter m=0.

- For walk step k=1,2,…:

  - Draw number of sub-steps for the kth walk step Nsubsteps,k from a geometric distribution with probability of success 1Nsubsteps.

  - Draw azimuthal velocity angle delta for the kth walk step Δϕv,k from a uniform distribution in the range [-45°, 45°].

  - Compute sub-step velocity angle delta: Δϕv,k,substep=Δϕv,kNsubsteps,k

  - For each sub-step n=1, …,Nsubsteps,k:

    - Increment total sub-step counter m=m+1. If the UE’s total travel time thus far mtgran is greater than tmax, terminate the walk. Otherwise, continue.

    - Update the UE’s sub-step velocity angle: ϕv,m=ϕv,m-1+Δϕv,k,substep.

    - Compute potential updated UE position: (xm,ym,zm)=xm-1+dgrancosϕv,m,ym-1+dgransinϕv,m,z0.

If xm,ym,zm  is outside the valid UE trajectory region Svalid, terminate the walk. Otherwise, update the UE’s position.

Figure 3 shows an example set of UE trajectories generating using the above algorithm.

Figure 3 Example random trajectories for 20 UEs within a 2-Tier UMa model

## Channel modelling: soft LoS

This section details the approach used to update line-of-sight (LoS) states for mobile UEs in data generation simulations. Discussion of the motivation for adopting this soft LoS model has been provided in [3].

In the soft LoS model provided in TR 38.901 [4], Section 7.6.3.3, each gNB-UE link maintains a soft LoS state LOSsoft∈[0, 1], calculated as

LOSsoftG,d=12+1πarctan20λG+Fd, where

- G is a spatially consistent Gaussian R.V. with correlation distance dcor according to 38.901 Table 7.6.3.1-2,

- λ is the wavelength of the center frequency fc (in meters), and

- Fd=2erf-12PrLOSd-1.

To apply this soft LoS state, for each link, both LoS and NLoS channel coefficients HLOS and HNLOS are generated per the 38.901 Fast Fading Model (Section 7.5, Steps 2-12), and the overall channel is calculated per 38.901 Equation 7.6-18 as

HLOSsoft=HLOSLOSsoft+HNLOS1-LOSsoft2.

Although TR 38.901 provides correlation distances for LoS random variables, a specific correlation function is not provided. To correlate the Gaussian random variables G used in soft LoS state generation in Option B, we use exponential correlation:

RLOSΔd=exp-Δddcor, where

- Δd is the distance between any two points in the xy-plane, and

- dcor is the correlation distance provided in Table 7.6.3.1-2.

As discussed in [3], this exponential correlation function is chosen to be consistent with the exponential autocorrelation function provided for shadow fading in 38.901, Section 7.4.4, as well as with the spatial correlation procedure provided in WINNER II [5], Section 3.3.1. We also note that the distribution statistics of the Gaussian random variables G are not specified in TR 38.901; for simplicity, we choose G to be standard normal Gaussians.

We assume that exponential correlation is applied on a per-gNB basis; that is, the Gaussians used to generate the soft LoS states for all UE-gNB links associated with a particular gNB are correlated per RLOSΔd, and there is no inter-gNB LoS correlation. We also assume that the generated LoS states for given xy-locations are fixed in time, meaning UEs will only experience LoS state transitions as a result of mobility. With these assumptions, the following procedure is used to calculate soft LoS states for mobile UEs:

- For each gNB in the simulation n=1,2,…,NNB:

  - Establish a grid of location points Dn that covers the area in which mobile UE trajectories will be constrained.

  - Over Dn generate a grid Gn of standard normal Gaussian realizations at each grid point correlated in distance by RLOSΔd=exp-Δddcor.

  - Using the soft LoS state procedure outlined in TR 38.901, Section 7.6.3.3, generate a grid of soft LoS realizations Ln= LOSsoft(Gn, Dn) from Gn and PrLOSd for the given scenario.

- During the channel update for each mobile UE i=1,2,…,NUE:

  - For each gNB in the simulation n=1,2,…,NNB, interpolate the soft LoS state for the UE-gNB link from Ln given the mobile UE’s xy-location.

Figure 4 shows an example soft LoS grid for an FR2 UMi simulation with the gNB located at (0, 0).

Figure 4 Example Soft LoS grid (UMi, fc = 30 GHz)

To demonstrate the impact of soft LoS on the generated channels for mobile UEs, Figure 5 provides a comparison between simulation results for a mobile UE with and without soft LoS updates enabled. In the simulation with LoS updates disabled (orange line), the mobile UE has a constant LoS = 1 state with its serving gNB, while in the simulation with LoS updates enabled (blue line), the LoS state is updated as a function of the UE’s position. As the mobile UE enters NLoS regions, the maximum RSRP across all beam pair links in the simulation with LoS updates tends to drop significantly compared to the maximum RSRP for the simulation where the LoS state remains 1.

Figure 5 Simulation comparison for a mobile UE with and without soft LoS updates enabled. Top: maximum RSRP for mobile UE with and without soft LoS updates. Bottom: LoS state for mobile UE with and without LoS updates. A value of 0 corresponds to a completely NLoS link, while a value of 1 corresponds to a completely LoS link.

Observation 3: Modelling LoS updates for mobile UEs can lead to large differences in channel measurement results compared to simulations with static LoS states. These differences could have significant ramifications in handover situations for mobile UEs that move far from their serving gNBs.

## Channel modelling: shadow fading

This section describes the model used for updating shadow fading values for mobile UEs in data generation simulations. Similarly to soft LoS state updates, we use a grid-based approach to update shadow fading values in a spatially consistent manner. Discussion of the motivation for adopting this approach has been provided in [3].

As with soft LoS values, we assume that shadow fading values are fixed in time for given xy-locations and independently calculated on a per-gNB basis. We also assume that a gNB’s shadow fading values are autocorrelated exponentially as per TR 38.901 [4], Section 7.4.4, with autocorrelation function

RΔx=exp-Δxdcor, where

- Δx is the 2D-distance between any two locations in the simulation, and

- dcor is the environment-specific correlation distance provided in TR 38.901, Table 7.5-6.

With these assumptions, the following procedure is used to update shadow fading values for mobile UEs:

- For each gNB in the simulation n=1,2,…,NNB:

  - Establish a grid of location points Dn that covers the area in which mobile UE trajectories will be constrained.

  - On top of Dn generate a grid of correlated shadow fading values SFn~N0, σsf, where σsf is the scenario/LoS-state-specific shadow fading standard deviation provided in TR 38.901, Table 7.5-6, and the R.V.s at each grid point are correlated in distance by RLOSΔd=exp-Δddcor .

- During the channel update for each mobile UE i=1,2,…,NUE:

  - For each gNB in the simulation n=1,2,…,NNB, interpolate the LoS and NLoS shadow fading values for the UE-gNB link from SFn given the mobile UE’s xy-location and UE-gNB link LoS.

Figure 6 shows an example pair of LoS and NLoS shadow fading grids for a UMi simulation.

Figure 6 Example LoS (left) and NLoS (right) shadow fading grids, UMi

## Derivation of cell-level metrics

For a given UE-gNB link, given measured RSRPs across all Tx-Rx beam pairs and polarizations, the cell-level metric for RRM prediction is derived from Tx-Rx beam pair RSRPs via the following procedure:

- RSRPs are summed across Rx polarizations, and the maximum RSRP is taken across Tx polarizations.

- For the given site with 3 co-located gNBs, the cell level metric Mn is calculated as

Mn=m1,n,m2,n,m3,n, where

- mi,n=1Nbj=1Nbri,j,n,

- ri,j,n is the RSRP of the jth top SSB for the ith sector at measurement instance n.

By taking the maximum RSRP across Rx beams, we assume a genie Rx beam selection algorithm that provides an upper-bound approximation for a realizable Rx beam selection algorithm.

The number of top beams averaged for this metric Nb is chosen to be Nb=3 for FR2 simulations and Nb=1 for FR1 simulations.

The resulting metrics are filtered in time via the Layer 3 filtering procedure outlined in 3GPP TS-38.331 [6], Section 5.5.3.2:

F0=M0

Fn=1-aFn-1+aMn, where

- Fn is the filtered cell metric result at measurement instance n,

- a=12k4, where k is the filter coefficient for the corresponding measurement quantity.

For the simulations discussed in this document, a filtering coefficient of k=4 is used to filter cell-level metrics. Figure 7 shows the effect of L3 filtering on calculated cell-level metrics for a given UE trajectory.

Figure 7 Cell-level metrics with and without L3 filtering, k=4

# Simulation evaluation for FR2 intra-frequency, intra-cell, temporal domain case A

In this section, we discuss the details of the simulation assumptions, parameters, and methodology for FR2 intra-frequency, intra-cell, temporal domain case A, and discuss the simulation results. The goal of the study of this scenario is to determine the achievable RRM measurement prediction accuracy.

## Trajectory generation

For FR2 intra-cell, intra-frequency case A dataset generation, we use the trajectory generation formulation outlined in Section 2.1 of this document. Table 2 lists the parameters used in this case.

Table 2: UE trajectory generation parameters for FR2 intra-frequency, intra-cell temporal case A

| Parameter | Value |
| --- | --- |
| tstep | 1 s |
| tgran | 0.01 s |
| tmax | 80 s |
| s | 60 km/h |
| dNb | 10 m |
| ISD | 200 m |

As we do not consider inter-cell mobility for this dataset, trajectories are constrained to the 3 geometric sectors corresponding to the gNBs located in the center x,y=0, 0 of the 2-tier model. Figure 8 visualizes several example random trajectories for this constrained case.

Figure 8 Example random trajectories, FR2 intra-frequency, intra-cell temporal case A

## Soft LoS and shadow fading grids

For this case, we use a single soft LoS grid to determine LoS states for mobile UEs across the entire dataset, modelling the LoS states between all mobile UEs in the dataset and the gNBs located at (0,0). Similarly, we use a single pair of LoS/NLoS shadow fading grids to model the shadow fading states between the mobile UEs and these gNBs. Figure 9 shows the soft LoS grid used for this dataset, and Figure 10 shows the shadow fading grids.

Figure 9 LoS grid used in dataset generation for FR2 intra-frequency, intra-cell temporal case A

Figure 10 LoS (left) and NLoS (right) shadow fading grids used in dataset generation for FR2 intra-frequency, intra-cell temporal case A

To evaluate the impact of shadow fading updates on predictor accuracy, two separate versions of the FR2 dataset were generated: one version with shadow fading updates per the grid-based approach outlined in Section 2.3, and one version in which shadow fading values remain static for mobile UEs. AI/ML predictors were trained separately on each of these datasets.

## Summary of simulation assumptions

Table 3 summarizes FR2 simulation assumptions and parameters.

Table 3: Simulation assumptions for FR2 intra-frequency, intra-cell, temporal domain case A

| Parameter | Value |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz; SCS: 120 kHz |
| Deployment | 200m ISD, 1 site, 3 sectors/cells per site |
| Channel model | UMi with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. Fast fading is considered for all the RRM sub-cases. Do not consider Oxygen absorption (7.6.1), Time-varying Doppler shift (7.6.6), Explicit ground reflection model (7.6.8), and blockage (7.6.4), for channel modelling (38.901). Soft LoS and Shadow Fading aspects of Channel modelling are discussed in Sections 2.1.2 and 2.1.3 in this document. |
| System BW | 80MHz |
| UE Speed | 60 km/h |
| UE distribution | 100% outdoor. UEs are randomly dropped within a cell with uniform distribution. |
| BS Antenna Configuration | Antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ. Number of BS beams: 8 SSB beams per sector Beam pattern: 8 azi. beams with 22.5-deg. separation |
| BS Antenna radiation pattern | TR 38.802 Table A.2.1-6. |
| UE Antenna Configuration | Antenna setup and port layouts at UE: (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) Number of UE beams: 4 Rx per UE panel Beam pattern: 4 beams with 45-deg. separation |
| UE Antenna radiation pattern | TR 38.802 Table A.2.1-8. |
| BS Tx Power | 40dBm |
| Maximum UE Tx Power | 23dBm |
| BS receiver Noise Figure | 7dB |
| UE receiver Noise Figure | 10dB |
| Inter site distance | 200m |
| BS Antenna height | 10m |
| UE Antenna height | 1.5m |
| Spatial consistency | Procedure A in TR38.901 |
| UE trajectory model | Based on Option 2 in TR 38.343 [2], Section 6.3.1, with modifications based on Option 4 and a minor modification for extended mobility. Refer to section 2.1.1 for a detailed discussion of the trajectory model. |
| L1 sampling period | 20ms |
| Prediction windows | 400 ms, 600 ms, 800 ms, 1000 ms |

## Simulation dataset evaluation

Our dataset for FR2 intra-frequency, intra-cell, temporal domain case A consists of 6003 total UE trajectories, split into 4801 training samples and 1202 test samples. Table 4 provides statistics for the trajectories within the dataset; Figure 11 shows a CDF of the trajectory lengths.

Table 4: FR2 dataset trajectory statistics

| Parameter | Value |
| --- | --- |
| # of trajectories (total) | 6003 |
| Training/test split | 80/20 (4801 training trajectories, 1202 test trajectories) |
| Mean trajectory length (s) | 5.57 |
| Median trajectory length (s) | 4.78 |
| Min. trajectory length (s) | 0.02 |
| Max. trajectory length (s) | 27.18 |

Figure 11 Empirical CDF of FR2 dataset trajectory lengths

To evaluate the generated dataset for FR2, in Figure 12 we plot the cell level metrics for each sector as a function of UE position across the dataset. These plots demonstrate the impacts of UE-gNB distance, gNB panel azimuthal pointing angle, and soft LoS state on the cell-level metric values.

Figure 12 Cell-level metric for each sector as functions of UE position, FR2 intra-frequency, intra-cell temporal case A dataset

## AI/ML model and methodology

Our AI/ML predictor is an encoder-only transformer model. In the RRM cell-level metric case A prediction problem, given measured and filtered cell-level metrics at a set of time instances within a sliding observation window, the model predicts future cell-level metric values at time instances within a sliding prediction window. Figure 13 visualizes the observation/prediction windows in the case A prediction problem:

Figure 13 sliding observation/prediction windows in temporal domain case A prediction

We compare AI/ML prediction results to prediction results from a baseline sample-and-hold prediction scheme, where the latest measured cell-level metrics are used as predictions for future metric values. We consider a variety of prediction windows from 100 ms to 2000 ms in length to understand how the performance of AI/ML predictors compares to this baseline as the prediction window length grows.

To evaluate predictor performance, we consider the following metrics:

- Average L3 cell level RSRP difference across prediction window (dB): the mean absolute error in dB between the predicted and measured cell-metric values across all predicted instances in the prediction window.

- Average L3 cell level RSRP difference at prediction window edge (dB): the mean absolute error in dB between the predicted and measured cell-metric values for the final predicted instance in the prediction window (the prediction window “edge”).

- Top sector prediction accuracy across prediction window (%): the average prediction rate accuracy for the sector with the top RSRP across all predicted instances in the prediction window.

- Top sector prediction accuracy at prediction window edge (%): the average prediction rate accuracy for the sector with the top RSP for the final predicted instance in the prediction window (the prediction window “edge”).

## Simulation results

The following table summarizes simulation results for FR2 intra-cell, intra-frequency, temporal Case A prediction.

Table 5: Simulation results for FR2 intra-frequency, intra-cell, temporal domain case A

| Report parameters |  | Qualcomm |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option | Option 2 in TR 38.343 [2], Section 6.3.1, with modifications based on Option 4 and a minor modification for extended mobility |  |  |  |  |  |  |  |
|  | UE trajectory boundary processing | Trajectory termination upon reaching the boundary |  |  |  |  |  |  |  |
|  | UE speed (30,60,90,120 Km/h) | 60 km/h |  |  |  |  |  |  |  |
|  | Inter-frequency correlation assumption in general (yes or no) | N/A |  |  |  |  |  |  |  |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no) | N/A |  |  |  |  |  |  |  |
|  | Whether soft LoS is modeled or not | Yes |  |  |  |  |  |  |  |
|  | Spatial consistency option (A or B) | A |  |  |  |  |  |  |  |
|  | Number of TX beams | 8 |  |  |  |  |  |  |  |
|  | Number of RX beams | 4 |  |  |  |  |  |  |  |
|  | Measurement reduction rate | N/A |  |  |  |  |  |  |  |
|  | Observation window | <= 1280 ms |  |  |  |  |  |  |  |
|  | Prediction windows | 100, 200, 400, 600, 800, 1000 ms |  |  |  |  |  |  |  |
| Data Size (Number of Samples) | Training/validity | 4801 trajectories |  |  |  |  |  |  |  |
|  | Testing | 1202 trajectories |  |  |  |  |  |  |  |
| AI/ML model input/output | Model input | Past measured L3-filtered cell-level metric values |  |  |  |  |  |  |  |
|  | Model output | Prediction for future L3-filtered cell-level metric values |  |  |  |  |  |  |  |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | Transformer |  |  |  |  |  |  |  |
|  | Model complexity in a number of parameters(M) | 1.28M parameters |  |  |  |  |  |  |  |
|  | Model complexity in model size (e.g. Mbyte) | 9.8 Mbytes |  |  |  |  |  |  |  |
| Metrics (shadow fading updates enabled) | Average L3 cell level RSRP difference across prediction window (dB) | Prediction window (ms) | 100 | 200 | 400 | 600 | 800 | 1000 | 2000 |
|  |  | AI/ML | 1.99 | 2.82 | 3.90 | 4.53 | 4.94 | 5.40 | 6.82 |
|  |  | Baseline (sample-and-hold) | 1.98 | 2.79 | 3.87 | 4.62 | 5.22 | 5.72 | 7.44 |
|  | Average L3 cell level RSRP difference at prediction window edge (dB) | Prediction window (ms) | 100 | 200 | 400 | 600 | 800 | 1000 | 2000 |
|  |  | AI/ML | 2.76 | 4.03 | 5.38 | 6.20 | 6.75 | 7.50 | 9.38 |
|  |  | Baseline (sample-and-hold) | 2.80 | 4.09 | 5.65 | 6.74 | 7.62 | 8.35 | 10.99 |
|  | Top sector prediction accuracy across prediction window (%) | Prediction window (ms) | 100 | 200 | 400 | 600 | 800 | 1000 | 2000 |
|  |  | AI/ML | 94.35 | 93.24 | 91.71 | 90.13 | 88.70 | 87.15 | 80.82 |
|  |  | Baseline (sample-and-hold) | 94.70 | 93.38 | 91.28 | 89.43 | 87.73 | 86.16 | 79.48 |
|  | Top sector prediction accuracy at prediction window edge (%) | Prediction window (ms) | 100 | 200 | 400 | 600 | 800 | 1000 | 2000 |
|  |  | AI/ML | 93.63 | 91.68 | 88.89 | 86.25 | 83.87 | 81.36 | 71.74 |
|  |  | Baseline (sample-and-hold) | 93.89 | 91.63 | 87.97 | 84.71 | 81.88 | 79.42 | 69.51 |
| Metrics (shadow fading updates disabled) | Average L3 cell level RSRP difference across prediction window (dB) | Prediction window (ms) | 100 | 200 | 400 | 600 | 800 | 1000 | 2000 |
|  |  | AI/ML | 1.56 | 2.03 | 2.85 | 3.20 | 4.13 | 4.33 | 5.39 |
|  |  | Baseline (sample-and-hold) | 1.34 | 1.76 | 2.43 | 2.98 | 3.48 | 3.92 | 5.63 |
|  | Average L3 cell level RSRP difference at prediction window edge (dB) | Prediction window (ms) | 100 | 200 | 400 | 600 | 800 | 1000 | 2000 |
|  |  | AI/ML | 1.94 | 2.65 | 3.83 | 4.47 | 5.59 | 6.03 | 7.81 |
|  |  | Baseline (sample-and-hold) | 1.75 | 2.47 | 3.62 | 4.60 | 5.45 | 6.20 | 9.11 |
|  | Top sector prediction accuracy across prediction window (%) | Prediction window (ms) | 100 | 200 | 400 | 600 | 800 | 1000 | 2000 |
|  |  | AI/ML | 94.40 | 93.39 | 91.16 | 89.95 | 87.92 | 86.11 | 79.77 |
|  |  | Baseline (sample-and-hold) | 94.98 | 93.67 | 91.61 | 89.77 | 88.08 | 86.51 | 79.69 |
|  | Top sector prediction accuracy at prediction window edge (%) | Prediction window (ms) | 100 | 200 | 400 | 600 | 800 | 1000 | 2000 |
|  |  | AI/ML | 93.75 | 91.85 | 88.32 | 85.99 | 82.67 | 80.39 | 70.42 |
|  |  | Baseline (sample-and-hold) | 94.17 | 91.91 | 88.25 | 85.01 | 82.13 | 79.55 | 69.09 |

Figure 14 plots the MAEs for transformers and sample-and-hold baseline predictors both across the prediction window as well as at the prediction window edge in the case when shadow fading updates are enabled.

Figure 14 Prediction MAE as a function of prediction window length for AI/ML (transformer) vs. baseline (sample-and-hold) predictors, shadow fading updates enabled

Figure 15 plots the empirical CDFs of MAE errors for transformers and sample-and-hold baseline predictors for each of the prediction windows considered in the case where shadow fading updates are enabled. At shorter (< 800 ms) prediction windows, the transformer predictions do not provide any advantage over the baseline predictors, but they provide a slight advantage at longer prediction window lengths (1000 ms, 2000 ms).

Figure 15 CDFs of MAE for AI/ML (transformer) vs. baseline (sample-and-hold) predictors, shadow fading updates enabled

Figure 16 compares AI/ML vs. baseline RRM prediction MAE for the 2000-ms predictor as a function of cycles into the future within the prediction window in the case where shadow fading updates are enabled. We note that while the MAE for both the AI/ML and baseline predictors increases as a function of distance into the future, the AI/ML predictor gains an increasing advantage over the baseline predictor as distance into the future increases, gaining a ~2-dB advantage at the 2000-ms mark.

Figure 16 RRM prediction MAE as a function of prediction window cycle for 2000-ms predictors, shadow fading updates enabled

Observation 4: AI/ML cell metric prediction gains an increasing advantage over a sample-and-hold baseline as a function of prediction distance into the future in the case of FR2 intra-frequency, intra-cell, temporal domain Case A prediction.

Figure 17 plots the accuracy with which these predictors predict the sector with the highest metric in the case where shadow fading updates are enabled, both at every instance in the prediction window (dashed line) as well as at edge of the prediction window (dotted line). As the prediction window length grows, accuracy for both the AI/ML and baseline predictors decrease; however, the AI/ML predictors achieve a slight advantage over the baseline sample-and-hold predictors, and the performance gap increases as a function of prediction window length.

Figure 17 Top sector prediction accuracy (%) as a function of prediction window length for AI/ML (transformer) vs. baseline (sample-and-hold) predictors, shadow fading updates enabled

Figure 18 plots MAEs for transformers and sample-and-hold baseline predictors both across the prediction window as well as at the prediction window edge in the case when shadow fading updates are disabled. We note that the MAEs for both AI/ML and baseline predictors are reduced by ~0.5-1 dB compared to corresponding MAEs in the case when shadow fading updates are enabled. These results demonstrate that different choices in shadow fading update modelling in mobility simulations can lead to significant differences in results for temporal predictors.

Figure 18 Prediction MAE as a function of prediction window length for AI/ML (transformer) vs. baseline (sample-and-hold) predictors, shadow fading updates disabled

Observation 5: The choice to model updating vs. static shadow fading values for mobile UEs can lead to significant differences in error values for temporal predictors.

# Simulation evaluation for FR1 intra-frequency, intra-cell, temporal domain case B

In this section, we discuss the details of the simulation assumptions, parameters, and methodology for FR1 intra-frequency, intra-cell, temporal domain case B, and discuss the simulation results. The goal of the study of this scenario is measurement reduction.

## Trajectory generation

For FR1 intra-frequency, intra-cell case B dataset generation, we use the trajectory generation formulation outlined in Section 2.1 of this document. Table 6 lists the parameters used in this case.

Table 6: UE trajectory generation parameters for FR1 intra-frequency, intra-cell temporal case A

| Parameter | Value |
| --- | --- |
| tstep | 1 s |
| tgran | 0.01 s |
| tmax | 160 s |
| s | 60 km/h |
| dNb | 35 m |
| ISD | 500 m |

As with FR2 intra-frequency, intra-cell case A data generation, trajectories are constrained to the 3 geometric sectors corresponding to the gNBs located in the center x,y=0, 0 of the 2-tier model. Figure 19 visualizes several example random trajectories for this constrained case.

Figure 19 Example random trajectories, FR1 intra-frequency, intra-cell temporal case B

## Soft LoS and shadow fading grids

As with FR2 intra-frequency, intra-cell case A data generation, we use a single soft LoS grid to determine LoS states for mobile UEs across the entire dataset and a single pair of LoS/NLoS shadow fading grids to model the shadow fading states between the mobile UEs and these gNBs. Figure 20 shows the soft LoS grid used for this dataset, and Figure 21 shows the shadow fading grids.

Figure 20 LoS grid used in dataset generation for FR1 intra-frequency, intra-cell temporal case B

Figure 21 LoS (left) and NLoS (right) shadow fading grids used in dataset generation for FR1 intra-frequency, intra-cell temporal case B

## Summary of simulation assumptions

The table below summarizes the simulation assumptions and parameters for FR1 intra-frequency, intra-cell temporal domain case B.

Table 7: Simulation assumptions for FR1 intra-frequency, intra-cell, temporal domain case B

| Parameter | Value |
| --- | --- |
| Frequency Range | Intra-frequency: (4GHz, 30KHz SCS). |
| Deployment | 500m ISD, 1 site, 3 sectors/cells per site |
| Channel model | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. Fast fading is considered for all the RRM sub-cases. Do not consider Oxygen absorption (7.6.1), Time-varying Doppler shift (7.6.6), Explicit ground reflection model (7.6.8), and blockage (7.6.4), for channel modelling (38.901). SoftLOS and Shadow Fading aspects of Channel modelling are discussed in the sections above. |
| System BW | 20MHz |
| UE Speed | 60 km/h |
| UE distribution | 100% outdoor. UEs are randomly dropped within a cell with uniform distribution. |
| BS Antenna Configuration | 1 TX antenna per panel 1 panel per sector |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern, 8 dBi |
| UE Antenna Configuration | 1 RX antenna per panel 2 panels |
| UE Antenna radiation pattern | Omni-direction |
| BS Tx Power | 44dBm |
| Maximum UE Tx Power | 23dbm |
| BS receiver Noise Figure | 5db |
| UE receiver Noise Figure | 9dB |
| Inter site distance | 500m |
| BS Antenna height | 25m |
| UE Antenna height | Follow TR36.873, which is 1.5m |
| Spatial consistency | Procedure A in TR38.901 |
| UE trajectory model | Based on Option 2 in TR 38.343 [2], Section 6.3.1, with modifications based on Option 4 and a minor modification for extended mobility. Refer to section 2.1.1 for a detailed discussion of the trajectory model. |
| L1 sampling period | 40ms |
| Prediction windows | 200ms, 400 ms, 600 ms, 800 ms |

## Simulation dataset evaluation

In Figure 22 we plot the cell level metrics for each sector as a function of UE position across the FR1 intra-frequency, intra-cell case B dataset. These plots demonstrate the impacts of UE-gNB distance, gNB panel azimuthal pointing angle, and soft LoS state on the cell-level metric values.

Figure 22 Cell-level metric for each sector as functions of UE position, FR1 intra-frequency, intra-cell temporal case B dataset

## AI/ML model and methodology

For the RRM cell-level metric case B prediction problem, we consider a repeating measurement pattern in which cell-level metrics are measured at x contiguous time instances followed by y contiguous time instances at which no measurements are taken, referred to as an “MxPy” measurement pattern. For given values of x and y, the corresponding measurement reduction rate can be calculated as yx+y.

Given this measurement pattern, the prediction problem can be stated as follows: given past cell metric measurements taken at measured beam management cycles, predict cell level metrics for the next y prediction cycles. Figure 23 visualizes cell-level metric prediction within an MxPy beam measurement pattern framework.

Figure 23 Illustration of Case B prediction within an MxPy measurement pattern framework

In this setting, the observation window (OW) can be defined as the period of time from the present stretching into the past containing all measurements used by the predictor, and the prediction window (PW) can be defined as the period of time containing the next y predicted beam management cycles.

Our AI/ML predictor is an encoder-only transformer model. We compare AI/ML prediction results to prediction results from a baseline sample-and-hold prediction scheme, where the latest measured cell-level metrics are used as predictions for future metric values. We consider a set of prediction window lengths from 200 ms to 800 ms in length to understand how the performance of AI/ML predictors compares to this baseline as the prediction window length grows.

To evaluate predictor performance, we consider the following metrics:

- Average L3 cell level RSRP difference (dB): the mean absolute error in dB between the predicted and measured cell-metric values.

- Average L3 cell level RSRP difference at prediction window edge (dB): the mean absolute error in dB between the predicted and measured cell-metric values for the final predicted instance in the prediction window (the prediction window “edge”).

- Top sector prediction accuracy across prediction window (%): the average prediction rate accuracy for the sector with the top RSRP across all predicted instances in the prediction window.

- Top sector prediction accuracy at prediction window edge (%): the average prediction rate accuracy for the sector with the top RSP for the final predicted instance in the prediction window (the prediction window “edge”).

## Simulation results

The following table summarizes simulation results for FR1 intra-cell, intra-frequency, temporal Case B prediction.

Table 8: Simulation results for FR1 intra-frequency, intra-cell, temporal domain case B

| Report parameters |  | Qualcomm |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option | Option 2 in TR 38.343 [2], Section 6.3.1, with modifications based on Option 4 and a minor modification for extended mobility |  |  |  |  |
|  | UE trajectory boundary processing option | Trajectory termination |  |  |  |  |
|  | UE speed (30,60,90,120 Km/h) | 60 km/h |  |  |  |  |
|  | Inter-frequency correlation assumption in general (yes or no) | N/A |  |  |  |  |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no) | N/A |  |  |  |  |
|  | Whether soft LoS is modeled or not | Yes |  |  |  |  |
|  | Spatial consistency option (A or B) | A |  |  |  |  |
|  | Number of TX beams | 1 |  |  |  |  |
|  | Number of RX beams | 1 |  |  |  |  |
|  | Measurement reduction rates | 80%, 90%, 93.3%, 95% |  |  |  |  |
|  | Observation window | <=3200, <=6400, <=9600, <=12800 ms |  |  |  |  |
|  | Prediction window (Note3) | 200, 400, 600, 800 ms |  |  |  |  |
|  | Any other parameters | N/A |  |  |  |  |
| Data Size (Number of Samples) | Training/validity | 4801 |  |  |  |  |
|  | Testing | 1202 |  |  |  |  |
| AI/ML model input/output | Model input | Up to 16 sets of past measured L3-filtered cell-level metric values |  |  |  |  |
|  | Model output | Prediction for future L3-filtered cell-level metric values |  |  |  |  |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | Transformer |  |  |  |  |
|  | Model complexity in a number of parameters (K) | 82.2K parameters |  |  |  |  |
|  | Model complexity in model size (e.g. Mbyte) | 642.8 kB |  |  |  |  |
| Metrics | Average L3 cell level RSRP difference across prediction window (dB) | Prediction window (ms) | 200 | 400 | 600 | 800 |
|  |  | AI/ML | 2.37 | 3.40 | 4.06 | 4.61 |
|  |  | Baseline (sample-and-hold) | 2.27 | 3.20 | 3.93 | 4.48 |
|  | Average L3 cell level RSRP difference at prediction window edge (dB) | Prediction window (ms) | 200 | 400 | 600 | 800 |
|  |  | AI/ML | 3.17 | 4.62 | 5.44 | 6.06 |
|  |  | Baseline (sample-and-hold) | 3.09 | 4.46 | 5.42 | 6.10 |
|  | Top sector prediction accuracy across prediction window (%) | Prediction window (ms) | 200 | 400 | 600 | 800 |
|  |  | AI/ML | 95.08 | 93.91 | 93.03 | 92.21 |
|  |  | Baseline (sample-and-hold) | 95.19 | 94.06 | 93.27 | 92.54 |
|  | Top sector prediction accuracy at prediction window edge (%) | Prediction window (ms) | 200 | 400 | 600 | 800 |
|  |  | AI/ML | 94.32 | 92.57 | 91.27 | 90.25 |
|  |  | Baseline (sample-and-hold) | 94.42 | 92.67 | 91.57 | 90.48 |

Figure 24 plots the MAE errors for transformers and sample-and-hold baseline predictors both across the prediction window as well as at the prediction window edge in the case when shadow fading updates are enabled.

Figure 24 Prediction MAE as a function of prediction window length for AI/ML (transformer) vs. baseline (sample-and-hold) predictors

Figure 25 plots the empirical CDFs of MAE errors for transformers and sample-and-hold baseline predictors for each of the prediction windows considered.

Figure 25 CDFs of MAE for AI/ML (transformer) vs. baseline (sample-and-hold) predictors

Figure 26 plots the accuracy with which these predictors predict the sector with the highest metric, both at every instance in the prediction window (dashed line) as well as at edge of the prediction window (dotted line). In both cases, the sample-and-hold predictor slightly outperforms the AI/ML predictor.

Figure 26 Top sector prediction accuracy (%) as a function of prediction window length for AI/ML (transformer) vs. baseline (sample-and-hold) predictors

Observation 6: Sample-and-hold predictors are competitive with AI/ML predictors in the case of FR1 intra-frequency, intra-cell, temporal domain Case B prediction, with both demonstrating high accuracy for extended observation windows.

# Simulation evaluation for FR1 inter-frequency (frequency domain) prediction

In this section, we discuss simulation assumptions, parameters, and methodology for FR1 inter-frequency prediction and discuss the simulation results. For this study, we consider the intra-site frequency domain prediction problem in which cell-level metrics measured with center frequency f1 = 2 GHz for a mobile UE are used to predict cell-level metrics for the same UE-Nb pair at f2 = 4 GHz. The goal of the study is measurement reduction.

## Trajectory generation

For FR1 inter-frequency dataset generation, we use the trajectory generation formulation outlined in Section 2.1 of this document. Table 9 lists the parameters used in this case.

Table 9: UE trajectory generation parameters for FR1 inter-frequency prediction

| Parameter | Value |
| --- | --- |
| tstep | 1 s |
| tgran | 0.01 s |
| tmax | 160 s |
| s | 60 km/h |
| dNb | 35 m |
| ISD | 500 m |

For inter-frequency prediction, trajectories are constrained to a single geometric sector corresponding to the gNB located in the center x,y=0, 0 of the 2-tier model with an azimuthal pointing angle of 30 degrees. Figure 27 visualizes two example random trajectories for this constrained case.

Figure 27 Single-sector trajectory examples for inter-frequency prediction datasets

## Simulation assumptions

For inter-frequency channel modelling, we assume that certain aspects of the 2-GHz and 4-GHz channels are perfectly correlated, while other modelling aspects are assumed to be uncorrelated. Correlated channel modelling aspects without a frequency dependency are identical; for channel modelling aspects with a frequency dependency, the underlying random variables used to generate the channel aspects are identical, although the final values may differ between frequencies. Table 10 summarizes inter-frequency correlation assumptions for various modelling aspects.

Table 10: Inter-frequency correlation assumptions for FR1 inter-frequency prediction

| Channel modelling aspect | Inter-frequency correlation assumption |
| --- | --- |
| UE locations/trajectories | Perfect correlation (identical) |
| Delay spreads (DS) | Perfect correlation of underlying random variables |
| Angular spreads (ASA, ASD, ZSA, ZSD) | Perfect correlation of underlying random variables |
| Shadow fading | Perfect correlation (identical) |
| Soft line-of-sight (LoS) state | Perfect correlation of underlying random variables |
| Per-cluster shadowing | Perfect correlation (identical) |
| Per-cluster arrival/departure angles (AoA, AoD, ZoA, ZoD) | Perfect correlation (identical) |
| Per-cluster random phases | Uncorrelated |

For both 2-GHz and 4-GHz simulations, we re-use the shadow fading grid generated for the FR1 intra-frequency temporal domain case B study (Section 4.2).

For 4-GHz simulations, we re-use the soft LoS grid generated for the FR1 intra-frequency temporal domain case B study (Section 4.2). We model correlation between the 2-GHz and 4-GHz soft LoS states by re-using the underlying Gaussian random variables used in generating the 4-GHz LoS grid: letting G4GHz be the set of Gaussians used to generate the 4-GHz LoS grid, the 2-GHz soft LoS grid LOSsoft,2GHz  is generated using the soft LoS formula provided in TR 38.901 [4], Section 7.6.3.3 as

LOSsoft,2GHz+1πarctan20λ2GHzG4GHz +Fd

where λ2GHz=0.15 m.

We introduce a delta of 1 meter between 2-GHz and 4-GHz antennas by using gNB antenna heights of 24 m and 25 m in 2-GHz and 4-GHz simulations, respectively.

Table 11 summarizes simulation assumptions for 2-GHz and 4-GHz simulations.

Table 11: Simulation assumptions for FR1 inter-frequency prediction

| Parameter | Value (2 GHz) | Value (4 GHz) |
| --- | --- | --- |
| Frequency Range | 2 GHz, 30KHz SCS | 4 GHz, 30KHz SCS |
| Deployment | 500m ISD, 1 site, 3 sectors/cells per site | 500m ISD, 1 site, 3 sectors/cells per site |
| Channel model | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. Fast fading is considered for all the RRM sub-cases. Do not consider Oxygen absorption (7.6.1), Time-varying Doppler shift (7.6.6), Explicit ground reflection model (7.6.8), and blockage (7.6.4), for channel modelling (38.901). SoftLOS and Shadow Fading aspects of Channel modelling are discussed in the sections above. | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. Fast fading is considered for all the RRM sub-cases. Do not consider Oxygen absorption (7.6.1), Time-varying Doppler shift (7.6.6), Explicit ground reflection model (7.6.8), and blockage (7.6.4), for channel modelling (38.901). SoftLOS and Shadow Fading aspects of Channel modelling are discussed in the sections above. |
| System BW | 20MHz | 20MHz |
| UE Speed | 60 km/h | 60 km/h |
| UE distribution | 100% outdoor. UEs are randomly dropped within a cell with uniform distribution. | 100% outdoor. UEs are randomly dropped within a cell with uniform distribution. |
| BS Antenna Configuration | 1 TX antenna per panel 1 panel per sector | 1 TX antenna per panel 1 panel per sector |
| BS Antenna radiation pattern | 1-sector antenna radiation pattern, 8 dBi | 1-sector antenna radiation pattern, 8 dBi |
| UE Antenna Configuration | 1 RX antenna per panel 1 panel | 1 RX antenna per panel 1 panel |
| UE Antenna radiation pattern | Omni-direction | Omni-direction |
| BS Tx Power | 44dBm | 44dBm |
| Maximum UE Tx Power | 23dbm | 23dbm |
| BS receiver Noise Figure | 5db | 5db |
| UE receiver Noise Figure | 9dB | 9dB |
| Inter site distance | 500m | 500m |
| BS Antenna height | 24m (1 meter lower than 4 GHz) | 25m |
| UE Antenna height | Follow TR36.873, which is 1.5m | Follow TR36.873, which is 1.5m |
| Spatial consistency | Procedure A in TR38.901 | Procedure A in TR38.901 |
| UE trajectory model | Based on Option 2 in TR 38.343 [2], Section 6.3.1, with modifications based on Option 4 and a minor modification for extended mobility. Refer to section 2.1.1 for a detailed discussion of the trajectory model. | Based on Option 2 in TR 38.343 [2], Section 6.3.1, with modifications based on Option 4 and a minor modification for extended mobility. Refer to section 2.1.1 for a detailed discussion of the trajectory model. |
| L1 sampling period | 40ms | 40ms |

## Simulation dataset evaluation

Our dataset for FR1 inter-frequency prediction consists of 3304 total UE trajectories, split into 2644 training samples and 660 test samples. Table 12 provides statistics for the trajectories within the dataset; Figure 28 shows a CDF of the trajectory lengths.

Table 12: FR2 dataset trajectory statistics

| Parameter | Value |
| --- | --- |
| # of trajectories (total) | 3304 |
| Training/test split | 80/20 (2644 training trajectories, 660 test trajectories) |
| Mean trajectory length (s) | 8.77 |
| Median trajectory length (s) | 7.72 |
| Min. trajectory length (s) | 0.08 |
| Max. trajectory length (s) | 50.8 |

Figure 28 Empirical CDF of FR2 dataset trajectory lengths

Across the training and test datasets, the L3-filtered 2-GHz and 4-GHz cell metrics have a Pearson correlation coefficient of 0.966. Figure 29 plots 2-GHz and 4-GHz cell-level metrics for a single mobile UE (with and without L3 filtering).

Figure 29 2-GHz and 4-GHz cell-level metrics with and without L3 filtering

## AI/ML model and methodology

We consider two subproblems for AI/ML inter-frequency prediction:

- Given past cell-metric measurements on a given frequency f1, predict the current cell level metric on frequency f2.

- Given past cell-metric measurements on a given frequency f1 (without subsampling) as well as sparse cell-metric measurements on f2, predict cell-metric measurements on f2 for unmeasured cycles. For this subproblem, we consider an “MxPy” cell-metric measurement pattern on f2 (as defined in Section 4.5.)

For our study, we choose f1=2 GHz and f2 = 4 GHz.

For Subproblem 1, as f2 is never measured, a measurement reduction rate of 50% is achieved. For Subproblem 2, for a given choice of MxPy the measurement reduction rate can be calculated as  y2(x+y). In our study, we consider an M1P4 measurement pattern, providing a measurement reduction rate of 421+4=40%.

Figure 30 illustrates the measurement and prediction patterns for Subproblem 1, and Figure 31 illustrates the measurement and prediction patterns for Subproblem 2.

Figure 30 Illustration of inter-frequency prediction and measurement pattern (Subproblem 1)

Figure 31 Illustration of inter-frequency prediction and measurement pattern (Subproblem 2)

## Simulation results

The following table summarizes simulation results for FR1 inter-frequency prediction.

Table 13: Simulation results for FR1 inter-frequency prediction

| Report parameters |  | Qualcomm |  |  |
| --- | --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option | Option 2 in TR 38.343 [2], Section 6.3.1, with modifications based on Option 4 and a minor modification for extended mobility |  |  |
|  | UE trajectory boundary processing | Trajectory termination upon reaching the boundary |  |  |
|  | UE speed (30,60,90,120 Km/h) | 60 km/h |  |  |
|  | Inter-frequency correlation assumption in general (yes or no) | Yes |  |  |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no) | Full |  |  |
|  | Whether soft LoS is modeled or not | Yes |  |  |
|  | Spatial consistency option (A or B) | A |  |  |
|  | Number of TX beams | 1 |  |  |
|  | Number of RX beams | 1 |  |  |
|  | Measurement reduction rate | 50% (Subproblem 1), 40% (Subproblem 2) |  |  |
|  | Observation window | <=1280 ms |  |  |
|  | Prediction windows | N/A |  |  |
| Data Size (Number of Samples) | Training/validity | 2644 trajectories |  |  |
|  | Testing | 660 trajectories |  |  |
| AI/ML model input/output | Model input | Past measured L3-filtered cell-level metric values on f1 (Subproblem 1) Past measurement L-3 filtered cell level metric values on f1 and f2 (Subproblem 2) |  |  |
|  | Model output | Prediction for current L3-filtered cell-level metric values on f2 |  |  |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | Transformer |  |  |
|  | Model complexity in a number of parameters | 160K parameters |  |  |
|  | Model complexity in model size (e.g. Mbyte) | 1.22 Mb |  |  |
| Metrics | Average L3 cell level RSRP difference (dB) | Subproblem | 1 | 2 |
|  |  | AI/ML (Transformer) | 2.18 | 1.51 |

For Subproblem 1, the AI/ML predictor achieves an MAE of 2.18 dB for a measurement reduction rate of 50%. For Subproblem 2, the AI/ML predictor achieves an MAE of 1.51 dB for a measurement reduction rate of 40%. Figures 32 and 33 plot the empirical CDF of MAEs for Subproblems 1 and 2, respectively.

Figure 32 Empirical CDF of prediction MAE, Subproblem 1

Figure 33 Empirical CDF of prediction MAE, Subproblem 1

Observation 7: For inter-frequency prediction, sparse measurements on the predicted frequency can reduce AI/ML prediction error at the cost of a decreased measurement reduction rate.

# Conclusions

Observation 1: For extended (inter-cell) UE trajectories, maintaining static LoS states may result in improbable or invalid channel realizations as UEs move very far away from (or very near to) gNBs.

Observation 2: For extended (inter-cell) UE trajectories, static shadow fading values may result in improbable situations during handover.

Observation 3: Modelling LoS updates for mobile UEs can lead to large differences in channel measurement results compared to simulations with static LoS states. These differences could have significant ramifications in handover situations for mobile UEs that move far from their serving gNBs.

Observation 4: AI/ML cell metric prediction gains an increasing advantage over a sample-and-hold baseline as a function of prediction distance into the future in the case of FR2 intra-frequency, intra-cell, temporal domain Case A prediction.

Observation 5: The choice to model updating vs. static shadow fading values for mobile UEs can lead to significant differences in error values for temporal predictors.

Observation 6: Sample-and-hold predictors are competitive with AI/ML predictors in the case of FR1 intra-frequency, intra-cell, temporal domain Case B prediction, with both demonstrating high accuracy for extended observation windows.

Observation 7: For inter-frequency prediction, sparse measurements on the predicted frequency can reduce AI/ML prediction error at the cost of a decreased measurement reduction rate.

# References

[1] Chair notes, RAN2 #127-bis meeting.

[2] “3GPP TR 38.843 V18.0.0: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”, 3GPP, January 2024, https://portal.3gpp.org/desktopmodules/Specifications/SpecificationDetails.aspx?specificationId=3983 (accessed May 7, 2024).

[3] “3GPP RAN2 2405205: Considerations for mobile UE trajectory generation and channel modelling for simulation evaluation”, 3GPP, May 2024, 3gpp.org.

[4] “3GPP TR 38.901 V17.0.0: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on channel model for frequencies from 0.5 to 100 GHz (Release 17).”  3GPP RAN Plenary, January 2024, 3gpp.org.

[5] IST-WINNER II Deliverable 1.1.2 v.1.2, "WINNER II Channel Models", IST-WINNER2, Tech. Rep., 2007

[6] “3GPP TR 38.331 V18.2.0: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Radio Resource Control (RRC) protocol specification (Release 18).” 3GPP RAN Plenary, June 2024, 3gpp.org.