3GPP TSG RAN WG2#128	R2-2410781

Orlando - Orlando , US      18th- 22nd November 2024

Agenda item:	         8.3.2

Source:	CEWiT

Title:	Simulation results for RRM Measurement Prediction

Document for:	     Discussion and Decision

# Introduction

In the SID on AI/ML Mobility , it was agreed to study the simulation parameters and AI/ML methods for different sub use cases. Additionally, in the RAN2#127bis[1] meeting, more agreements were reached on defining the AI/ML terminologies, frameworks, use cases, evaluation methodologies, KPI’s etc.

Agreements on simulation table

1: 	The spreadsheets are organized into three separate folders, with each folder corresponding to one of the three use cases.

2: 	The RRM prediction use case is used as a template for the documentation process. This approach will be similarly applied to the measurement event prediction and RLF/HOF prediction use cases, the spreadsheets of which are subject to revision upon availability of the simulation results.

3: 	Individual spreadsheet for each identified scenario for the use case of RRM prediction is created, e.g., scenarios 1~6 with the understanding that we can add more spreadsheets as required e.g., when other scenarios are identified.

4: 	Distinct sheets are initially set up for capturing the simulation assumptions, evaluated KPIs and definitions, and simulation results from companies, with the understanding that we will add more sheets as needed and in accordance with discussions that emerge during the evaluation process.

5: 	The columns in the simulation results sheet are categorized into five main groups: general information, variable settings, selectable simulation assumption, model-related information and performance metrics for various KPIs.

6: 	The ‘case’ column considers the three sub-use cases and their combination with additional factors. One colume for additional factors will be introduced. Those factors can be determined through discussions as the evaluation progresses.

7:   Adopt the example spreadsheets provided in the attachment as a starting point, understanding that their content is flexible and can be modified as the evaluation progresses.

8:	Create a folder on the 3GPP FTP server for companies to upload their simulation results. Within this folder, create individual subfolders for different use cases. For each use case different subfolders under the main directory corresponds to the different identified scenairos.The file name of the excel table follows the format: 'MeetingNumber_CompanyName_TdocNumber_version number'.

- In this contribution we present our simulation results for RRM Measurement prediction in temporal domain for all the sub-use cases of case 2 and our observation.

# 2. Discussion

In this document, we will present our results for the temporal domain, covering Case B  for  all Sub Use cases.

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |

## 2.1 Evaluation Methodology

- Case 4 - FR1 to FR1 intra-frequency temporal domain case B

- In case B, measurement results in prediction window are predicted by historical measurement result(s) in observation window. Then observation window and prediction window slide forward with sampling period(s) (with sliding L1/L3 filtering option)  after measurement result(s) in previous prediction window is/are skipped.

- Figure 1: Temporal domain case B [2]

For all the sub-use cases,we have used the sliding L1/L3 filtering.

2.2 Evaluation assumptions

In our simulations we followed the simulation agreements that were agreed in the RAN2 previous meetings. The detailed simulation assumptions are given in the Appendix. According to the simulation assumptions, the gNB is equipped with one panel which has 128 antennas The gNB antenna array supports 1 transmitting beams,10 users in 2 tier model having 57 gNB. Measurement period as 10s and sample period 40ms.

The dataset was generated using system level simulations based on the simulation parameters given in the appendix. Out of the around 1.4M datasets generated using SLS, 80% of data points are used for training, 10% for validation and 10% for testing for each sector. Since the RSRP values can be quite varied, we normalize the RSRP values before giving it as input to the AI/ML model.

We modeled the ML model as neural network with three fully connected LSTM layer and one dense layer in output. The activation function used is linear. The DNN was trained for a maximum of 150 epochs with a batch size of 16. The loss function used was MSE(mean square error) and an Adam optimizer was used to minimize the loss.

In the subsequent discussion simulated results are given.

- 2.3 Simulation results

- The data is preprocessed with an observation window of 5 instances and a prediction window of one instance, the CDF plot reflects how well the model predicts the RSRP differences based on those 5 observations.

- The input to the model is a multi-dimensional tensor of shape (#Users, #Measured_Timesinstances, #TX_beams) and the output of the model is (#Predicted_Timeinstances, #TX_beams)

- 2.3.1 RRM sub case 1: To predict beam level results, then generate cell level results based on the predicted beam results

- In sub-use case 1, L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then cell-level measurement result is generated taking the L3 filtered RSRP instances.

- Average RSRP Difference: 1.1457842784599153

- 2.3.2 RRM sub-use case 2: Cell-level measurement result(s) is predicted based on actual cell-level measurement result(s)

- Average RSRP Difference: 0.83613485

- 2.3.3  RRM sub case 3: To directly predict cell level results based on beam level results.

Fig : CDF of average L3 RSRP difference

Average RSRP Difference: 1.322971

- The x-axis represents the average RSRP difference in dB

The goal of case B is to reduce the measurement. The prediction accuracy simulation results are shown in following table,  The samples within OW represent the measured samples, i.e., the number of yellow blocks in Figure 1

| MRRT=50% | Sub-case 1 | Sub-case 2 | Sub-case 3 |
| --- | --- | --- | --- |
| Filtering option | sliding | sliding | Sliding |
| Detailed skipping pattern | example 2 | example 2 | example 2 |
| OW | 240ms (3 samples) | 240ms (3 samples) | 240ms (3 samples) |
| PW | 40ms (1 sample) | 40ms (1 sample) | 40ms (1 sample) |
| Average L3 cell-level RSRP difference (dB) | 1.14 | 0.86 | 1.3 |

Observations :

1. Among sub cases 1, 2, and 3, sub case 2 demonstrates the highest prediction accuracy, with sub case 1 being the second best. The prediction error of sub cases 1 and 3 is more than 1.0 dB larger than that of case 2 at 30km/h.

# 3. Conclusion

1. Among sub cases 1, 2, and 3, sub case 2 demonstrates the highest prediction accuracy, with sub case 1 being the second best. The prediction error of sub cases 1 and 3 is more than 1.0 dB larger than that of case 2 at 30km/h.

# References

- [1] RAN2 Chair (InterDigital)

- [2] R2-2407849 Draft Summary of [POST127][030][AI mobility] RRM simulation assumptions (OPPO)

# Appendix

| Parameters | Values |
| --- | --- |
| Frequency Range | FR1 @ 4 GHz - SCS: 30 kHz |
| Deployment | 500m ISD, - 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Channel mode | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. |
| System BW | 20MHz |
| UE Speed | 30km/h |
| UE distribution | 100% outdoors |
| BS Antenna Configuration | [One panel: ( Mg, Ng, M, N, P,) = (1,1,8, 8, 2), (dV, dH) = (0.8, 0.5)λ as baseline] Azimuth angles (degrees) = [0] Zenith angles (degrees) = [102] |
| BS Tx Power | 44 dBm |
| Maximum UE Tx Power | 23 dBm |
| BS receiver Noise Figure | 5dB |
| UE receiver Noise Figure | 9 dB |
| Inter site distance | 500m |
| BS Antenna height | 25m |
| UE Antenna height | 1.5 m |
| Spatial consistency | Procedure A in TR38.901 |
| UE trajectory model | Option 1 in 38.843 section 6.3.1 |
| UE trajectory boundary processing model | Option2 (bouncing circle model) |
| Sampling period | 40ms |