3GPP TSG-RAN WG2 Meeting #128	R2-2410803

# Orlando, USA, Nov. 18th – 22nd, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38331 | CR | 5189 | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alternative solution for clarification on the unit of offsetThresholdTA-r18 for ATG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-Core |  |  |  |  |  | Date: |  |  | 2024-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In current specification, the configuration offsetThresholdTA-r18 for ATG TA reporting is in the unit of symbols. However, considering a UE may be configured with multiple BWPs with multiple subcarrier spacings, it’s unclear how to interpret the symbol duration. This issue has been discussed in RAN2#127bis and two options were proposed: - Option 1: The SCS of active BWP is used, the active BWP is the one when UE receives the RRC configuration and the UE does not autonomously change the value upon BWP swtiching. - Option 2: The SCS of initial BWP is used. Based on the discussion RAN2#127bis, either option has supporters and opponents. For Option 1, the concern from companies is that, BWP switching can be triggered by lower layer(e.g. DCI), when network configures/reconfigures the offsetThresholdTA-r18 value, it is possible lower layer also triggers BWP switching at the same time, thus misalignment may occur between the network and the UE. For Option 2, the benefit is that misaligment between network and UE won’t happen, but some companies think it is NBC to existing UE implementation. To address above concerns, we think another solution is to follow the SCS of BWP that indicated by firstActiveDownlinkBWP-Id which received in the same RRC message. Based on current specification, firstActiveDownlinkBWP-Id indicates the BWP that should be activated upon reception of the message. Meanwhile, based on existing IE presence condition, firstActiveDownlinkBWP-Id is mandatory present upon RRCSetup, RRCResume and reconfigurationWithSync, so, it covers all the cases (e.g. initial access, handover, etc). In addition, firstActiveDownlinkBWP-Id can also be included in RRCReconfiguration without reconfigurationWithSync, so, if the network intends to update the value of offsetThresholdTA-R18, the network can simply configure the offsetThresholdTA-r18 together with firstActiveDownlinkBWP-Id in the same RRC message. With this approach, it addresses the NBC concern and it also avoids configuration misalignment eventually. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify in the field description that the unit of offsetThresholdTA-r18 is the symbol using subcarrier spacing of BWP indicated by firstActiveDownlinkBWP-Id received in the same RRC message. This field is included only if firstActiveDownlinkBWP-Id is configured in the same RRC message. Impact analysis Impacted functionality: NR ATG Inter-operability: If the UE is implemented according to this CR but the network is not, or if the network is implemented according to this CR but the UE is not, the network and the UE have different understandings on the value of offsetThresholdTA-r18 that applied by the UE, this may result in unneccessary TAR MAC CE reporting. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It’s unclear how to interpret the unit of offsetThresholdTA-r18 when the UE is configured with multiple BWP with multiple SCS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

### 6.3.2	Radio resource control information elements

Unchanged Text is omitted

#### –	TAR-Config

The IE TAR-Config is used to configure Timing Advance reporting in non-terrestrial networks and ATG network.

TAR-Config information element

-- ASN1START

-- TAG-TAR-CONFIG-START

TAR-Config-r17 ::=                      SEQUENCE {

offsetThresholdTA-r17               ENUMERATED {ms0dot5, ms1, ms2, ms3, ms4, ms5, ms6 ,ms7, ms8, ms9, ms10, ms11, ms12,

ms13, ms14, ms15, spare13, spare12, spare11, spare10, spare9, spare8, spare7,

spare6, spare5, spare4, spare3, spare2, spare1}          OPTIONAL,    -- Need R

timingAdvanceSR-r17                 ENUMERATED {enabled}                                                OPTIONAL,    -- Need R

...

}

TAR-Config-r18 ::=                      SEQUENCE {

offsetThresholdTA-r18               INTEGER (1..56)                                                     OPTIONAL,    -- Need R

timingAdvanceSR-r18                 ENUMERATED {enabled}                                                OPTIONAL,    -- Need R

...

}

-- TAG-TAR-CONFIG-STOP

-- ASN1STOP

TAR-Config field descriptions

offsetThresholdTA Offset for TA reporting as specified in TS 38.321 [3]. Network only configures this parameter for MCG. For ATG, network only configures offsetThresholdTA-r18, which is in unit of symbols using subcarrier spacing of active BWP indicated by firstActiveDownlinkBWP-Id received in the same RRC message. This field is included only if firstActiveDownlinkBWP-Id is configured in the same RRC message.

timingAdvanceSR Used to configure whether a Timing Advance report may trigger a Scheduling Request as specified in TS 38.321 [3].

End of changes