3GPP TSG-RAN WG2 Meeting #128	R2-2410864

Orlando, FL, USA, November 18th – 22nd, 2024

Agenda Item:	8.9.4

Source:	Ericsson

Title:	Support for PWS in NB-IoT NTN

Document for:	Discussion, Decision

# 1	Introduction

The Rel-19 WID on IoT-NTN [1] was updated in RAN#105 with the following additional objective:

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2]

Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI.

Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

RAN2#127bis has made several agreements. In this contribution we focus on some of the open issues.

# 2	PWS support for NB-IoT NTN

### 2.1	Background

Provided the objective of this WID makes an explicit mention to the re-use of existing mechanisms, this section presents an overview of the mechanisms that are employed in LTE to inform UEs about PWS notifications:

- Paging. Ordinary LTE UEs (not in CE) receive an indication of a CMAS/ETWS notification is present in System Information via paging messages. This applies to both RRC_IDLE and RRC_CONNECTED (refer to TS 36.331 – Clause 5.3.2).

- Direct Indication Information. Ordinary LTE UEs which are in RRC_CONNECTED mode and in coverage enhancements (CE) operation may have the capability, since Rel-16, to receive an indication of a CMAS/ETWS notification present in System Information via a Direct Indication Information message scrambled using SI-RNTI (TS 36.331, Table 6.6-2.). In contrast, BL UEs and UEs in CE in RRC_IDLE can receive an indication of a CMAS/ETWS notification present in System Information via Direct Indication Information using P-RNTI but without an associated paging message (TS 36.331, Table 6.6-1).

### 2.2	PWS indication in paging messages

RAN2#127bis agreed to include etws-indication and cmas-Indication in the Direct Indication Information, which is applicable to NB-IoT in RRC_IDLE. However, there were split views on whether to support those indications in paging messages. Even though BL UEs and UEs in CE support the use of paging messages for PWS notifications, we consider that the Direct Indication Information should be sufficient for NB-IoT. Thus, we do not see the additional benefit of duplicating functionality by including PWS notifications in Paging-NB message.

The delivery of PWS notifications via paging messages is not introduced for NB-IoT in Rel-19.

### 2.3	PWS performance in eDRX

RAN2#127bis agreed to capture a disclaimer on PWS performance in eDRX as follows:

Clarify in the spec (and at least in 36.300) that for NB-IoT the ETWS, CMAS, PWS requirement may not be met in some scenarios, e.g. when the UE is in eDRX (can come back to further clarify the specific cases)

On one hand, we think such a disclaimer is reasonable. On the other hand, PWS is a safety-related mechanism, and it should not be best-effort. This could lead to threatening situations where the user is expected a warning message while the UE is sleeping. Therefore, it is desirable to consider potential optimizations to improve PWS performance at least for those NB-IoT NTN UEs which may be equipped with a larger battery and prefer a timely service. In this scenario, a UE may provide an indication to the network of its interest in PWS services. In a similar way to what exists in current NAS protocols, the UE can indicate its desired power saving (e.g., eDRX or PSM) configuration parameters. Based on such assistance information, the network can adjust the assigned power saving mechanism configurations to suit the specific interest of the UE to listen to PWS notifications.

RAN2 discusses methods to improve PWS performance under eDRX.

A UE can indicate its interest in PWS service to the network.

### 2.4	Contents of the PWS SIBs

RAN2#127bis agreed to introduce PWS SIBs for NB-IoT based on the PWS SIBs for LTE.

For NB-IoT NTN PWS, introduce the following new SIBs, taking the content within the corresponding LTE SIBs as a baseline (but also checking whether we can have some optimization/ skip some unnecessary fields):

- SystemInformationBlockType10-NB for primary ETWS notification;

- SystemInformationBlockType11-NB for secondary ETWS notification;

- SystemInformationBlockType12-NB for CMAS notification.

Looking at the ASN.1 structure of the existing SystemInformationBlockType10, SystemInformationBlockType11, SystemInformationBlockType12, most of the fields can be re-used with the exception of the dummy field in SystemInformationBlockType10 that can be removed.

Re-use ASN.1 structure of SystemInformationBlockType10/11/12 for SystemInformationBlockType10/11/12-NB but remove the un-used dummy field.

### 2.4	Support of geofencing in ETWS

As with ETWS in NR NTN, which is being specified within the context of MBS broadcast for NTN in a parallel Rel-19 WI, we see the need to add geofencing solutions for ETWS for NB-IoT NTN. This is simply because the satellite beam footprint can be much larger than the geographical area affected by an earthquake or a tsunami. The detailed solution can be adopted from the solution agreed for ETWS in NR NTN.

Similar to NR NTN, adopt geofencing solutions for ETWS for NB-IoT NTN.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	The delivery of PWS notifications via paging messages is not introduced for NB-IoT in Rel-19.

Proposal 2	RAN2 discusses methods to improve PWS performance under eDRX.

Proposal 3	A UE can indicate its interest in PWS service to the network.

Proposal 4	Re-use ASN.1 structure of SystemInformationBlockType10/11/12 for SystemInformationBlockType10/11/12-NB but remove the un-used dummy field.

Proposal 5	Similar to NR NTN, adopt geofencing solutions for ETWS for NB-IoT NTN.

# 4	References

RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, RAN#105, Sep. 2024.