3GPP TSG-RAN WG2 #128	R2-2410881

Orlando, USA, 18 - 22 November 2024

Agenda Item:	8.8.2

Source:	Ericsson

Title:	DL coverage enhancements

Document for:	Discussion/Decision

# 1	Introduction

The WID was updated in [1], but this objective was not updated:

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4]

- Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth.

- Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements.

- Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint.

  - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any)

  - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements.

- Notes for this objective:

  - SSB channel enhancement other than SSB periodicity extension is not considered

    - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension

  - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation

  - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE

  - NGSO to be considered in priority: LEO Set-1 @ 600 km

  - Rel-18 network energy saving techniques should be considered as baseline in the system level study

RAN2#126 made these agreements:

Agreements;

- Based on the solution being investigated in RAN1, RAN2 will further discuss whether/how legacy UEs might operate in a cell supporting DL coverage enhancements.

- RAN2 assumes that both EFC and EMC are supported

RAN2 has started to study on RAN2 aspects of DL coverage enhancement. To progress the study, RAN2 has identified questions to RAN1 for aspects where the input is required.

Related to this, RAN2 would like to request RAN1 to provide feedback on the following questions.

Question1 : Can RAN1 provide the information on their progress on whether the existing SSB pattern for an NR cell (e.g. SSB position in burst, SSB index number, etc.) is changed in Rel-19 NR NTN, and whether the SSB periodicity is extended compared with existing TN values?

Question2 : Can RAN1 provide the information on whether/how the solution RAN1 is investigating is expected to impact common control signalling for UEs in RRC idle / RRC inactive?

Question3 :  Can RAN1 provide the feedback on whether UL beam hopping is also being studied in RAN1 (and whether this is separate from DL beam hopping)?

Question4 : RAN2 would like to remind RAN1 that satellite beams are currently not visible to UEs and any decision about different beam status (i.e. "off", "common messages only" and "active traffic") will likely have to relate to beams visible to the UE (e.g. SSB beams). RAN2 would also like to know whether RAN1 intends to define beam status for beams not visible to the UE or to define new beam status for beams visible to the UE.

Question5 : Can RAN1 provide the feedback on whether the beam status in different beams (visible to the UE) of one cell are the same or can be different in any given time, i.e.. the beam status is cell specific or beam specific?

And sent an LS to RAN1 [2] with these questions:

And RAN1 has partly replied [3]:

Answer to Q1: According to the updated WID (RP-241667), SSB enhancement other than SSB periodicity extension is not considered. Thereby, RAN1 understanding is that enhancements to the existing SSB patterns, e.g. SSB position in burst, SSB index number, are not within the scope. The extension of the SSB periodicity is still under RAN1’s discussion.

Answer to Q4: Beam footprint status in terms of "off", "common messages only" and "active traffic” in one of the RAN1 agreements is only defined for the sake of system-level evaluation methodology.

At RAN2#127, RAN2 made these agreements:

Agreements:

1.	From RAN2 point of view, if the SSB periodicity is no larger than 160ms, there is no RAN2 impact on SSB configuration (there might still be impacts on DTX aspects)

2.	From RAN2 point of view, If the SSB periodicity is larger than 160ms, for example ssb-PeriodicityServingCell, measurement gap periodicity, SMTC configuration, ssb-Periodicity-r17 for NonCellDefiningSSB-r17 may need to be extended. And the field description of nAndPagingFrameOffset may need to be enhanced to consider the SSB periodicity higher than 160ms.

3.	RAN2 can further consider SMTC impacts due to beam-hopping / larger SSB periodicity

4.	If there is a need to bar pre-Rel19 NTN UEs from accessing a cell operating with DL coverage enhancement (e.g. because of extreme SSB periodicity) the existing NTN bar bit can be used. FFS about the behaviour for Rel-19 UEs not supporting DL coverage enhancement when the existing NTN bar bit is set.

Furter RAN2#127, RAN2 made these agreements:

Agreements:

1.	If it turns out that there is a need to bar UEs not supporting DL-CE, Rel-19 UEs not supporting DL-CE can be barred from accessing a cell operating with DL-CE using the existing NTN bar bit, in the same way as pre-Rel19 NTN UEs (this is an extension of the previous agreement to include also Rel-19 UE not supporting DL-CE)

2.	If it turns out that there is a need to bar UEs not supporting DL-CE, then we need to introduce a barring mechanism to control access of UEs supporting Rel19 NTN DL-CE. FFS on the details. (This also implies that UEs supporting Rel19 NTN DL-CE will not consider the existing NTN barring bit)

3.	(also depending on the details of the RAN1 solution) we can further consider methods to allow UEs not supporting DL CE to down-prioritize or prevent re-selection to cells operating with DL CE.

And the Agenda has the following instructions for this agenda item:

Contributions should focus on RAN2 aspects of DL coverage enhancements (e.g. cell level / beam level DTX/DRX mechanism, etc.).

Here we discuss further details of this WI objective.

# 2	Discussion

This objective shall be studied and specified if beneficial.

DL coverage enhancements shall only be specified if studies show it to be beneficial.

The areas to evaluate are Satellite beam power sharing and feeder link rate limits, including link and system level enhancements, and the R18 network energy saving techniques are the baseline.

This will require RAN1 to define KPIs and potential effects on the coverage. This is evident in RAN1’s answer to Q4.

RAN1#118-bis made some progress and agreed:

Agreement For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is at least 160 ms. - FFS: whether 320ms can be supported as the maximum of the additional default value instead of or in addition to 160ms

In RAN1, it has been agreed to support 160ms as an additional default SSB periodicity, at least. Regarding the impact of this agreement in RAN2 aspects, we do not foresee major changes to the specification since this periodicity value is already supported in the specification.

RAN1 has agreed on a new default SSB periodicity value of 160ms, which is already supported in the specification.

During last RAN2 meeting, it was briefly discussed whether this could have an impact on the configuration of SMTCs and whether enhancements such as UE autonomous location-based SMTC selection should be considered. Assuming all beams have the same SSB periodicity and that not all the neighbours may be active at the same time, a sensible network implementation may use frequency re-use factor and a beam-hopping pattern that minimizes beam interference within the same satellite.

Figure 1. Potential beam hopping pattern where each colour represents a frequency.

In the example provided in Figure 1, it is clear that whenever beam hopping patterns are combined in a sensible manner and all beams have the same periodicity, it would be sufficient for a UE to implement 4 parallel SMTCs to secure efficient operation. This is already possible with the current specification without the need of further enhancements.

For NTN beam-hopping, 4 SMTCs are sufficient to account for 160ms SSB periodicity.

Do not increase the number of SMTCs for 160ms SSB periodicity.

We believe there are no other RAN2 aspects of DL coverage enhancements that are not dependent on the further evaluations in RAN1. This includes the DTX behaviour, barring of UEs, and the RAN1 answers to the unanswered questions in the LS to RAN1.

The discussions will be fast if RAN2 do not need to speculate on what RAN1 agrees to. Therefore, RAN2 needs to wait for the results in RAN1.

RAN2 wait for the RAN1 conclusions on downlink coverage enhancements.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	DL coverage enhancements shall only be specified if studies show it to be beneficial.

Observation 2	RAN1 has agreed on a new default SSB periodicity value of 160ms, which is already supported in the specification.

Observation 3	For NTN beam-hopping, 4 SMTCs are sufficient to account for 160ms SSB periodicity.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Do not increase the number of SMTCs for 160ms SSB periodicity.

Proposal 2	RAN2 wait for the RAN1 conclusions on downlink coverage enhancements.

# References

RP-241789, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3”, 3GPP TSG RAN#105, Sep. 2024.

R2-2405767, “LS on DL coverage enhancements”, Contact: CMCC, To:RAN1, May. 2024.

R1-2407538, “Reply LS on DL coverage enhancements”, Contact: CMCC, To: RAN2, Aug. 2024.