3GPP TSG-RAN2 Meeting #128	R2-2410937

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 2016 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Co-configuration of IUC scheme-2 and random selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2024-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to R1 LS reply in R1-2409174, R2 made decision regarding co-configuration of IUC scheme-2 and random selection. R2-2409551 Discussion on R1 LS Reply (R1-2409174) OPPO discussion Rel-17 NR_SL_enh-Core Proposal 1: R2 decide either to allow IUC scheme-2 and random-selection co-configuration or not, and adopt CR correspondingly (TP1 if allow, TP2 if disallow). => TP2 is agreed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the case of co-configuration of IUC scheme-2 and random selection. Impact analysis Impacted 5G architecture options: NR SA Impacted functionality: R17 SL IUC Operation Inter-operability: If the network is implemented according to the CR and the UE is not, there are no inter-operability issues, since network would not configure IUC scheme-2 and random selection in the same resource pool. If the UE is implemented according to the CR and the network is not, there are no inter-operability issues, since anyway the random selection operation in case of IUC scheme-2 was not specified in the current spec. If one UE is implemented according to the CR and the other UE is not, there are no inter-operability issues, since the concerned operation is limited to Tx-side UE internal operation only. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS 38.321 includes a case for which R2 concluded not to support. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.22.1.2b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change Start

5.22.1.2b	Re-selection for using a received resource conflict indication

If the MAC entity has been configured with Sidelink resource allocation mode 2 to transmit using pool(s) of resources in a carrier as indicated in TS 38.331 [5] based on full sensing, or partial sensing or any combination(s), the MAC entity shall for each Sidelink process:

1>	if sl-interUECoordinationScheme2 enabling reception/transmission of a resource conflict indication is configured by RRC; and

1>	if the next resource of the selected sidelink grant which has been indicated by a prior SCI is overlapped with conflict resource(s) indicated by the physical layer as specified in clause 16.3.1 of TS 38.213 [6]:

2>	remove the resource from the selected sidelink grant associated to the Sidelink process;

2>	randomly select the time and frequency resource from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7] excluding the conflict resource(s) for the removed resource, according to the amount of selected frequency resources, the selected number of HARQ retransmissions and the remaining PDB of either SL data available in the logical channel(s) by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources, and that a resource can be indicated by the time resource assignment of an SCI for a retransmission according to clause 8.3.1.1 of TS 38.212 [9];

NOTE 1:	If retransmission resource cannot be selected by ensuring that the resource can be indicated by the time resource assignment of a prior SCI, how to select the time and frequency resource for more transmission opportunities from the available resources is left for UE implementation by ensuring the minimum time gap between any two selected ‎resources in case that PSFCH is configured for this pool of ‎resources.

2>	replace the removed resource by the selected resource for the selected sidelink grant.

NOTE 2:	It is left for UE implementation to reselect any pre-selected but not reserved resource(s) other than the resource overlapping with the conflict resource(s) indicated by the physical layer during reselection triggered by the conflict resource(s) indicated by the physical layer.

NOTE 3:	It is up to UE implementation whether and how to set the resource reservation interval in the re-selected resource to replace the resource overlapping with the conflict resource(s) indicated by the physical layer.

Change Stop