3GPP TSG RAN WG2#128	R2-2409515

Orlando, USA, 18th - 22nd November 2024

3GPP TSG RAN WG1 #118bis	R1-2409301

Hefei, China, October 14th – 18th, 2024

Title:	LS on waveform determination for PUSCH scheduled by fallbackRAR UL grant and MsgA PUSCH

Response to:

Release:	Rel-17

Work Item:	NR_redcap-Core, NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_slice-Core

Source:	RAN1

To:	RAN2

Cc:

Contact person:	Yanping Xing

xingyanping@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1		Overall description

In RAN1#118 and RAN1#118bis, RAN1 discussed UE procedure for applying transform precoding on PUSCH with introduction of additionalRACH-ConfigList in R1-2406324 and R1-2408006.

For a PUSCH scheduled by fallbackRAR UL grant, the UE shall consider the transform precoding either 'enabled' or 'disabled' according to the higher layer configured parameter msg3-transformPrecoder according to TS38.214.

For a PUSCH scheduled by RAR UL grant, or for a PUSCH scheduled by fallbackRAR UL grant, or for a PUSCH scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, the UE shall consider the transform precoding either 'enabled' or 'disabled' according to the higher layer configured parameter msg3-transformPrecoder.

For this case, the following two alternatives were discussed on which msg3-transformPrecoder should be used without conclusion in RAN1.

  - Alt 1: It is always msg3-transformPrecoder in rach-ConfigCommon included directly within BWP configuration.

  - Alt 2:

    - For msgA-ConfigCommon included directly within BWP configuration, it is msg3-transformPrecoder in rach-ConfigCommon included directly within BWP configuration.

    - For msgA-ConfigCommon included in additionalRACH-ConfigList, it is

msg3-transformPrecoder included in the same AdditionalRACH-Config.

For MsgA PUSCH if msgA-TransformPrecoder is not configured, the UE shall consider the transform precoding either 'enabled' or 'disabled' according to the higher layer configured parameter msg3-transformPrecoder according to TS38.214.

For a MsgA PUSCH, the UE shall consider the transform precoding either 'enabled' or 'disabled' according to the higher layer configured parameter msgA-TransformPrecoder. If higher layer parameter msgA-TransformPrecoder is not configured, the UE shall consider the transform precoding either 'enabled' or 'disabled' according to the higher layer configured parameter msg3-transformPrecoder.

For this case, the following alternatives were discussed on which msg3-transformPrecoder should be used without conclusion in RAN1.

  - Alt 1: It is always msg3-transformPrecoder in rach-ConfigCommon included directly within BWP configuration.

  - Alt 2:

    - For msgA-ConfigCommon included directly within BWP configuration, it is msg3-transformPrecoder in rach-ConfigCommon included directly within BWP configuration.

    - For msgA-ConfigCommon included in additionalRACH-ConfigList, it is msg3-transformPrecoder included in the same AdditionalRACH-Config.

# 2		Actions

To RAN2

ACTION: 	RAN1 respectfully asks RAN2 to answer the following questions.

Q1: From RAN2 point of view, which msg3-transformPrecoder is used to determine waveform for PUSCH scheduled by fallbackRAR UL grant and for MsgA PUSCH when msgA-TransformPrecoder is not configured respectively?

Q2: Is the related UE behavior already defined in RAN2 spec?

# 3	Dates of next TSG RAN WG1 meetings

RAN1#119	2024-11-18 - 2024-11-22	Orlando, US

RAN1#120	2025-02-17 - 2025-02-21	Athens, Greece