3GPP TSG-RAN WG2 Meeting #129                                                                   R2-2500087

Athens, Greece, February 17th – 21st, 2025

Agenda item:	8.19

Source:	vivo

Title:	Discussion on RAN4 LS on Ku Band Numerology

Document for:	Discussion and Decision

- Introduction

In the LS [1], we know that RAN 4 has decided to specify two sets of bands for the exact same frequency range, one for FR1 numerology and the other for FR2 numerology. RAN4 sent that LS to ask if RAN2 sees any reason why two sets of Ku bands, one using FR1 numerology and the other using FR2 numerology cannot be defined.

Further, in the RAN#106 meeting, a technical discussion was launched to decide whether only one band should be defied or two sets of bands should be defined for the Ku band [2]. Finally, with the majority of support, it was agreed that define two sets of bands. The corresponding objective is quoted below [3],

- Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing.

In this contribution, we would like to provide our understanding of the two sets of bands from the RAN2 point of view.

- Discussion

After defining two sets of bands with the exact same frequency range but independent band number (i.e., one for FR1-NTN with 30 kHz subcarrier spacing, while the other for FR2-NTN with 120 kHz subcarrier spacing), the main RAN2 impact is the SCS configuration and UE supported bandwidth reporting.

For the SCS configuration, currently, the candidate value is dependent on the operating band, e.g., 15 or 30 kHz for FR1-NTN bands, as per the RRC specification,

ssbSubcarrierSpacing Subcarrier spacing of SSB. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1/FR2-NTN: 120 or 240 kHz FR2-2: 120, 480, or 960 kHz subcarrierSpacing The SubcarrierSpacing for this resource pattern. If the field is absent, the UE applies the SCS of the associated BWP. The value kHz15 corresponds to µ=0, the value kHz30 corresponds to µ=1, and so on. Only the following values are applicable depending on the used frequency (see TS 38.214 [19], clause 5.1.4.1): FR1: 15, 30 or 60 kHz FR2-1/FR2-NTN: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz

With two sets of bands, from SCS configuration point of view, the first set of band (Ku band X) can be regarded as a FR1-NTN band and another set of band (Ku band Y) can be regarded as a FR2-NTN band. With this assumption, the current RRC spec can smoothly support the SCS configuration for Ku band without any spec impact.

For the UE supported bandwidth reporting, as the candidate support bandwidth range is a subset of that of the existing FR1-NTN band and FR2-NTN band, RAN2 can reuse the existing capability field for UE capability reporting. We fail to figure out any functional spec impacts.

Based on the above, we think RAN2 has no technical reason to defy two sets of Ku bands. Meanwhile, to facilitate the SCS configuration for Ku band, RAN2 assumes the first set of band is regarded as a FR1-NTN band and the other set of band is regarded as a FR2-NTN band.

Proposal 1: RAN2 confirms that it is possible to define two sets of Ku bands, one using FR1 numerology and the other using FR2 numerology from RAN2 point of view.

Proposal 2: From RRC configuration point of view, the first set of Ku band is regarded as a FR1-NTN band and another set of Ku band is regarded as a FR2-NTN band upon SCS configuration.

3 Conclusion

In this contribution, we have provided our understanding of the two sets of bands from RAN2 point of view. And we make the following proposals,

Proposal 1: RAN2 confirms that it is possible to define two sets of Ku bands, one using FR1 numerology and the other using FR2 numerology from RAN2 point of view.

Proposal 2: From RRC configuration point of view, the first set of Ku band is regarded as a FR1-NTN band and another set of Ku band is regarded as a FR2-NTN band upon SCS configuration.

4 Reference

[1] R4-2419902, LS on Ku band numerology, RAN4.

[2] draft_MeetingReport_RAN_106_241212_eom.

[3] RP-243314, Revised WID: Introduction of Ku bands for NR NTN.