3GPP TSG-RAN WG2 Meeting #129	R2-2500094

Athens, Greece, February 17-21, 2025

Agenda item:	8.7.5

Source:	Qualcomm Incorporated

Title:	Discussion on RLC enhancements

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion and decision

# Introduction

In this paper, we discuss enhancements for timely RLC retransmissions and avoiding unnecessary retransmissions.

# Discussion

## Timely retransmission

At RAN2#128, the following agreement was made:

• Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement and NW can configure either or both of them.

This agreement implies that a way-forward for timely retransmission should unify autonomous retransmission scheme and polling enhancements.

Before discussing possible options for this unified solution, we think that there should be conditions for triggering/applying this solution. There have been many proposals on that, but we think two of those make the most sense, namely,

- When SDUs have become delay critical; or

- When the transmitter can determine with high confidence that transmission of an SDU has failed.

It can be left for further study whether additional criteria should be included in the triggering conditions.

Proposal 1. 	Triggers for timely retransmission may include at least one of the following criteria:

- When SDUs have become delay critical;

- When the transmitter can determine with high confidence that transmission of an SDU has failed.

FFS any additional criteria should be included.

Proposal 2.	Network configures which criteria are included as triggers for timely retransmission.

Another aspect of this unified solution is whether to trigger a poll or perform autonomous retransmission. In our view, it is better to separate this design from the triggering conditions. Firstly, it makes technical sense for them to share a common set of triggering conditions, because both are options for a transmitter to react to SDUs becoming delay critical. They only differ in how aggressive network wants the transmitter to react. Secondly, they have been contentious topics and challenging for companies to converge on. It helps facilitate more progress (hopefully) if their designs are discussed separately.

We see two possible options for this configuration:

- Option A. Network configures a one-bit indicator, which indicates whether UE (as a transmitter) should trigger a poll or perform autonomous retransmission immediately after the triggering conditions are met.

- Option B. Adopt a scheme similar to the legacy poll retransmission timer, i.e. after the triggering conditions for timely retransmission are met, a poll is triggered, and a timer is started. If no status report for the SDU is received before the timer expires, the UE autonomously triggers retransmission of the SDU.

We think Option A is simple to understand and easy to implement for both network and UE. Option B in essence is a delay-aware enhancement to the legacy polling procedure. It may give network a bit more flexibility in controlling how aggressively UE may perform autonomous retransmission (e.g. by configuring different durations for the timer). However, in some cases, this extra timer may not be necessary. For example, if the transmitter can determine with high confidence that transmission of an SDU has failed, there is little benefit for the transmitter to wait extra time before performing the transmission. Or if network has configured a small remaining time threshold (especially if it is shorter than the RLC round trip time), then it makes more sense for UE to autonomously start the retransmission, instead of waiting extra time.

Therefore, we think Option A is preferable over Option B.

Proposal 3.	Network configures an indicator, which indicates whether UE triggers a poll or autonomous retransmission after the triggering criteria are met.

If Option A is not agreeable, we then think Option B can be a baseline for further discussion. In that case, we think a new timer instead of the legacy t-PollRetransmit timer should be used. For a delay-critical SDU, it may need to use a shorter timer, because its short remaining time requires an earlier retransmission. On the other hand, if t-PollRetransmit timer is configured based on the need of delay-critical SDUs, it may result in more retransmissions being performed than necessary.

To unify polling enhancement and autonomous retransmission in a single solution, another necessary difference with the legacy t-PollRetransmit timer is that this new timer can be configured with zero duration. In that case, UE performs autonomous retransmission once the triggering criteria are met.

Proposal 4.	If Proposal 3 is not agreeable, the legacy procedure for t-PollRetransmit timer can be considered as a baseline for further enhancements. In that case, a new timer shall be used instead, and its duration can be zero.

## Avoid unnecessary retransmissions

At RAN2#127bis meeting, the following agreements were made on the options for avoiding unnecessary retransmissions:

- RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e. - TX side stops transmissions of an outdated SDU - RX side abandons the SDU based on a local timer - Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified

An issue that remains open is how a RLC transmitter should “stops transmission of an outdated SDU”. There can be different options for determining outdated SDUs and stopping their transmissions. For example, expiry of PDCP discard timer can be used to trigger the termination. Or network can configure the legacy maxReTxThreshold based on the PDB of SDUs. Either option requires some changes to the legacy procedure:

- The first option requires removal of SDUs from RLC retransmission buffer upon the indication from PDCP. This is a new procedure and can have impact on some UE implementation (e.g. those implement pre-build of RLC PDUs). Since RLC is directly in the data path and operates on a very short timeline, such a removal may require significant changes to the data path and hence are not desired.

- In the second option, termination of RLC retransmission upon reaching maxReTxThreshold is a legacy procedure and hence has no impact on the data path. However, new behavior needs to be defined to avoid unnecessary RLF when number of retransmissions reaches maxReTxThreshold due to depletion of delay budget instead of broken link. For example, the new trigger can be that RLF is triggered after N or more SDUs have reached maxReTxThreshold in the last T msec. The downside of this option is that the transmitter may not be able to immediately stop retransmission of an SDU once it becomes outdated. But its advantage is that it does not have any impact on the legacy user-plane procedures at RLC transmitter.

Observation 1.	Terminating outdated RLC SDUs based on indication from PDCP may have significant impact on UE implementation of RLC user-plane procedures.

Comparing the required changes, we can see that the second option has less impact on the legacy user-plane procedures at RLC transmitter. Although it may not allow a transmitter to immediately stop retransmission of an SDU once it becomes outdated, we do not believe that delay is a critical performance metric and hence can yield to the benefit of having minimal impact on the legacy procedures. Therefore, the second option is more preferred.

Proposal 5.	RLC transmitter stops retransmission of an outdated SDU after maxReTxThreshold is reached. Network can configure a new RLF trigger for such a DRB, e.g. RLF is triggered after N such terminations in the last M PDUs.

# Conclusion

Based on the above analysis, we respectively request RAN2 to discuss and agree to the following proposals:

Timely retransmission

Proposal 1. 	Triggers for timely retransmission may include at least one of the following criteria:

- When SDUs have become delay critical;

- When the transmitter can determine with high confidence that transmission of an SDU has failed.

FFS any additional criteria should be included.

Proposal 2.	Network configures which criteria are included as triggers for timely retransmission.

Proposal 3.	Network configures an indicator, which indicates whether UE triggers a poll or autonomous retransmission after the triggering criteria are met.

Proposal 4.	If Proposal 3 is not agreeable, the legacy procedure for t-PollRetransmit timer can be considered as a baseline for further enhancements. In that case, a new timer shall be used instead of the legacy t-PollRetransmit, and its duration can be zero.

Avoid unnecessary retransmissions

Observation 1.	Terminating outdated RLC SDUs based on indication from PDCP may have significant impact on UE implementation of RLC user-plane procedures.

Proposal 5.	RLC transmitter stops retransmission of an outdated SDU after maxReTxThreshold is reached. Network can configure a new RLF trigger for such a DRB, e.g. RLF is triggered after N such terminations in the last M PDUs.