3GPP TSG-RAN WG2 Meeting #129 	R2-2500553

Athens, Greece, 17-21 Feb. 2025

Agenda item:	8.5.4

Source:	Nokia, Nokia Shanghai Bell

Title:	On Adaptation of common signal/channel transmissions for NES

WID/SID:	Netw_Energy_NR_enh

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we discuss aspects related to the potential adaptations of the common signal/channel transmissions.

# 2	Adaptation of SSB in time domain

During the previous meetings, there were no RAN2 agreements on this topic. As per the current agreements, based on the understanding of the RAN1 agreements, Cases B2’ and B3’ of the RAN1#118 meeting agreement, have been accepted, there was no consensus to support Cases A2 and A3 (i.e., SSB adaptation in PCell) and only B1 remains open for further discussion SSB adaptation in the time domain.

Agreement​ – [RAN1#118] For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’)

A scenario where adaptation of the SSB burst periodicity can be applied is the activation of an SCell.

For example, a RAN1#119 agreement of Objective 1 (On-demand SSB, OD-SSB) is:

“The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB”.

A potential alternative to the agreement, namely Alt.2, is also provided, referring to the case where always-on SSB is not CD-SSB:

“Alt 2: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB.”

This agreement and alternative can realize a scenario of faster SCell detection and activation by adapting the periodicity of the always-on SSBs. In detail, in both cases, the UE would be able to detect the always-ON SSB and OD-SSB in the same frequency location and understand via RRC configuration that they reside on the same frequency. Thus, when always-ON SSB and OD-SSB are in the same frequency, we can consider it as adaptation of always-ON SSBs in time domain.

Observation 1: When always-ON SSB and OD-SSB are in the same frequency, we can consider it as adaptation of always-ON SSBs in time domain.

The configuration for performing such adaptation of always-ON SSBs can be provided based on parameters dedicated to SSB adaptation configured via Serving Cell Configuration, e.g., new periodicity value.

The overall configuration of SSB adaptation, at least for adapting always-ON SSB during SCell activation, can be of similar structure as in our accompanying paper on Objective 1. Thus, from RAN2 perspective, the two features could be rather same in the end.

In terms of signaling, the MAC-CE implementing OD-SSB could be re-used to enable the SSB adaptation in time domain, at least for the cases of B2 and B3 of the RAN1#118 agreement. For example, in Objective 1, periodic CSI reporting based on OD-SSB has been agreed. This reporting takes place after SCell activation and may continue as long as OD-SSB is transmitted. Extending this agreement to always-ON SSBs, different SSB burst periodicity can be indicated dynamically based on RRC signaling or MAC-CE or dynamic control information. Thus, it seems reasonable to consider simultaneously at least RRC signaling and MAC-CE aspects to for these two features, i.e., OD-SSB and SSB adaptation in time domain (always-ON), simultaneously.

In our view, the described formulation for enabling SSB adaptation in the time domain is quite similar to the Objective 1 that focuses on “On-demand SSB SCell operation”. For this reason, we believe that at least RRC signaling and MAC-CE aspects to indicate SSB adaptation in time domain can be treated in conjunction with the respective designs discussed/agreed in Objective 1.

Proposal 1: When designing on-demand SSB and SSB adaptation activation, MAC CE/RRC signaling should be done jointly in RAN2.

During the RAN4 #112bis meeting, an LS was sent from RAN4 to RAN2 asking about the SMTC impact due to SSB adaptation, as observed below.

RAN4#112bis extensively discussed impact on RRM requirements from SSB adaptation. The following agreements were reached after discussion: Agreements: - RAN4 to discuss whether there are any impacts on at least the following RRM requirements due to SSB adaption: - L1-RSRP for serving cell - L1-SINR - BFD - CBD - SCell activation - TCI state switching - NR intra-frequency L3 measurement - NR inter-frequency L3 measurement However, L3 measurement requirement in RAN4 is defined based on SMTC configuration rather than SSB configuration in legacy. RAN4 would like to ask RAN2 that whether there is any SMTC impact due to SSB adaptation. 2. Actions: To RAN2. ACTION: RAN4 kindly asks RAN2 to answer the SMTC impact question due to SSB adaptation above.

Dynamic adaptation of this SSB transmission may have an impact on L3 measurements performed by the UE, e.g., with the UE performing excess measurements when there is no SSB burst transmission, as in Figure 1 or e.g., with the UE missing transmission occasions of SSB bursts, as in Figure 2.

Figure 1: SSB burst periodicity is adapted to a longer value and UE attempts unnecessary measurements.

Figure 2: SSB burst periodicity is adapted to a shorter value and UE misses useful measurement occasions.

A way to address these issues would be to align, the SSB adaptation in time domain, e.g., burst periodicity, with a dedicated SMTC to be applied for measurements of adapted SSBs.

Such adaptation can be performed based on parameters dedicated for SSB adaptation configured via Serving Cell Configuration, in addition to, e.g., a new periodicity value (potentially other parameters too).

This adaptation can be indicated dynamically together with the SSB adaptation based on RRC signaling or MAC-CE or dynamic control information.

For adaptation of non-cell defining SSBs in an SCell, we believe that the configuration of an SMTC can be aligned with Objective 1 that focuses on “On-demand SSB SCell operation.

Proposal 2: When designing on-demand SSB and SSB adaptation activation, SMTC configuration for the two features should be done jointly in RAN2.

# 3		PRACH Adaptation in time domain

As per the latest RAN1#117 meeting outcome, it was agreed to support at least the PRACH adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any). It was also noted that NES-capable UEs can use both additional PRACH resources and PRACH resources configured for legacy UEs.

In our view, the agreed PRACH adaptation based on additional PRACH resources should be applied for both 4-step RACH and 2-step RACH (if configured in the cell). Particularly for 2-step RACH, further clarifications and considerations would be needed. For instance, it is not clear whether additional PUSCH resources for NES-capable shall be considered for MsgA payload transmission or the same PUSCH as configured for legacy UEs shall be considered.

Proposal 3: Clarify whether further considerations are needed for the 2-step PRACH adaptation e.g., the resource used by the UE to transmit MsgA.

There are two types of PRACH that can be configured:

- PRACH associated with CBRA configured by cell-specific signaling.

- PRACH associated with CFRA configured by UE-dedicated signaling.

The agreed PRACH adaptation shall at least target CBRA procedures. PRACH adaptation associated with CFRA procedures shall be studied as well, if time allows.

Proposal 4: PRACH adaptation solution(s) are at least applied to CBRA. PRACH adaptation applied to CFRA shall also be investigated, if time allows, however no CFRA-specific PRACH adaptation solutions should be considered.

The likelihood that cells are designed to only serve R19 UEs is low; thus, the PRACH adaptation should be applicable to cells that support both legacy and Rel-19 (NES-capable) UEs. Moreover, in order to avoid any impact on legacy UEs as noted in the WID, and to enable the initial access for all UEs before being able to identify whether they are legacy or NES-capable UEs, the cell should support both legacy and NES-capable UEs.

Proposal 5: PRACH adaptation should be applicable to cells that support both legacy and Rel-19 UEs.

The configuration of a PRACH configuration index for the additional PRACH resources different from the PRACH configuration index for the legacy resource offers more flexibility to the NW and can help accommodating scheduler needs. However, the possible overlaps between sub-sets of ROs would need to be further studied in this case, which is under discussion in RAN1.

The configuration of the PRACH configuration index for the additional PRACH resources as the legacy resource allows the network energy consumption for PRACH reception to occur mainly (or only) during system frames where legacy ROs are located and, at the same time, ensures that PRACH resources are not overlapping in time and/or frequency.

Additional mechanisms are also being discussed in RAN1 for determining the additional PRACH resources in time domain including time offset, RO muting/masking but we should wait for RAN1 progress on this before discussing RAN2 implications.

Proposal 6: RAN2 to wait for RAN1 to conclude on PRACH adaptation mechanisms before discussing RAN2 implications.

One potential implication on RAN2 could be on the Conditional handover (CHO) procedure as the NW does not know the time instant when UE initiates CHO and the UE can’t be aware in advance about the potential activation/deactivation of additional PRACH resources at one or more potential target cell(s). So RAN2 should consider whether it is feasible to support additional PRACH resources for CHO and if they want to be supported then how it can be achieved.

Proposal 7: Study whether further enhancements are needed for the conditional handover procedure when the additional PRACH resources are activated at one or more candidate target cells.

Another implication on RAN2 will be the beam failure recovery configuration as for beam failure recovery RAN2 needs to discuss further how additional PRACH resources can be used for beam failure recovery.

Proposal 8: RAN2 to consider how to enable usage of additional PRACH resources for beam failure recovery procedure.

From our perspective, it is natural to expect that additional PRACH resources are configured semi-statically in Rel-19. Whether adaptation of the resources will be performed only via DCI format 1_0, as per RAN1#118bis agreements, or also via semi-static signalling will need to be further discussed. Indeed, system Information (SI), e.g. SIB1, could also be considered for indicating PRACH adaptation to UEs that have missed the DCI-based indication of additional PRACH resources for NES-capable UEs. The SIB1 would then include the information about the configuration of additional PRACH resources and the activation status. Such procedure would not require any SI change.

Proposal 9: Higher-layer signalling based on SI, e.g. SIB1, for PRACH adaptation could be considered, in addition to the DCI-based adaptation.

Proposal 10: RAN2 needs to wait for more RAN1 inputs before we can progress detailed signaling of PRACH adaptation

## PRACH Adaptation in time domain in conjunction with adaptation of paging occasions

A key open aspect is how to handle the potential PRACH load increase, especially for Ns=8 or larger, when the POs are clustered in time as agreed by RAN2.

The PRACH load level will inherently increase if the paging occasions are confined in time domain. This is because the UEs will have to be paged during such confined time period, thereby causing the UEs to respond to the paging also in a confined time interval. So it should be possible to temporarily increase the amount of PRACH resources in order to accommodate more clustered paging responses from UEs. The legacy approach of configuring a high amount/frequently occurring PRACH resources will impact the network’s resource availability and RO monitoring energy consumption. Thus, one approach which can be considered by RAN2 is to temporarily make additional PRACH resources available, when one or more UEs are paged. Such additional PRACH resources would not be available, when no UEs are paged, and thus the PRACH resource can be repurposed and the RO monitoring omitted.

Proposal 11: PRACH adaptation in time domain is supported to temporarily increase the PRACH capacity to be aligned with clustered paging and spread the PRACH load when paging occasion adaptation is used.

# Adaptation of paging occasions

In RAN2 #127bis the following was agreed:

Agreements on paging adaptation - Select option-b as baseline for R19 NES paging enhancement. - R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values) - Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration. - Legacy UE is not barred. - Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration.

In RAN2 #128 it was further agreed:

Agreements on paging adaptation - Introduce value for Ns=8. FFS on 16. - Introduce value for N= T/32. FFS on T/64, T/128, T/256.

As previously discussed by RAN2 the PF & PO calculations in TS 38.304 also apply to the new Ns and N values. However, the new Ns and N values will also impact the paging configuration in TS 38.331 (in PCCH-Config). The firstPDCCH-MonitoringOccasionOfPO information element may be provided per PO to define the first PDCCH monitoring occasion. With Ns = 8 the number of firstPDCCH-MonitoringOccasionOfPO information elements will double compared to Ns = 4. The number of bits used per information element depends on the subcarrier space used in the corresponding bandwidth part and the N. For example, the combination of 15 kHz SCS and T/1 results in a PDCCH monitoring range of 0-139 i.e. 8 bits are required. For the same SCS and T/16 the range is 0-2239, which will require 12 bits. Since N is increased to T/32 an additional bit is required. In summary, the doubling of Ns and halving of N may require more than double the number of bits to transmit the firstPDCCH-MonitoringOccasionOfPO. Such a significant increase in signaling overhead needs to be addressed by RAN2 as already agreed in RAN2 #127bis.

Observation 2: The use of Ns=8 and N=T/32 will result in more than double number of bits required for firstPDCCH-MonitoringOccasionOfPO as compared to the legacy signaling.

The signaling of the firstPDCCH-MonitoringOccasionOfPO concerning the first 4 POs (i.e. the maximum for legacy UEs) cannot be optimized due to the requirement of backwards compatibility, but we think it is beneficial for RAN2 to further discuss how to efficiently configure the firstPDCCH-MonitoringOccasionOfPO for the additional for POs of a PF.

In our view, the current firstPDCCH-MonitoringOccasionOfPO provides a very high degree of flexibility to the network scheduler because the PDCCH MO can be indicated for any symbol in the paging frame. One approach to minimize the signaling overhead can therefore be to transmit the additional firstPDCCH-MonitoringOccasionOfPO with a reduced granularity. Another implication of the current ability to indicate any symbol in the paging frame is that a second PO’s PDDCH MO may be defined to occur before a first PO’s PDCCH MO. To reduce the signaling RAN2 could instead discuss to signal the second PO’s PDCCH MO to be relative to the first PO’s PDCCH MO.

Proposal 12: RAN2 to discuss how to efficiently transmit firstPDCCH-MonitoringOccasionOfPO for the 4 additional POs of a PF.

In release 17, 3GPP specified the Paging Early Indication (PEI). The PEI is provided via DCI format 2_7 and consists of a paging indication field, which is the product of the number of POs indicated by the PEI and the number of subgroups per PO, and a TRS availability indication bitmap. Currently, the number of POs per PEI is in the range [1, 2, 4, 8] and since the legacy number of POs per PF is 4, the configuration of 8 POs per PEI implies two PFs are addressed by the PEI. This seems unlikely in the low paging load scenario currently targeted for paging enhancements by RAN2, because the PF interval would be long, which would then result in a very long time-distance between the PEI and the second PF. Therefore, we expect the number of POs per PEI to be configured as maximum 4.

The current TS 38.213 paging occasion index function iPO=UE_IDmodN⋅NS+i_smodNPOPEI is applicable to this scenario as well, because the additional POs (if Ns = 8 for release 19 UEs) will map into the same bits as used by the legacy UEs. The issue is that it will increase the false alarm rate, when both legacy and release 19 UEs monitor the same bit in the PEI. Furthermore, if the number of POs per PEI is 4 there may be spare bits in the DCI format 2_7, because it has a maximum size of 43 bits including up to 6 bits for the TRS availability indication. Assuming 4 subgroups per PO, up to 4*4+6=22 bits will be used.

Observation 3: There may be more than 20 spare bits in the PEI DCI format 2_7, when the number of POs per PEI is 4 (for legacy UEs).

The exact bit configuration is up to RAN1 (being part of TS 38.213), but first RAN2 needs to discuss whether the legacy and release 19 UEs can be addressed by the same PEI. In our view, this is beneficial due to the availability of the spare bits and the potential network energy saving of sending a single PEI instead of two PEIs (one for legacy UEs, one for release 19 UEs).

Proposal 13: RAN2 to discuss the use of a single PEI to address both legacy and release 19 UEs.

# Conclusion

Observation 1: When always-ON SSB and OD-SSB are in the same frequency, we can consider it as adaptation of always-ON SSBs in time domain.

Proposal 1: When designing on-demand SSB and SSB adaptation activation, MAC CE/RRC signaling should be done jointly in RAN2.

Proposal 2: When designing on-demand SSB and SSB adaptation activation, SMTC configuration for the two features should be done jointly in RAN2.

Proposal 3: Clarify whether further considerations are needed for the 2-step PRACH adaptation e.g., the resource used by the UE to transmit MsgA.

Proposal 4: PRACH adaptation solution(s) are at least applied to CBRA. PRACH adaptation applied to CFRA shall also be investigated, if time allows, however no CFRA-specific PRACH adaptation solutions should be considered.

Proposal 5: PRACH adaptation should be applicable to cells that support both legacy and Rel-19 UEs.

Proposal 6: RAN2 to wait for RAN1 to conclude on PRACH adaptation mechanisms before discussing RAN2 implications.

Proposal 7: Study whether further enhancements are needed for the conditional handover procedure when the additional PRACH resources are activated at one or more candidate target cells.

Proposal 8: RAN2 to consider how to enable usage of additional PRACH resources for beam failure recovery procedure.

Proposal 9: Higher-layer signalling based on SI, e.g. SIB1, for PRACH adaptation could be considered, in addition to the DCI-based adaptation.

Proposal 10: RAN2 needs to wait for more RAN1 inputs before we can progress detailed signaling of PRACH adaptation

Proposal 11: PRACH adaptation in time domain is supported to temporarily increase the PRACH capacity to be aligned with clustered paging and spread the PRACH load when paging occasion adaptation is used.

Observation 2: The use of Ns=8 and N=T/32 will result in more than double number of bits required for firstPDCCH-MonitoringOccasionOfPO as compared to the legacy signaling.

Proposal 12: RAN2 to discuss how to efficiently transmit firstPDCCH-MonitoringOccasionOfPO for the 4 additional POs of a PF.

Observation 3: There may be more than 20 spare bits in the PEI DCI format 2_7, when the number of POs per PEI is 4 (for legacy UEs).

Proposal 13: RAN2 to discuss the use of a single PEI to address both legacy and release 19 UEs.