3GPP TSG-RAN WG2 #129	R2-2501002

Athens, Greece, 17th – 21st February 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on the LP-WUS handling for Emergency call back

Document for:	Discussion and decision

Agenda Item:	8.4.2

- Introduction

RAN2#128 agreed made the following agreement on subgrouping [

FFS whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session.

In this contribution, we would like to discuss the handling of emergency call back after the emergency PDU session has been terminated when the UE apply LP-WUS for idle mode/RRC inactive mode

- Discussion

It is assumed that there are no restrictions on initiating an emergency call while the UE is in the LP-WUS state. In some cases, an emergency call back call may be initiated. Normally, this terminating call or paging is treated as a regular terminating call.

Note: For the CN assigned subgrouping it is out of scope of RAN2 Here we would like to discuss the case of UE_ID based subgrouping. However, if the gNB handles it in this manner, it may result in the fail to establish the emergency call back call or cause delays.

Observation 1.	From the operator's perspective, any delays related to emergency calls caused by the addition of new features must be prevented.

Proposal 1.	RAN2 confirms the delays caused by LP-WUS feature is the issue to be solved.

Several solutions can be considered to address this issue.

One possible approach is up to implementation on the base station side, e.g. where the network controls the UE to prevent it from transitioning to the Idle state for a specific period after an emergency call. However, considering the importance of emergency call back communication, leaving this process to implementation may not satisfy the legal requirement.

Proposal 2.	Any standardized solution is needed for the case of emergency call back e.g. LP-WUS should not be applied at the UE side

- Summary and proposal

Observation 1.	From the operator's perspective, any delays related to emergency calls caused by the addition of new features must be prevented.

Proposal 1.	RAN2 confirms the delay of  caused by LP-WUS feature is the issue to be solved.

Proposal 2.	Any standardized solution is needed for the case of emergency call back e.g. LP-WUS should not be applied at the UE side

- References

[1]	R2-2410841 Discussion on LP-WUS for IDLE_INACTIVE state, NTT DOCOMO, INC.

[2]	Draft_RAN2_128_Meeting_Report_v1

- Annex: LS to SA2

3GPP TSG-RAN WG2 #129	R2-25xxxxx

Athens, GR, 17 – 21 Feb. 2025

Title:	[Draft] LS on LP-WUS requirements for emergency call back

Response to:	-

Release:	Rel-19

Work Item:	NR_LPWUS-Core

Source:	NTT DOCOMO (to be RAN2)

To:	SA2

Cc:	RAN3, CT1

Contact Person: 	Kenichiro Aoyagi

kenichirou.aoyagi.yv@nttdoomo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- Overall Description:

Currently, RAN2 is discussing whether the delay caused by LP-WUS is a problem in emergency call back. This point was also discussed in the discussion of Paging Early Indication in Release 17, but no explicit solution was introduced. On the other hand, since the wake up delay caused by LP-WUS might be longer than that of PEI. RAN2 would like guidance if RAN2 needs to discuss a solution to avoid the delay in emergency call back from SA2 requirement perspective.

- Actions:

RAN2 kindly asks SA2 to provide feedback on the wake up delay when a paging for emergency call back is sent to the UE.

- Date of Next TSG-RAN2 Meetings:

TSG-RAN2 Meeting #129bis	7th – 11th April 2025		China

TSG-RAN2 Meeting #130	19th – 23th May 2025		Malta