3GPP TSG-RAN WG2 #129	R2-2501003

Athens, Greece, 17th – 21st February 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on LS from RAN1 on LP-WUS CONNECTED

Document for:	Discussion and decision

Agenda Item:	8.4.4

- Introduction

This contribution provides our view on open issues related to procedures for LP-WUS in CONNECTED.

- Discussion

According to the LS (R2-2500012) RAN1 agreed as follows

Conclusion - For Option 1-1 of LP-WUS CONNECTED mode operation, RAN1 assumes existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and drx-InactivityTimer for the DRX group - When UE is not in C-DRX active time, UE monitors LP-WUS - Send LS to RAN2 to ask whether RAN2 has same understanding or not. LS in R1-2410909. Agreement - For Option 1-2 of LP-WUS CONNECTED mode operation, for explicit indication to stop the new timer for PDCCH monitoring in Option 1-2 and drx-InactivityTimer for the DRX group, RAN1 identified the following potential solution - Use existing DRX Command MAC CE and Long DRX Command MAC CE - When UE is not in C-DRX active time, UE monitors LP-WUS - Send LS to RAN2 to ask the feasibility of the above potential solution. LS in R1-2410909.

For Option1-1, It is specified in specified in TS 38.321 Section 5.7 that DRX Command MAC CE and Long DRX Command MAC CE to stop drx-onDurationTimer and drx-InactivityTimer for the DRX group.

Proposal1: RAN2 confirms that existing DRX Command MAC CE and Long DRX Command MAC CE can stop drx-onDurationTimer and drx-InactivityTimer for the DRX group in Option 1-1.

In Option 1-2, a new timer that is very similar to the drx-onDurationTimer is introduced, and the timer stopping behavior is equivalent to that of Option 1-1. In other words, just as the relevant MAC CEs can stop the drx-onDurationTimer, it is considered feasible that the new timer can also be stopped.

Proposal2: RAN2 confirms that it is feasible for existing DRX Command MAC CE and Long DRX Command MAC CE to stop the new timer for PDCCH monitoring in Option 1-2 and drx-InactivityTimer.

In summary from a technical perspective, both options are considered feasible. From the RAN2 perspective, the main difference is whether a new timer is introduced or not, and there is no significant impact on the specifications.

- Summary and proposal

Proposal1: RAN2 confirms that existing DRX Command MAC CE and Long DRX Command MAC CE can stop drx-onDurationTimer and drx-InactivityTimer for the DRX group in Option 1-1.

Proposal2: RAN2 confirms that it is feasible for existing DRX Command MAC CE and Long DRX Command MAC CE to stop the new timer for PDCCH monitoring in Option 1-2 and drx-InactivityTimer

- References

[1]	R2-2409052, “Discussion on LP-WUS for CONNECTED state,” NTT DOCOMO, INC., RAN2#127bis.

[x]	R2-2500012, “LS on LP-WUS operation in CONNECTED mode,” RAN1, RAN2#129.

- Attachment Draft LS Reply

3GPP TSG-RAN WG2 #129	R2-25xxxx

Athens, Greece, 17th – 21st February 2025

Title:	Reply LS on LP-WUS operation in CONNECTED mode

Response to:	R2-2500012

Release:	Rel-19

Work Items:	NR_LPWUS

Source:	RAN WG2

To:	RAN WG1

CC:

Contact Person:

#### Name:	Kenichiro Aoyagi

E-mail Address:	kenichirou.aoyagi.yv@nttdocomo.com

Attachment:	               -

- Overall Description:

RAN2 thanks RAN1 for the LS on LP-WUS operation in CONNECTED mode.

Regarding to the agreement on Option 1-1, RAN2 confirmed that existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and drx-InactivityTimer for the DRX group.

Regarding to the agreement on Option 1-2, RAN2 confirmed that existing DRX Command MAC CE and Long DRX Command MAC CE can be used for explicit indication to stop the new timer for PDCCH monitoring in Option 1-2 and drx-InactivityTimer for the DRX group.

2. Actions:

To RAN WG1

ACTION: RAN2 respectfully asks RAN1 to take the reply above into account.

3. Date of Next RAN WG2 Meetings:

TSG-RAN WG2 Meeting #129bis	April 7 to April 11, 2025			China, CN

TSG-RAN WG2 Meeting #130	May 19 to May 23, 2025			Malta, MT