3GPP TSG-RAN WG2 Meeting #129	R2-2501005

Athens, Greece, 17th February – 21th February, 2025

Agenda Item		:	8.8.4

Source		:	LG Electronics Inc.

Title		:	Discussion on MII procedure in NTN

Document for		:	Discussion and Decision

1. Introduction

In the last meeting, RAN2 agreed following statement:

RAN2#127 Agreements: - For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2) - For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3) RAN2#128 Agreements: - A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbor cell(s). - RAN2 understands that the geographic area information for the intended service areas can be semi-static and not cause frequent updates.

In RAN2#127bis, RAN2 discussed the following issue, but it is not treated in RAN2#128:

- MII procedure initiation based on the intended service area Proposal 4: For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the MBS Interest Indication procedure when UE is entering or leaving the intended area. - QC thinks this is up to UE and there is nothing to reflect in the spec. Xiaomi agrees - HW thinks that when entering the area this makes sense, but wonder about the case when the UE leaves the area. - Apple thinks the UE could initiate the MII procedure in any case and then it’s up to NW implementation how to behave. LG agrees with Apple. Oppo also agrees. CMCC thinks in this case the NW has no idea of the UE location in this case. - Come back in the next meeting

In this contribution, we provide our views on MII procedure initiation in NTN.

2. Discussion

RAN2 did not conclude whether the enhancement is needed or not to support MII procedure initiation upon entering/leaving the intended service area. One company thinks that the UE in RRC_CONNECTED does not need to initiate the MII procedure upon leaving the intended service area. However, it is worth that the UE indicates absence of interest in any of MBS broadcast services provided in serving cell. The network may release the MBS broadcast service reception capability of the UE by using this information. As a result, an MBS-capable UE in RRC_CONNECTED may need to inititate the MII procedure initiation upon entering or leaving the intended service area.

On the other hand, most of the companies commented in RAN2#127bis thinks that the MII procedure may be initiated by RRC_CONNECTED UE in any time. However, in our understanding, it is a misunderstanding of the procedural text. According to the TS 38.331 section 5.9.4.2, several cases are specified when UE may initiate the MII procedure. Although UE behavior is specified as “may”, it does not mean that the MBS-capable UE in RRC_CONNECTED could initiate the MII procedure whenever it wants. Therefore, modification of specification is needed to support the MII procedure initiation upon entering or leaving the intended service area.

Observation 1	When UE in RRC_CONNECTED is not interested in any of MBS broadcast services provided in serving cell, it is redundant to maintain the MBS broadcast service reception capability.

Proposal 1	An MBS-capable UE in RRC_CONNECTED may initiate the MII procedure upon entering or leaving the intended service area.

It can be easily implemented in the current spec by adding a simple clarification as a note. There is a condition initiating the MII procedure upon entering or leavig the broadcast service area. In our understanding, the intended service area is the broadcast service area.

Proposal 2	Add a note to section 5.9.4.2 of TS 38.331 as the intended service area can be considered as the broadcast service area.

3. Conclusion

In this contribution, following statements are proposed:

Observation 1	When UE is not interested in any of MBS broadcast services provided in serving cell, it is redundant to maintain the MBS broadcast service reception capability.

Proposal 1	An MBS-capable UE in RRC_CONNECTED may initiate the MII procedure upon entering or leaving the intended service area.

Proposal 2	Add a note to section 5.9.4.2 of TS 38.331 as the intended service area can be considered as the broadcast service area.

4. Reference

[1] R2-2409502, Report of 3GPP TSG RAN WG2 meeting #127bis, Hefei, China.