3GPP TSG-RAN WG2 #129	R2-2501006

Athens, Greece, 17th – 21st February 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on RRC CONNECTION load balancing for LP-WUS capable UEs

Document for:	Discussion and decision

Agenda Item:	8.4.2

- Introduction

RAN-P#106 agreed in RP-243266 as following

- For idle/inactive mode, consider a standardized solution for intra-frequency case (e.g. solution 1, 2, and 3 in RP-242656).

- For Rel-19, inter-frequency solutions (e.g. solution 4) will not be considered.

- Discussion

Under intra-frequency conditions, it is necessary to minimize the impact on legacy UEs. Currently, congestion control for legacy UEs is achieved through common priority via SIB, redirection via RRC CONNECTION RELEASE, or dedicated priority. These controls are in accordance with the NW operation policy, and therefore, even if the NW operates LP-WUS cells, they must be similarly applicable to LP-WUS Capable UEs.

On the other hand, to maximize the benefits of LP-WUS, it is understandable to operate by concentrating LP-WUS Capable UEs in LP-WUS cells as much as possible. To minimize the impact on the current Cell Selection/Reselection, it is preferable to adopt an operation where the LP-WUS frequency is given the highest priority only when an LP-WUS UE enters the LP-WUS state. At the same time, the activation of controls that prioritize the LP-WUS band for LP-WUS UEs should be under the control of NW. It is considered appropriate to configure this information in SIB1 along with other LP-WUS-related information.

Proposal 1: Indicate some information on SIB(SIB1) for the LP-WUS capable UEs to prioritize the current LP-WUS band is the highest priority in which the UE monitors LP-WUS.

When performing an operation that concentrates LP-WUS capable terminals in an LP-WUS cell, congestion may occur in the LP-WUS cell. In order to avoid or mitigate congestion in a specific band caused by LP-WUS terminals, it is necessary to control the operation of both LP-WUS UEs and non-LP-WUS UEs(incl. legacy UEs). For example, if LP-WUS is configured only in a specific band, an operation that concentrates LP-WUS terminals in the LP-WUS band can be considered.

Observation 1. The existing cell selection/reselection and redirection can be applied for both LP-WUS capable UEs and non LP-WUS UEs.

In addition, some UEs may not support specific bands or LP-WUS due to their band compatibility in UE capability. Since system information cannot control with individual UE capabilities, such control with UE capability shall be implemented using dedicated signaling.

Proposal 2: Dedicated signal can be applied to support LP-WUS capable UE to camp on LP-WUS Band

- Summary and proposal

Proposal 1: Indicate some information on SIB(SIB1) for the LP-WUS capable UEs to prioritize the current LP-WUS band is the highest priority in which the UE monitors LP-WUS.

Proposal 2: Dedicated signal can be applied to support LP-WUS capable UE to camp on LP-WUS Band