3GPP TSG-RAN WG2 Meeting #129		         R2-2501008

Athens, Greece, 17th – 21st February 2025

Agenda Item:	8.5.3

Source: 	ITRI

Title:  	Discussion on handling RLF in a NES cell

Document for:	Discussion

Introduction

In RAN plenary #106 meeting, whether a connected mode UE could support on-demand SIB1 procedure was discussed in RP-243297. The conclusion of this discussion was “RAN2 will address the specific case of RLF, to use the same procedure as idle/inactive to request OD-SIB1 transmission”.

In this contribution, we present our preliminary thoughts on the connected mode UE requesting OD-SIB1 transmission due to RLF.

Discussion

When a connected mode UE declares Radio Link Failure (RLF), it will attempt to recover the connection by triggering the connection re-establishment procedure. During this procedure, the UE should perform cell selection to camp on a suitable cell within the T311 timer. Otherwise, the UE will enter idle mode.

To verify whethet the selected cell is a suitable cell, the UE will check the PLMN information within System Information Block 1 (SIB1).  If the connected mode UE triggers re-establishment and selects a NES cell that currently does not provide SIB1, it should be able to request On-Demand SIB1 (OD-SIB1) transmission. Otherwise, the UE might spend additional time selecting another cell that provides SIB1.

Proposal 1: For re-establishment caused by RLF, a connected mode UE should be able to request OD-SIB1 transmission if the selected cell is an NES cell that currently does not provide SIB1, similar to how an idle mode/inactive state UE requests OD-SIB1.

To support OD-SIB1 requests from connected mode UEs during RLF-triggered re-establishment, the UE needs UL WUS configuration.  As previously agreed, this configuration will be carried in a new SIB.  Therefore, the connected mode UE will acquire the UL WUS configuration when acquires this new SIB.

Because the UL WUS configuration is carried in a new SIB, its modification follows the SI change scheme. This means the updated UL WUS configuration is broadcast in the modification period following the SI change indication transmission.

However, after declaring RLF, the UE cannot receive the SI change indication or the updated UL WUS configuration. This can lead the UE to use an incorrect UL WUS configuration to request OD-SIB1. An incorrect UL WUS configuration can cause the OD-SIB1 request to fail and interfere with other UEs served by the NES cell.

Current specifications cannot prevent RLF when an NES cell attempts to modify its UL WUS configuration. Therefore, it is possible that a connected mode UE uses an incorrect UL WUS configuration to request OD-SIB1 due to RLF.

Consequently, it is important to discuss how to mitigate the impact when a connected mode UE requests OD-SIB1 using an incorrect UL WUS configuration.

Proposal 2: RAN2 should study how to reduce the impact when a connected mode UE requests OD-SIB1 by a wrong UL WUS configuration.

Mitigating this impact can be approached from either a network-driven or a UE-driven perspective.

For a network-driven solution, the network sholuld be able to configure which NES cells support OD-SIB1 requests from connected mode UEs. The NES cells that frequently modify their UL WUS configuration should be able to prevent connected mode UEs from requesting OD-SIB1, as the probability of the UE using an incorrect configuration is higher.

For a UE-driven solution, the number of a connected mode UE's retries OD-SIB1 request should be limited to one, or fewer than preambleTransMax. Because a connected mode UE's OD-SIB1 request failure can be due to signaling issues or an incorrect UL WUS configuration. Limiting retries number could reduce the impact on other UEs.  Therefore, the connected mode UE should not retry OD-SIB1 requests as frequently as idle mode/inactive state UEs.

Proposal 3: RAN2 should futher study network-driven or UE-driven solutions to reduce the impact when a connected mode UE requests OD-SIB1 with an incorrect UL WUS configuration.

Conclusion

Proposal 1: For re-establishment caused by RLF, a connected mode UE should be able to request OD-SIB1 transmission if the selected cell is an NES cell that currently does not provide SIB1, similar to how an idle mode/inactive state UE requests OD-SIB1.

Proposal 2: RAN2 should study how to reduce the impact when a connected mode UE requests OD-SIB1 by a wrong UL WUS configuration.

Proposal 3: RAN2 should futher study network-driven or UE-driven solutions to reduce the impact when a connected mode UE requests OD-SIB1 with an incorrect UL WUS configuration.