3GPP TSG RAN WG2#129	R2-2501010

Athens, Greece, 17th - 21st February 2025

Agenda item:	8.8.2

Source: 	ITRI

Title: 	Discussion on SMTC for NTN DL coverage enhancements

Document for:	Discussion

1	Introduction

RAN1 agreed that the SSB periodicity for NTN DL_CE could be extended up to 160 ms, The following agreements were made in RAN2 [1]:

Agreements (RAN2#128): 1. As part of the work on DL-CE, RAN2 investigates related UE power consumption impact (including legacy UEs) 2. RAN2 will consider whether to introduce separate signalling (e.g. new SMTC5 list) for DL CE cells SMTCs, e.g. if different periodicities need to be signalled or to prevent reselection to specific cells

For UE to perform measurements for cell reselection efficiently, the SMTC impact according to extended SSB periodicity is investigated in this contribution, and proposals are provided accordingly.

2	Discussion

2.1 	Impacts to SMTC of supporting extended SSB periodicity

SMTC could be configured for UE to reduce power consumption in SSB-based measurements of neighbour cells. Network may provision the SMTC occasions for intra-frequency and inter-frequency neighbour cell measurements by SMTC parameters periodicity, duration, and offset. UE is not required to perform SSB measurements beyond SMTC duration.

In RAN1#118bis it was agreed that the default SSB periodicity could be extended to 160 ms (apart from the existing 20 ms value) for NTN DL_CE [2]. It is possible that the neighbour cells are deployed with different default SSB periodicities, e.g., according to satellite capabilities.

SMTC for intra-frequency cell reselection is configured by SIB2, SMTC for inter-frequency cell reselection is configured by SIB4. Based on the existing SMTC framework, both intra-frequency and inter-frequency SMTC for cell reselection supports only one SMTC periodicity per frequency. To enable flexible and power efficient SMTC configuration, it is proposed to enhance the SMTC mechanism to support multiple periodicities per frequency.

Observation 1: Existing SMTC mechanism supports configuring only one periodicity per frequency. For flexible SMTC configuration and power efficient SSB-based measurement, it is beneficial to enhance the SMTC mechanism to support multiple periodicities per frequency.

Proposal 1: RAN2 to agree on supporting SMTC with different periodicities per frequency.

2.2 	Association between SMTC periodicity and PCI

NTN capable UE needs the neighbour cell PCI for evaluating the actual propagation delay difference between the serving cell and the neighbour cell to adjust the actual offset value, as indicated in the field description of SIB2 in TS 38.331 [3].

smtc4list Measurement timing configuration list for NTN deployments, see clause 5.5.2.10. The offset of each SSB-MTC4 in smtc4list is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. For a UE that supports less SMTCs than what is included in this list, it is up to the UE to select which SMTCs to consider.

Observation 2: For NTN, UE needs the PCI of neighbour cells to adjust the actual offset value of the associated SMTC.

From another angle, a UE may be provisioned with allowed list or not-allowed list for cell reselection on a specific frequency (e.g., by RRCRelease message upon RRC connection release) to assist UE in cell reselection. The periodicity parameter may associate with PCI to allow UE to avoid creating additional SMTC for the measurements of a cell which is not allowed to be reselected.

Observation3: Neighbour cells may be deployed with different SSB periodicities, the association between PCI and the value of SMTC periodicity parameter could assist UE to prevent creating SMTC for the measurements of a cell which is not allowed to be reselected.

Proposal 2: RAN2 consider to associate the periodicity parameter of SMTC with PCI.

2.3 	Creating and applying SMTC

As shown in Figure. 1, the two neighbour cells A and B are deployed on the same frequency but with different default SSB periodicity. Without additional assistant information, a UE may need to create two additional SMTC with different periodicities on the same frequency for the measurements of cell reselection upon the reception of the configuration.

Figure 1. Neighbour cells with different SSB periodicities

Considering use cases of NGSO satellites with transparent NTN payload, satellite movement is predictable. For quasi-earth fixed deployment, the time a satellite stops serving an area is provided to UE by System Information. Taking Figure 1 as example, a UE needs not to perform SSB measurements for a cell which is no longer serving the area of the UE. Therefore, the UE may stop applying or remove the SMTC associated with the cell which stopped serving the area for power efficiency.

Observation 4: For quasi-earth fixed NTN cells, the time of satellite switch (service link switch) is predictable. Time-based SMTC could assist UE choosing the proper SMTC for the cells served by the present and upcoming satellites.

Proposal 3: It is proposed RAN2 to support time-based SMTC.

For earth-moving NTN cell deployment, network may provide information of location-based measurements for cell reselection. UE with the capability of location-based measurements could perform measurements accordingly. A UE only needs to perform measurements for the NTN cells serving the area of the UE. Providing location-based SMTC by configuring reference location and distance threshold from NTN cell to assist UE to only perform measurements of the cells serving the area of the UE for cell reselection could enhance power efficiency of UE. Therefore, it is proposed RAN2 to consider supporting location-based SMTC for NTN.

Observation 5: For earth-moving NTN cells, location-based SMTC could assist UE choosing the proper SMTC for the cells according to the reference locations and distance thresholds associated with neighbour cells.

Proposal 4: It is proposed RAN2 to support location-based SMTC.

3	Conclusion

Based on the three parts of discussions, we provide the following observations:

Observation 1: Existing SMTC mechanism supports configuring only one periodicity per frequency. For flexible SMTC configuration and power efficient SSB-based measurement, it is beneficial to enhance the SMTC mechanism to support multiple periodicities per frequency.

Observation 2: For NTN, UE needs the PCI of neighbour cells to adjust the actual offset value of the associated SMTC.

Observation3: Neighbour cells may be deployed with different SSB periodicities, the association between PCI and the value of SMTC periodicity parameter could assist UE to prevent creating SMTC for the measurements of a cell which is not allowed to be reselected.

Observation 4: For quasi-earth fixed NTN cells, the time of satellite switch (service link switch) is predictable. Time-based SMTC could assist UE choosing the proper SMTC for the cells served by the present and upcoming satellites.

Observation 5: For earth-moving NTN cells, location-based SMTC could assist UE choosing the proper SMTC for the cells according to the reference locations and distance thresholds associated with neighbour cells.

And the proposals are provided accordingly:

Proposal 1: RAN2 to agree on supporting SMTC with different periodicities per frequency.

Proposal 2: RAN2 consider to associate the periodicity parameter of SMTC with PCI.

Proposal 3: It is proposed RAN2 to support time-based SMTC.

Proposal 4: It is proposed RAN2 to support location-based SMTC.

4	References

[1] R2-25xxxxx	Report of 3GPP TSG RAN WG2 meeting #128, Orlando, USA.

[2] R1-2409341	Report of RAN1#118bis meeting

[3] 3GPP TS 38.331 V18.4.0: "NR; Radio Resource Control (RRC); Protocol specification".