# 3GPP TSG-WG2 Meeting #129              	   R2-2501012

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

# Source: 			CMCC

# Title:  Discussion on IoT NTN Store and Forward

# Agenda item:	8.9.2

# Document for: 	Discussion

Introduction

In the last RAN2#128 meeting, the following agreements [1] were approved as below:

Agreements:

1.	Time information can be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed (FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer). FFS which SIB is used (SIB1 or SIB31). FFS if absolute or relative time is used. FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start)

2.	It is FFS whether we need to introduce a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable

3.	“S&F operation” indication is provided in SIB1

4.	RAN2 assumes that AS signalling is not needed to indicate the architecture option (Split MME or Full CN).

5.	RAN2 waits for further progress in SA2, if any, before working on RAN2 aspects of a CIoT-UP solution for SF operation

6.	RAN2 understands that the UE configured with a satellite ID list by MME is not prevented to camp on a satellite operating in normal IoT NTN mode (i.e. with feeder-link connection), and perform subsequent access and data/signalling communication with that satellite

7.	RAN2 understands the UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.

Based on the discussions and conclusions in the previous meetings, we continue discussing some RAN2 related aspects in S&F operation.

Discussion

Based on the discussions and conclusions in the previous meetings, we continue discussing some RAN2 related aspects in S&F operation.

Time information

In the RAN2#128 meeting, RAN2 had made a conclusion that：“Time information can be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed”[1]. Besides, several FFS for time information have been left as below:

- FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer

- FFS which SIB is used (SIB1 or SIB31)

- FFS if absolute or relative time is used

- FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start)

Next we are going to discuss these FFS for time information one by one. For the first FFS, before we differentiate R19 UEs supporting and not supporting S&F mode, we first need to figure out two kinds of UEs behaviour when the mode switches from S&F mode to normal mode. The following Table 1 gives a brief summary for using time information:

Table 1 The benefits for using time information for R19 S&F capable UE and not capable UE

| Benefits | R19 UE supporting S&F mode | R19 UE not supporting S&F mode |
| --- | --- | --- |
| S&F to normal | - Adjust configurations/services in advance to reduce performance degradation - UE can defer some NAS procedure until normal mode. | 1.UE can know when to access the cell and do not need to wait cellBarredNTN in SIB1 changed. 2.UE can defer some NAS procedure until normal mode. |
| Normal to S&F | Adjust configurations/services in advance to reduce performance degradation | UE can know when to go back to idle state/reselect to other cells in advance |

From Table 1, we can see that time information is useful for R19 S&F capable UE and not capable UE including that:

- If the current mode is in normal NTN mode and soon switch to S&F operation:

- For S&F capable UEs, UE can adjust configurations/services in advance (e.g. release services with high QoS requirement or high latency requirements). So that UE can try to prevent performance degradation.

- For R19 UE not supporting S&F mode, UE can know when the service cell will be unavailable, thus UE also can adjust configurations/services for the interruption and try to reselect to other cells or change to other available satellite if any.

- If the current mode is in S&F operation and soon switch to normal NTN mode:

- For S&F capable UEs, similarly to normal NTN switching to S&F operation, UE can adjust configurations/services in advance (e.g. resume high services with high QoS requirement or high latency requirements); Besides, if UE wants to start initial access to the current satellite and the time information of S&F operation is short, then UE can wait till the cell switches back to normal NTN cell, then UE try initiate the access procedure to prevent multi rounds RA steps and save signaling overheads.

- For R19 UE not supporting S&F mode, UE can know when to access the cell and do not need to wait cellBarredNTN in SIB1 changed. Based on previous meeting conclusion, R19 UE can also defer some NAS procedure until normal mode.

According to the UE behaviour during mode switching from S&F to normal, both kinds of UE behave similarly, thus, we think there is no need to differentiate the behaviour for R19 UEs supporting and not supporting S&F mode.

Proposal 1: There is no need to differentiate the behaviour for R19 UEs supporting and not supporting S&F mode.

In the FFS 1, the Wait timer is also mentioned whether can be considered as time information of mode switching, from our point of view, the Wait Timer is only received by S&F capable UEs in the MME-split architecture when the network rejects the initial access of UE. After the Wait Timer expires, the next coming satellite may still be in S&F mode. There is no direct relationship between the Wait Timer and mode switching between S&F and normal mode. Therefore, the Wait Timer cannot be used to provide time information during mode switching.

Proposal 2: It is proposed to not use the Wait Timer provide time information during mode switching between normal mode and S&F mode.

As for FFS 2, we prefer using SIB31 message to send time information, since SIB1 resource is precious and limited. Only S&F indication included in SIB1 is enough. For other specific S&F parameters or configuration, we can put them in other SIBs (e.g. SIB31, 32 and 33). Thus, for this time information used in current serving cell, we think it is better to use SIB31 to send time information.

Proposal 3: It is proposed to use SIB31 to send time information.

For FFS 3 of the details of time information, we prefer to use a absolute value to indicate the time information. Since if just a relative value is indicated, there may be latency or loss during the transmission which may cause inconsistent time information between UE and satellite after the transmission. In this circumstance, UE may go back to idle state too early or too late and behave not as the network expects. Thus, we think a absolute value  is a safer way to use

Proposal 4: It is proposed to use absolute time for the time information.

For FFS 4, as per the summary of Table 1, we list some benefits for both R19 S&F capable UE and not capable UE in the bi-directional mode switching. Thus, we think the mode switching from normal mode to S&F mode can also be considered, for example, R19 UEs can adjust related AS configurations and parameters or switch to different services in order to adjust different satellite modes.

Proposal 5: The mode switching from normal mode to S&F mode can also be considered.

For the multi mode switching scenarios, if the satellite knows there are frequently operation switching between normal operation and S&F operation before t-service timeout (e.g. the feeder link status is not stable), it’s better for network to indicate all switching patterns via SIB message (which means sending all indication information of every operation switching in SIB message at once). To be specific, if there are lots of operation switching between normal and S&F operation (e.g. more than 4 times), dynamic indication requires the network to send four indications when switching that will cause big signaling overheads. If UE failed to receive SIB message and there will be some latency and inconsistency. Thus, by using this kind of half-static method, the network can send all patterns of every operation switching in SIB message for the first time UE accesses in. When the patterns are not changed, network does not need to send it again. If changed, only sending changed part is enough. In this way, UE is clear to know the total numbers of operation transition and the duration time of each operation which will bring great convenience for UE to set related parameters and configurations in advance and also adjust service based on different patterns. If the S&F operation information does not change, the network also does not need to change the broadcast message and if some information is changed (e.g. duration time or S&F operation start time, etc), then the network only needs to broadcast the changed information in SIB message.

Proposal 6: For multi operation transition scenario, satellite can send all time information of every S&F mode switching in SIB message when the UE first camps on the cell .

Proposal 7: For multi operation transition scenario, the satellite only needs to broadcast the changed S&F operation information (e.g. starting time, duration time) after UE camps on the cell.

RRC release

In the last meeting, we discussed whether to introduce a new RRC release cause when the feederlink is unavailable, from our point of view, when R19 S&F capable UE is in the RRC connected state, the network will not release R19 S&F capable UE to idle state when mode switching from normal to S&F happened. Thus, what the network needs to release is R19 UE not supporting S&F during the mode switching from normal to S&F. However, introducing a new RRC release value due to S&F but for R19 UE not support S&F mode doesn't seem necessary. R19 UE that does not support S&F can return to idle based on the existing RLF process. Thus, we think introducing a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable is not necessary.

Proposal 8: Introducing a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable is not necessary.

Neighbor cell information

In the last meeting, we talked about adding S&F neighbor cell information in SIB5 but didn’t reach an agreement. For adding S&F neighborhoods to SIB5 intraFreqAllowedCellList and interFreqAllowedCellList, the neighbor cells can be from the same satellite or from different satellites, The S&F neighbor cell information from different satellites can be obtained from the satellite ID list sent by the CN in the initial access rejection message. However, at present, we are still waiting for SA2's reply on whether all satellites in the satellite ID list support S&F mode. If all satellites support S&F, then we can use this list to obtain the neighborhood information of different satellites, but if some satellites in the list do not support S&F, the list then cannot be used to obtain the neighborhood information of different satellites, and other methods such as ISL or core network allocation may need to be considered to get the neighbour satellite cells information. Therefore, it is recommended to discuss this issue after SA2 gives feedback on the satellite ID list.

Proposal 9: It is proposed to defer introducing S&F specific neighbor cell list in SIB5 till SA2 gives feedback on the satellite ID list issue.

Paging enhancement

When S&F capable UE work in the cell operating in S&F mode or out of satellite coverage, UE does not need to monitor paging message all the time. That means UE can relax the paging monitoring for some scenarios shown as below:

- When UE is not within the satellite coverage area:

  - The UE can determine when the next satellite carrying the DL data it needs (paging itself) will arrive based on the waiting timer in NAS rejection message. Therefore, when the UE is not within the coverage area of the satellite that means there is no satellite carrying downlink data to wake up the UE. Thus, the UE does not need to monitor the paging message.

- When UE is within the satellite coverage area and camps on a cell in S&F operation:

- The network has already transmitted stored DL data: as for the normal procedure, the satellite sends the stored downlink messages to the UE. If there is no data communication and the DRX timer runs out, the UE goes back to idle state. Therefore, during the time information of the S&F operation, the network will not send new downlink data to the UE since feederlink does not recover. Consequently, unless the UE actively needs to transmit UL data, it will not enter the connected state, and thus the UE does not need to monitor paging messages.

- The satellite never recover its feederlink: That means the current satellite does not have data transmission with ground EPC. Thus, the satellite only has stored UE uplink data and no new arrived downlink data. Therefore, even UE is within the coverage area, without recovered feederlink, UE does not need to monitor paging and only switch back to RRC connected state when has uplink data to send.

Proposal 10: S&F capable UE can relax the paging monitoring in the following scenarios:

- UE is not within the satellite coverage area

- UE is in a cell operating in S&F mode:

- The network has already transmitted stored DL data

- The satellite never recover its feederlink

Conclusions

- Time information

Proposal 1: There is no need to differentiate the behaviour for R19 UEs supporting and not supporting S&F mode.

Proposal 2: It is proposed to not use the Wait Timer provide time information during mode switching between normal mode and S&F mode.

Proposal 3: It is proposed to use SIB31 to send time information.

Proposal 4: It is proposed to use absolute time for the time information.

Proposal 5: The mode switching from normal mode to S&F mode can also be considered.

Proposal 6: For multi operation transition scenario, satellite can send all time information of every S&F mode switching in SIB message when the UE first camps on the cell .

Proposal 7: For multi operation transition scenario, the satellite only needs to broadcast the changed S&F operation information (e.g. starting time, duration time) after UE camps on the cell.

- RRC release

Proposal 8: Introducing a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable is not necessary.

- Neighbor cell information

Proposal 9: It is proposed to defer introducing S&F specific neighbor cell list in SIB5 till SA2 gives feedback on the satellite ID list issue.

- Paging enhancement

Proposal 10: S&F capable UE can relax the paging monitoring in the following scenarios:

- UE is not within the satellite coverage area

- UE is in a cell operating in S&F mode:

- The network has already transmitted stored DL data

- The satellite never recover its feederlink

References

- 3GPP Chair_Notes, R2-128 NR-NTN-IoT-NTN (Sergio) EOM