3GPP TSG-RAN2 Meeting #129			R2-2501013

Location, Country, Date

Agenda item:	8.2.3

Title:	Further consideration on A-IoT random access

Source:	CMCC

Document for:	Decision

Introduction

In the last meeting, RAN2 discusses some details on 3-step and 2-step CBRA, unified solution for CBRA and CFRA, re-transmission and feedback, and re-access. Besides, RAN2 also reaches the following agreements.

Agreements

- For 2 step and 3step CBRA, capture the unified solution in the TR and capture advantage and disadvantage of the solutions.  This requires that devices would support both forms.

- FFS recommendation on whether we have 3step only or we have unified solution for 2step and 3step CBRA.

- Capture in TR as one option for unification for CBRA and CFRA.  For this option random ID is always included in msg1.  Capture disadvantage of reader doesn’t know whether the intended device is responding on the given resources.

- RAN2 assumes, A-IoT reader can send a Msg2 transmission to a specific device for the random ID, if it doesn’t receive msg3 from that ID

- If msg2 contains multiple devices, re-transmitted Msg2 will only include the random IDs of failed devices (if supported).

- The "R2D failure/success feedback indication" also applies to "following D2R data" after the random access procedure for re-access purpose

- Capture the following option in TR: The devices can be triggered by R2D message to re-access before subsequent paging retransmissions.

This contribution will discuss some unsettled issues of A-IoT random access and introduce some potential solutions for them.

Discussion

  - Recommendation on CBRA type

RAN2 has introduced 2-step and 3-step CBRA before, and in the last meeting, a unified solution for 2-step and 3-step CBRA is discussed in the last meeting.

For the unified solution, the reader should signal the device whether to send data with A-IoT Msg1, then the device will assemble Msg1 based on the reader's instruction. However, it is worth noting that device 1, which is currently the primary focus of current WID, is a highly constrained device with limited capabilities and a strict cost.

In the existing 2-step and 3-step CBRA procedures, Device 1 can transmit A-IoT Msg1 in a fixed-length format, simplifying implementation and reducing processing overhead. In contrast, under a unified CBRA scheme, the content and format of Msg1 would be dynamically determined by the reader’s signalling. This flexibility imposes additional burdens on device 1, as it must process varying instructions and potentially support multiple Msg1 formats.

Given that device 1 is specifically designed to operate as a low-cost and low-capability device, introducing such dynamic requirements may contradict the design principles and optimization goals for this type of devices. It is therefore critical to carefully evaluate whether the benefits of a unified CBRA procedure outweigh the increased complexity for constrained devices.

Observation 1a: A unified 2-step and 3-step CBRA scheme can make device 1 more complicated.

Another critical issue worth highlighting is the energy consumption disparity between 2-step and 3-step CBRA procedures. A key factor contributing to this gap is the high likelihood of collisions in A-IoT Msg1 when multiple devices attempt access simultaneously. If data is included in A-IoT Msg1, collisions not only result in access failures but also lead to significant energy wastage.

The following figure provides an illustrative comparison of energy consumption for 2-step and 3-step CBRA under different traffic load conditions. The traffic load (G) considered are 0.1, 0.5, and 0.9, whose definition is the ratio of the number of device packets to the number of access occasions. For this analysis, the device ID or data length is assumed to be 128 bits.

Figure 1. Average bits sent per device in 2-step and 3-step CBRA with vary load(G).

As can be seen from the figure, both two-step and three-step access methods, it can be observed that the average bits per device increase with higher traffic loads (from G=0.1 to G=0.9). This trend indicates that increased contention leads to more bits being required for successful access resolution.

When comparing the 2-step and 3-step approaches, for the same traffic load, the 2-step CBRA consistently results in a higher average number of bits compared to its 3-step counterpart. As the number of devices increases, the gap between the 2-step and 3-step CBRA widens slightly, suggesting that with higher device densities, 3-step can be even more energy-efficient.

Observation 1b: In case of collision, 3-step CBRA requires device to send fewer bits than 2-step CBRA, due to Msg1 in 3-step CBRA is much shorter than that in 2-step CBRA.

As shown in the figure, 3-step CBRA consistently demonstrates better energy efficiency compared to 2-step CBRA, regardless of the load conditions. This is primarily because in 3-step CBRA, only the 16-bit Msg1 will collides and Msg3 should be sent without collision. While in 2-step CBRA, if Msg1 is collided, the energy to send data in Msg1 is also wasted.

Given the stringent energy constraints of A-IoT devices, the improved energy efficiency of 3-step CBRA makes it a more suitable option in scenarios with moderate to high device loads. While 2-step CBRA offers simplicity, its susceptibility to collisions poses significant drawbacks in terms of energy consumption, particularly for network with a high load.

Observation 1c: 3-step CBRA always has less device energy consumption compared to 2-step CBRA.

Hence, for Release 19, it is recommended to focus on the study and specification of the 3-step CBRA procedure exclusively. From our perspective, while 2-step CBRA may prove to be beneficial for certain time-critical use cases that could emerge in the future, the current priority should remain on refining and specifying the 3-step CBRA.

Another observation is that if device determines to segment A-IoT Msg1 with data, it would be naturally transfer 2-step CBRA to 3-setp CBRA, i.e., the first segmentation send the random ID, and reader echoes Msg2 with random ID, then device sends the remaining segmentations that can be regarded as Msg3.

Observation 1d: Segmenting A-IoT Msg1 with data can naturally transition a 2-step CBRA procedure into a 3-step CBRA procedure.

The 3-step CBRA procedure offers a simpler and energy-efficient solution, especially under moderate to high load conditions, making it better suited for addressing the current requirements of A-IoT devices.

Proposal 1: RAN2 is recommended to have 3-step CBRA only in Release 19 for simplicity and universality.

Additionally, 2-step CBRA can be considered as a potential enhancement to the 3-step CBRA scheme in the future.

  - Unification of CBRA and CFRA

In the previous meeting, RAN2 discussed the unified design for CBRA and CFRA, which is kind of related to the undecided AS ID functionality. Under the current CFRA scheme, devices can directly transmit data, e.g., the device ID, to the reader. Therefore, the necessity of transmitting a random ID in CFRA remains unclear.

In CFRA, the primary role of introduction a random ID seems to be related to scheduling and resource allocation as there is no contention to solve. However, the device ID or the identifier in CFRA paging message is equally capable of serving this purpose, rendering the additional use of a random ID redundant. And if reader is allowed to assign AS ID or random ID to device, it would be guaranteed that the assigned ID is unique. More importantly, the motivation and purpose behind mandating the reporting of a random ID have not been clearly articulated.

Observation 2a: The random ID is at least not always necessary for CFRA, as the device ID and the device identifier in CFRA paging message may serve a similar functionality.

Observation 2b: Compared to device to report random ID in CFRA, reader to assign an ID in CFRA has some benefits, e.g., the assigned ID is unique.

Considering that both the generation and transmission of a random ID consume device energy, especially for energy-constrained A-IoT devices, it is essential to critically assess whether this requirement is truly justified. Unless RAN2 provides clear and sufficient justification for the necessity of always reporting random IDs within the CFRA scheme, it would be unreasonable to mandate this functionality without careful consideration of its implications.

Observation 2c: Always including random ID in CFRA consumes more device energy.

There do exist some options for scheduling and resource allocation enhancement for CFRA, for example, reader to assign AS ID instead of device reporting, hence it is suggested not to unify CBRA and CFRA at least in this release.

Proposal 2: RAN2 is not recommended to unify CBRA and CFRA at least in Release 19.

Please note that there is no Msg1 in A-IoT CFRA.

  - Segmentation in A-IoT random access

    - Segmentation for 3-step CBRA

RAN2 has already agreed to introduce segmentation functionality for A-IoT devices. However, it remains unclear whether A-IoT random access messages, i.e., Msg1 and Msg3, can be segmented. This issue has not been previously discussed, and considering the time plan, it is necessary to study segmentation in A-IoT random access.

Given the potentially large size of Msg3, and the fact that Msg3 itself can be regarded as data, allowing segmentation of Msg3 is quite reasonable, especially for energy-constrained A-IoT devices.

Observation 3: For energy-constrained device type 1, supporting segmentation of large A-IoT Msg3 is beneficial.

However, this raises the question of how to handle the transmission of the remaining Msg3 segments. Due to the limited energy storage capability of A-IoT devices, immediate retransmissions, e.g., resending Msg2 to trigger remaining Msg3 segments, may not always be feasible.

Hence, triggering the transmission of Msg3 segments by retransmitting Msg2 might not be an optimal solution. Instead, a more practical approach could be for the reader to decide the transmission occasion of remaining Msg3 segments with appropriate intervals, giving the device sufficient time to recharge. This interval-based strategy would help balance energy consumption and ensure reliable delivery of segmented Msg3 data.

Proposal 3: RAN2 agrees that A-IoT Msg3 can be segmented.

The segmentation issue on Msg1 is notably different. Segmentation of Msg1, particularly Msg1 without data is probably unnecessary for the following reasons:

- Short Message Length: Msg1 with no data is typically short, potentially consisting of only a brief header and a 16-bit random ID. The need for segmentation in such cases is minimal.

- Incomplete random ID: One of Msg1’s primary motivation is to resolve contention during the CBRA process. An incomplete or segmented Msg1 might fail reader to determine whether there is a collision, undermining the effectiveness of the contention resolution mechanism.

Given these factors, Msg1 without data should not be segmented.

Observation 4: Segmenting A-IoT Msg1 without data deteriorates the efficiency of A-IoT random access.

If a device lacks sufficient energy to send a complete Msg1, it should consider this round of access a failure and prepare for a re-access round.

Proposal 4a: RAN2 agrees that A-IoT Msg1 without data should not be segmented.

Proposal 4b: RAN2 agrees that if a device has insufficient energy to transmit A-IoT Msg1 without data, the device should terminate the access procedure and wait for a potential re-access opportunity.

    - Segmentation for 2-step and unified CBRA (if agreed)

If the majority view of RAN2 is to support 2-step CBRA, a new issue, i.e., device with insufficient energy to send A-IoT Msg1 with data, needs to be addressed by RAN2.

For Msg1 with data, due to its potentially long length, a segmentation strategy similar to Msg3 could be adopted. Specifically, it would be beneficial if RAN2 allows segmentation of Msg1 with data, but the first segmentation must at least include the complete random ID to ensure the reader can identify whether there is contention or not.

Such a segmentation strategy is kind of similar to transform 2-step CBRA into 3-step CBRA, i.e., transforms segmented Msg1 with data as Msg1 without data and a following segmented Msg3. This is one of the reasons why we advocate for 3-step CBRA.

RAN2 may further discuss the advantages and disadvantages of these two approaches.

Proposal 5: If 2-step or unified CBRA procedure were agreed, RAN2 could discuss the following options for segmentation in A-IoT Msg1 with data:

Option 1: Device sends A-IoT Msg1 without data and segmented A-IoT Msg3, i.e., reuse 3-step CBRA procedure.

Option 2: Device sends segmented A-IoT Msg1 with data and the first segmentation must include the complete 16-bit random ID, FFS there is any impact on Msg2 design and paging message.

The choice of Option 1 and Option 2 is also depending on whether RAN2 supports 3-step CBRA only, if so, we are happy to revisit this issue.

  - Re-access for failure handling

For our observation, the A-IoT random access failures can generally be categorized as short-term or long-term:

- Short-term failures: typically caused by issues such as collisions with other devices or temporary degradation of channel conditions. For such cases, reader may trigger immediately re-access, ensuring the effectiveness.

- Long-term failures: on the other hand, are more likely caused by limitations of the device itself, such as insufficient energy or the device moving out of the reader's coverage area. In cases where random access fails due to insufficient energy, initiating re-access too early (e.g., triggering Msg3 retransmission via Msg2) may prevent the device from harvesting enough energy. For such cases, reader may trigger re-access after some time.

Re-access failures caused by insufficient energy can significantly reduce efficiency and waste valuable energy. If a device initiates re-access with inadequate energy, it risks failing again, leading to further inefficiencies.

To address this, the device should evaluate its energy status before initiating re-access. By allowing the device to skip one or more re-access attempts when energy is insufficient, it could harvest more energy, ensure successful transmissions, and ultimately improve overall efficiency while extending the device's lifespan.

Proposal 6: RAN2 confirms that although re-access is triggered by R2D message, device can decide whether to perform re-access based on its energy status, e.g., device can skip one or multiple re-access opportunities to harvest more energy.

Conclusion

This contribution discusses the remaining issue on random access for Ambient IoT and propose the following,

CBRA type:

Observation 1a: A unified 2-step and 3-step CBRA scheme can make device 1 more complicated.

Observation 1b: In case of collision, 3-step CBRA requires device to send fewer bits than 2-step CBRA, due to Msg1 in 3-step CBRA is much shorter than that in 2-step CBRA.

Observation 1c: In case of collision, 3-step CBRA always has less device energy consumption compared to 2-step CBRA.

Observation 1d: Segmenting A-IoT Msg1 with data can naturally transition a 2-step CBRA procedure into a 3-step CBRA procedure.

Proposal 1: RAN2 is recommended to have 3-step CBRA only in Release 19 for simplicity and universality.

Unified CBRA and CFRA:

Observation 2a: The random ID is at least not always necessary for CFRA, as the device ID and the device identifier in CFRA paging message may serve a similar functionality.

Observation 2b: Compared to device to report random ID in CFRA, reader to assign an ID in CFRA has some benefits, e.g., the assigned ID is unique.

Observation 2c: Always including random ID in CFRA consumes more device energy.

Proposal 2: RAN2 is not recommended to unify CBRA and CFRA at least in Release 19.

Segmentation in Msg3:

Observation 3: For energy-constrained device type 1, supporting segmentation of large A-IoT Msg3 is beneficial.

Proposal 3: RAN2 agrees that A-IoT Msg3 can be segmented.

Segmentation in Msg1:

Observation 4: Segmenting A-IoT Msg1 without data deteriorates the efficiency of A-IoT random access.

Proposal 4a: RAN2 agrees that A-IoT Msg1 without data should not be segmented.

Proposal 4b: RAN2 agrees that if a device has insufficient energy to transmit A-IoT Msg1 without data, the device should terminate the access procedure and wait for a potential re-access opportunity.

Proposal 5: If 2-step or unified CBRA procedure were agreed, RAN2 could discuss the following options for segmentation in A-IoT Msg1 with data:

Option 1: Device sends A-IoT Msg1 without data and segmented A-IoT Msg3, i.e., reuse 3-step CBRA procedure.

Option 2: Device sends segmented A-IoT Msg1 with data and the first segmentation must include the complete 16-bit random ID, FFS there is any impact on Msg2 design and paging message.

Re-access:

Proposal 6: RAN2 confirms that although re-access is triggered by R2D message, device can decide whether to perform re-access based on its energy status, e.g., device can skip one or multiple re-access opportunities to harvest more energy.

References

[1]	R2_128_ChairNotes_24-11-22_17-00_eom