3GPP TSG-RAN2 Meeting #129			R2-2501014

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.7.4.1

Title:	Consideration on LCP enhancement for XR

Source:	CMCC

Document for:	Decision

Introduction

In the last meeting [1], RAN2 has the following agreements,

Agreements on LCP prioritization - As a baseline, the additional LCH priority is applied to both the first round and the second round of the LCP procedure. The UE does not fall back to the default LCH priority in the second round even if there is no more LCH priority-adjusted data after the first round. - As an optional capability, the UE can also support to fallback to default priority in the 2nd round of LCP.

This contribution will discuss some remaining issues on LCP enhancement for XR and propose some solutions for that.

Discussion

  - Delay-aware LCP configuration

As RAN2 excludes intra-LCH prioritization in previous meeting, i.e., per PDU or PDU set prioritization is not infeasible. However, per UE and per LCH group prioritization remains too coarse-grained to address the diverse QoS requirements of traffic flows e.g., mixed delay-critical XR traffic and non-delay-critical RRC messages. This lack of granularity also creates challenges in ensuring efficient resource allocation for traffic with varying delay and reliability requirements.

As a starting point, per LCH configuration is a suitable solution for additional priority. It can be configured in the LogicalChannelConfig IE. This approach balances the need for finer-grained prioritization while maintaining compatibility with existing RAN2 decisions.

RAN, i.e., gNB RRC layer, can configure the additional priority for delay aware LCP. CN may indicate some assistance information via NGAP signaling, e.g., whether there will be delay-critical data or the corresponding LCH is delay-aware but it should be left for gNB to decide how and whether to configure delay-aware LCP configurations. UE can also report its desired additional priority for delay-critical data, e.g., in UAI but it is valid only if gNB confirms.

Proposal 1a: RAN is responsible for configuration of delay-aware LCP based on CN information, e.g., existence of PDB/PSDB/PSI/PBR/another new indicator.

For LCG, which consist of multiple similar LCHs. They may share the same additional priority. In this case, the LCG can be configured with one additional priority and the LCHs in it can use this priority to save some signaling overheads.

Proposal 1b: RAN2 confirms the baseline is that additional priority is configured per LCH, and discusses whether it can be per LCG if multiple LCHs from an LCG has similar delay requirement.

Moreover, another issue related to LCP configuration is the maximum number of additional priorities that can be configured to a single LCH. Allowing more additional priorities could provide more refined LCP procedures for delay-critical data with different remaining times and increase the likelihood that data with less remaining time will be prioritized for faster transmission.

Considering delay-critical data have different remaining time, multiple additional priorities enable differentiated handling of delay-critical data based on remaining PDBs, for example, High/Medium/Low priorities corresponding to remaining PDB thresholds of 5ms/10ms/20ms.

Observation 1a: With multiple additional priorities, delay-critical data with different remaining time can be handled with different priority.

Another advantage of multiple priorities is that, by adjusting the priority based on the ratio of non-delay-critical data to delay-critical data in the current LCP, it is possible to achieve a balance between efficiency and fairness. For example, UE MAC entity may choose one additional priority from multiple priorities based on the ratio of delay-critical to non-critical data within an LCH. A higher priority assigned if ≥80% of MAC PDU data is delay-critical (e.g., XR video frame) and a lower priority assigned if ≤20% is delay-critical (e.g., background metadata).

Observation 1b: It would be beneficial for efficiency and fairness if MAC entity could choose a proper priority based on the ratio of delay-critical data and non-delay-critical data.

Supporting multiple additional priorities is generally a suitable approach. However, this comes with the potential cost of increased complexity in the implementation on both the UE and gNB sides. And to reduce the complexity, up to 3 additional priority is a good compromise between complexity and flexibility.

Proposal 2a: For fairness and efficiency, RAN2 agrees that multiple (up to 3) additional priorities can be configured for a single LCH,

Proposal 2b: UE can select one additional priority from multiple priorities based on minimal remaining time among PDUs and/or the ratio of delay-critical and non-delay-critical data.

For multiple additional priorities, chances are that some UEs may only support 2 or less of them, therefore UE to report the maximum priority is necessary.

Proposal 2c: The maximum number of additional priorities per LCH can be signalled via UE capability

If UE indicates zero, UE may indicate that it’s not capable of delay-aware LCP.

  - Improvement on existing LCP procedure

For XR traffic with heterogenous delay budgets (e.g., 5ms for haptic feedback and 20ms for video frame), using a same additional priority can be sufficient. But when it comes to Bj and PBR, due to the difference in their data size, a constant PBR and Bj may face some challenges, for example, if haptic feedback and video frame are generated at the same time, the PBR may be insufficient cause some delay.

Figure 1. An example of PBR impact on XR traffic with video and other data with high load

To mitigate potential issues caused by excessive use of additional priorities and make sure that delay-critical data has sufficient token, it is advisable to have configurations that ensure data utilizing these priorities does not grow excessively, thereby avoiding impacts on other logical channels or UEs.

For instance, a new delay-aware PBR could be introduced per LCH and there will be delay-aware Bj for LCH j respectively. Delay-critical data and non-delay-critical data can utilize separated PBR and Bj.

In this approach, if the gNB observes changes in the DSRs reported from UEs, for example, more UEs report DSR or an increasing volume of delay-critical data in the DSR — it might indicate that gNB should assign more PBR for delay-critical data or non-delay-critical data has incorrectly utilized additional priority.

The gNB can determine whether excessive non-delay-critical data has improperly used additional priority based on its implementation. The specific mechanism for this verification can be left to the gNB's implementation discretion, allowing flexibility for different network requirements and scenarios.

The proposed dedicated PBR for delay-critical data could be referred to as a delay-critical PBR. Introducing such a delay-critical PBR, independent of the existing PBR, provides greater flexibility. The gNB's RRC layer can dynamically manage both delay-aware PBR and the legacy or non-delay-critical PBR. This capability enables the gNB to effectively control delay-critical data and limit the impact of non-delay-critical data utilizing additional priority on other users and logical channels.

Proposal 3: Introduce dedicated delay-critical PBR and Bj for delay-critical data while the legacy PBR and Bj can be reused for non-delay-critical data.

The details, such as whether redundant and dynamic PBR can be reused, can be further discussed.

  - Improvement on intra-UE prioritization

The issue on intra-UE prioritization was not discussed in the last meeting, but it is closely related to the prioritization of uplink grants, particularly in scenarios involving overlapping grants and the prioritization between scheduling requests and overlapping grants.

Although the case of overlapping uplink grants and SRs can be relatively uncommon, it may still have implications, especially when the additional priority configuration for certain LCHs is set too high. Such a situation could inadvertently affect LCHs containing SRB data, potentially leading to undesirable outcomes, such as delays in the transmission of critical RRC messages.

Whether additional priority can be effectively applied to intra-UE prioritization depends primarily on its impact on the LCHs containing SRB data. If additional priority configurations result in negative effects on SRB data, particularly for high-priority SRBs, it would pose significant challenges. However, RAN2 has not reached a consensus on this issue.

Proposal 4: The additional priority applies to intra-UE prioritization only if it will not affect SRB.

A potential solution could involve gNB-level implementation mechanisms to ensure that SRB data remains unaffected during intra-UE prioritization. If such safeguards can be established, applying additional priority to intra-UE prioritization becomes a reasonable approach. For lower-priority SRBs, such as SRB5, this issue warrants further discussion, as the potential impact and mitigation strategies may differ.

Conclusion

This contribution discusses some remaining issues on XR LCP enhancement, and propose the following,

Configurations:

Proposal 1a: RAN is responsible for configuration of delay-aware LCP based on CN information, e.g., existence of PDB/PSDB/PSI/PBR/another new indicator.

Proposal 1b: RAN2 confirms the baseline is that additional priority is configured per LCH, and discusses whether it can be per LCG if multiple LCHs from an LCG has similar delay requirement.

Observation 1a: With multiple additional priorities, delay-critical data with different remaining time can be handled with different priority.

Observation 1b: It would be beneficial for efficiency and fairness if MAC entity could choose a proper priority based on the ratio of delay-critical data and non-delay-critical data.

Proposal 2a: For fairness and efficiency, RAN2 agrees that multiple (up to 3) additional priorities can be configured for a single LCH,

Proposal 2b: UE can select one additional priority from multiple priorities based on minimal remaining time among PDUs and/or the ratio of delay-critical and non-delay-critical data.

Proposal 2c: The maximum number of additional priorities per LCH can be signalled via UE capability

PBR and Bj enhancement:

Proposal 3: Introduce dedicated delay-critical PBR and Bj for delay-critical data while the legacy PBR and Bj can be reused for non-delay-critical data.

Intra-UE prioritization:

Proposal 4: The additional priority applies to intra-UE prioritization only if it will not affect SRB.

References

[1]	R2-2410915, Report from session on R18 MBS, R18 QoE and R19 XR, Session chair (Huawei)