3GPP TSG-RAN2 Meeting #129			R2-2501016

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.7.6

Title:	Consideration on rate control enhancement for XR

Source:	CMCC

Document for:	Decision

Introduction

In the last meeting [1], RAN2 has the following agreements,

Agreements on XR rate control - RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level. - gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN - Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB. - RAN2 will not discuss/support rate indication for DL unless WID is updated to include it by RANP. - RAN2 assumes that the congestion situation can be known at the gNB without any indication from the UE - FFS whether UL MAC CE rate query/preference is supported as UE recommendation to the NW or whether legacy MAC CE can serve this already. FFS in which scenarios this is useful.

This contribution will discuss some remaining issues on rate control enhancement for XR and propose some solutions for that.

Discussion

  - Rate control indication

Based on discussions from the previous meeting, the majority view is that controlling the rate on a per QoS flow basis allows for more fine-grained adaptation to the highly variable XR traffic and channel conditions, and such an approach is indeed feasible.

However, indicating QoS flow rate control information in the MAC CE conflicts with our long-established design principle of maintaining layer independence. In particular, the MAC scheduler, which operates at the granularity of LCH (or DRB), lacks the necessary information and configuration of individual QoS flows. Therefore, embedding QoS flow details in the MAC CE is not appropriate. If a per QoS flow indication is to be pursued, such indication should be conveyed at a higher layer, e.g., via RRC messages or other upper-layer signaling.

The advantage of per QoS flow control lies in its finer granularity and per QoS flow is more compatible with L4S/ECN and other application layer mechanisms. The per QoS flow rate control is more aligned with “long-term” control, e.g., when a gNB, faced with an excessive number of UEs and limited radio resources, needs to reduce rates without relying on PDCP discarding caused by excessive queuing, where control the source rate to manage both throughput and congestion can be utilized. An important benefit of this approach is that, unlike traditional rate or congestion control mechanisms that rely on packet loss ratio as a trigger or threshold, a congestion indication allows the network to notify the source or application to adjust the rate before actual packet loss occurs.

In contrast, “short-term” rate or congestion control is more likely to be implemented on a per-DRB/LCH basis because the MAC scheduler allocates resources at the LCH/DRB granularity and can directly control the rate. This makes it particularly effective in handling sudden, transient demands for rate reduction, e.g., those resulting from short deteriorations in channel conditions.

Thus, based on the analysis above, it can be concluded that per QoS flow and per DRB rate control are not mutually exclusive alternatives; rather, each has its own advantages. Specifically, per QoS flow rate control is better suited for long-term (source/application-based) rate management, whereas per DRB rate control is more appropriate for short-term (scheduler-based) adjustments.

Observation 1: QoS flow level rate control and DRB level rate control serve complementary purposes, QoS flow level is optimal for long-term source rate control and DRB level is effective for short-term scheduling adaptation.

Proposal 2: RAN2 agrees to support both DRB level rate control in MAC CE and QoS flow level rate control in RRC message.

If agreed, RAN2 may discuss simplified approaches in the future. The proposed solution maintains layer independence while enabling cross-layer optimization through standardized interfaces, i.e., MAC layer mechanisms handle time-critical adjustments, while RRC/QoS-level controls enable end-to-end service optimization.

  - Bit value Indication

Although the issue of bit value indication awaits specific reply LS from SA4, it is considered necessary to introduce a mechanism for rapid rate adjustment. This mechanism can be imprecise yet fast, e.g., employing a traditional TCP-like multiplicative decrease and additive increase approach. Such a method would help quickly reduce the rate in RAN overload and fast increase it when resources become available, e.g., 0.5x or 2x.

This can be implemented by having the gNB signal a maximum or minimum rate. Importantly, this approach is not affected by SA4 feedback since the indicated rate represents only the capacity supported by the gNB and radio resource, leaving UEs and higher layers free to adjust the rate based on their own requirements.

Proposal 3: RAN2 agrees to capture optional and simple indication (e.g., 0.5x or 2x) without bit value in rate control MAC CE to decrease or increase rate in for fast rate control.

The precise rate value or step can be indicated later.

  - Rate control query

Given that ECN/L4S mechanisms implement congestion control at the transport and application layers, the primary motivation for introducing RAN-side rate control in XR is to address the inability of both the source and sink to directly observe changes in the radio environment and resource availability. Fundamentally, the necessity for a query process is not very strong, as the gNB can infer congestion through BSR, DSR reported by UE and resource allocation status.

Moreover, a query process generally involves determining an appropriate value for a variable. For static information, e.g., whether a DRB or QoS flow supports rate control, this is more akin to stable static capability information. From this standpoint, the main function of a query process should be to retrieve a rate limit or a long-term stable rate from the gNB, and to support fast recovery after congestion or to obtain a rate that can still be maintained during congestion.

However, the gNB can directly signal a rate control MAC CE to facilitate fast recovery or sustain transmission rate during congestion.

Overall, the specific purpose and scenarios for such a query mechanism remain unclear, and RAN2 may discuss whether any new use cases warrant its introduction.

Observation 2a: There is no strong need for UE to report UL rate control query MAC CE to indicate which DRB or QoS flow supports rate control, they can be specified in UE capability or UAI.

Observation 2b: UL rate control query may be useful for fast recovery after congestion or query a sustain data rate even in congestion, but it can also be done by gNB sending rate control MAC CE.

Proposal 4: RAN2 recommends against standardizing UL rate control query MAC CE until concrete use cases emerge.

  - Assistance information

Considering the performance and version differences among UE video/audio codecs, which may only be able to switch among a limited number of fixed rate levels, and the possibility that the gNB might not receive rate control capability information for a DRB or QoS flow from the CN (especially in AR services), it would be beneficial if the UE could obtain and report such information from the application layer or other sources.

Optionally reporting this assistance information via the UAI may prove advantageous, particularly when the UE’s codec supports only a few fixed encoding rate levels, making excessive rate allocation wasteful.

Proposal 5: RAN2 agrees that it’s beneficial if UE can gather and report assistance information to gNB about which DRBs or QoS flows are capable of rate control and/or the fixed code rate levels support by UE codec, if exists.

It may have some SA4 impacts, which can be check in the reply LS from SA4.

Conclusion

This contribution discusses the remaining issue on rate control for XR service and propose the following,

Granularity:

Observation 1: QoS flow level rate control and DRB level rate control serve complementary purposes, QoS flow level is optimal for long-term source rate control and DRB level is effective for short-term scheduling adaptation.

Proposal 2: RAN2 agrees to support both DRB level rate control in MAC CE and QoS flow level rate control in RRC message.

Fast rate control:

Proposal 3: RAN2 agrees to capture optional and simple indication (e.g., 0.5x or 2x) without bit value in rate control MAC CE to decrease or increase rate in for fast rate control.

Rate control query:

Observation 2a: There is no strong need for UE to report UL rate control query MAC CE to indicate which DRB or QoS flow supports rate control, they can be specified in UE capability or UAI.

Observation 2b: UL rate control query may be useful for fast recovery after congestion or query a sustain data rate even in congestion, but it can also be done by gNB sending rate control MAC CE.

Proposal 4: RAN2 recommends against standardizing UL rate control query MAC CE until concrete use cases emerge.

Assistance infomation:

Proposal 5: RAN2 agrees that it’s beneficial if UE can gather and report assistance information to gNB about which DRBs or QoS flows are capable of rate control and/or the fixed code rate levels support by UE codec, if exists.

References

[1]	R2-2410915, Report from session on R18 MBS, R18 QoE and R19 XR, Session chair (Huawei)