3GPP TSG-RAN WG2 Meeting #129	R2-2501018

Athens, Greece, Feb. 17th – 21st, 2025

# Agenda item:	8.4.4

# Source:	CMCC

# Title:	Discussion on LP-WUS operation in CONNECTED mode

# Document for:	Discussion, Decision

- Introduction

In the previous meetings, RAN2 discussed LP-WUS operations in RRC_CONNECTED mode, and mainly focused on Option1-1 and Option1-2, following is the related agreements:

RAN2#126 meeting agreements [1]:

In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1. For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1. For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE. The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

RAN2#127 meeting agreements [2]:

For option 1-2, - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.) - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1.

RAN2#127bis meeting agreements [3]:

For Option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2. In option 1-2, a new timer triggered by LPWUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LPWUS monitoring. When UE is in C-DRX active time, UE PDCCH monitoring behaviors related to other legacy DRX timers (except for drx-onDurationTimer) are not affected.

RAN2#128 meeting agreements [4]:

The drx-onDurationTimer is not started with Option 1-2 LP-WUS. For Option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer, for the case when UE is outside C-DRX active time. For option 1-2, if UE receives DRX command MAC CE or Long DRX command MAC CE, UE stops the new timer triggered by LP-WUS. Don’t support Option 1-1 and Option 1-2 simultaneously configured for the same UE.

In this contribution, we will discuss the potential RAN2 impacts with LP-WUS/WUR in RRC_CONNECTED state.

- Discussion

  - LP-WUS use option

Option 1-1

In RAN2#126 meeting, RAN2 reached the consensus on Option 1-1 that the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer, and RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. In this section, we will discuss the detail configuration and restrictions of Option 1-1 taking DCP as reference.

Following is the DCP configuration specified in TS 38.331[4]:

DCP-Config-r16 ::= SEQUENCE { ps-RNTI-r16 RNTI-Value, ps-Offset-r16 INTEGER (1..120), sizeDCI-2-6-r16 INTEGER (1..maxDCI-2-6-Size-r16), ps-PositionDCI-2-6-r16 INTEGER (0..maxDCI-2-6-Size-1-r16), ps-WakeUp-r16 ENUMERATED {true} OPTIONAL, -- Need S ps-TransmitPeriodicL1-RSRP-r16 ENUMERATED {true} OPTIONAL, -- Need S ps-TransmitOtherPeriodicCSI-r16 ENUMERATED {true} OPTIONAL -- Need S }

ps-RNTI is the RNTI value for scrambling CRC of DCI format 2-6, while for LP-WUS, UE doesn’t monitor related RNTI but a LP sequence, therefore, RNTI is not needed. As to the LP sequence, whether it should be defined in Option 1-1 specific configuration as DCP can be decided by RAN1.

ps-Offset is used to indicated the UE the LP-WUS monitoring occasion before start of  the drx-onDurationTimer, it’s also needed for Option 1-1.

ps-WakeUp indicates UE’s default behaviour when if DCP is not detected outside active time, similar mechanism can also be applied for Option 1-1.

ps-PositionDCI-2-6 indicates UE’s starting position in DCI 2-6, and when Option1-1 is used, the UE needs to know the starting position in the LP sequence too.

As to PeriodciL1-RSRP and ps-TranmsitOtherPeriodcCSI, they are used to indicate the UE’s default behaviour on periodic L1-RSRP and CSI reporting when the drx-onDurationTimer is not started. And RAN1 has reached the following agreements on periodic CSI/L1-RSRP reporting, same as Rel-16 DCP mechanism.

Agreement For Option 1-1 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. - LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer - UE is configured with legacy C-DRX configurations as Rel-18 - UE behaviors related to CDRX active time triggered by all legacy DRX timers are not affected unless included in this proposal - No impact on RRM/RLM/BFD measurement requirements is assumed - For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP) - Periodic CSI/L1-RSRP is not reported during the time given by the configured drx-onDurationTimer if UE is not indicated to wake-up - Periodic CSI/L1-RSRP is periodically reported during the time given by the configured drx-onDurationTimer regardless if UE is indicated to wake-up or not - Note: Periodic CSI/L1-RSRP is reported during CDRX active time as in legacy specification

Proposal 1: Configuration of Option 1-1 can include the following contents:

- Offset to indicate the UE the LP-WUS monitoring occasion before start of  the drx-onDurationTimer;

- UE’s starting position in the LP sequence;

- UE’s default behaviour when LP-WUS is not detected outside active time;

- UE’s default behaviour on periodic L1-RSRP and CSI reporting when the drx-onDurationTimer is not started.

Proposal 2: Whether LP-WUS related LS sequence configuration is included in Option 1-1 specific configuration is decided in RAN1.

Option 1-2

In RAN2#127 meeting RAN2 discussed Option 1-2, and achieved the basic understanding of Option 1-2 that: After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring.  And in last RAN2 meeting, it was agreed that for Option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time, and a new timer triggered by LPWUS is introduced. But whether the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2 is under discussion.

Following is the original description of Option 1-2 agreed in RAN1:

Agreement in RAN1#116bis meeting: Update the following agreement in RAN1#116 in red: Agreement in RAN1#116 meeting: - For RRC CONNECTED mode, from RAN1 perspective, further study following LP-WUS procedures to trigger PDCCH monitoring: - Case 1: PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - This option may replace DCP functionality - Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - PDCCH monitoring possibly irrespective of drx-onDurationTimer - Option 1-2-1: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - If this is adopted, it should be configured together with Option 1-1 to achieve power saving gain compared to legacy C-DRX - Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - Case 2: PDCCH monitoring is triggered by LP-WUS without C-DRX configuration. LP-WUS can be monitored at any time according to the LP-WUS monitoring configuration - FFS duty-cycled and/or continuous LP-WUS monitoring - Combination of options in Case 1 and combination of options in Case 1 and Case 2 are not precluded should be considered. - RAN1 does not discuss C-DRX related timers other than drx-onDurationTimer, this topic is up to RAN2 - Note: Above does not preclude to support fallback mechanism to trigger PDCCH monitoring, if any

As highlighted, all options discussed currently are configured with C-DRX.

Observation 1: Option 1-2 is configured with C-DRX.

In our understanding, for Option1-2-1, which should be configured with Option 1-1, drx-onDurationTimer is started based on Option 1-1 LP-WUS, irrespective whether a new timer is started by Option 1-2 or not. However, based on current RAN2 agreements, the drx-onDurationTimer is not started with Option 1-2 LP-WUS, and Option 1-1 and Option 1-2 cannot be simultaneously configured for the same UE,  this is not aligned with RAN1 agreements. In some sense, Option 1-2-1 is excluded in RAN2. Therefore, we think RAN2 should inform RAN1 about this.

Proposal 3: RAN2 should send LS to RAN1/4 that Option1-2-1 is excluded to avoid unnecessary discussion in RAN1/4.

Common issues of Option 1-1 and Option 1-2

DCP is only configured on the PCell when CA is configured. For LP-WUS operations with CA, it is discussed in RAN1, and following is the agreements made in RAN1#118 meeting [5]:

RAN1#118 Agreements: Agreement LP-WUS is at least supported for the case where a UE is configured with CA in RRC CONNECTED mode - FFS: DC

Based on RAN1 agreements, LP-WUS operation for RRC_CONNECTED UE with CA is supported. Currently, when CA is configured, DCP is only configured on the PCell, the same principle can be applied to LP-WUS operation.

As to DC case, when the UE is configured with DC, it may have one or two DRX group. If one of the node is FR1, but the other is FR2, there’re two DRX groups. Therefore, for the case UE has only one DRX group, LP-WUS operation can be configured on PCell of MN/MCG; for the case UE has two DRX groups, LP-WUS operation can be configured on SpCells.

Proposal 4：LP-WUS operation for UE configured with CA is supported. When CA is configured, LP-WUS operation is only configured on the PCell.

Proposal 5:  For the case UE has only one DRX group, LP-WUS operation can be configured on PCell of MN/MCG; for the case UE has two DRX groups, LP-WUS operation can be configured on SpCells.

Following is the LS [6]  from RAN1 to RAN2 about the use of DRX Command MAC CE and Long DRX Command MAC CE with Option 1-1 and Option 1-2:

In RAN#119, RAN1 made following conclusion and agreement to use existing L2 signalling to stop C-DRX related timers and resuming LP-WUS monitoring: Conclusion - For Option 1-1 of LP-WUS CONNECTED mode operation, RAN1 assumes existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and drx-InactivityTimer for the DRX group - When UE is not in C-DRX active time, UE monitors LP-WUS - Send LS to RAN2 to ask whether RAN2 has same understanding or not. LS in R1-2410909. Agreement - For Option 1-2 of LP-WUS CONNECTED mode operation, for explicit indication to stop the new timer for PDCCH monitoring in Option 1-2 and drx-InactivityTimer for the DRX group, RAN1 identified the following potential solution - Use existing DRX Command MAC CE and Long DRX Command MAC CE - When UE is not in C-DRX active time, UE monitors LP-WUS - Send LS to RAN2 to ask the feasibility of the above potential solution. LS in R1-2410909.. Regarding the above conclusion on Option 1-1, RAN1 would like to ask RAN2 whether RAN2 has same understanding or not. Regarding the above agreement on Option 1-2, RAN1 would like to ask RAN2 the feasibility of the above potential solution. 2. Actions: To RAN WG2 ACTION: RAN1 respectfully asks RAN2 to reply to the questions above.

Generally, no matter which option is used, LP-WUS is monitored when UE is not in C-DRX active time, and the difference for the two options is the timer started by LP-WUS. And in RAN2#128 meeting, RAN2 agreed that for option 1-2, if UE receives DRX command MAC CE or Long DRX command MAC CE, UE stops the new timer triggered by LP-WUS. For Option 1-1, since it has less impact to current C-DRX mechanism than Option 1-2, we think there’s no technique reason to define different UE behaviour when it receives DRX command MAC CE or Long DRX command MAC CE. Therefore, for Option1-1, existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and drx-InactivityTimer for the DRX group, and RAN2 applies RAN1’s LS to confirm the feasibility.

Proposal 6: For Option1-1, existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and drx-InactivityTimer for the DRX group. And RAN2 confirms RAN1’s understanding about this.

Option 1-3

Both Option 1-1 and Option 1-2 are used to trigger the MR into active time for PDCCH monitoring,  which means that with only Option 1-1 or Option 1-2, once MR is activated, the UE need to always monitor all PDCCH occasions. While Option 1-3 works inside UE’s active time, with LP-WUS indication, UE can skip some PDCCH occasions monitoring for further power saving. Based on the discussion in SI phase, Option 1-3 main focuses on XR service, which the service is periodic, but there is jitter for each packet, to reduce the effects introduced by jitter, the C-DRX on-duration may be configured to cover the jitters. But it results in UE’s extra power assumption for PDCCU monitoring. With Option 1-3, UE can start PDCCH monitoring when the traffic arrives and achieve energy saving.

Figure 3: Example for Option 1-3

Based on RAN1’s definition/description of Option 1-3, LP-WUS indication is received in UE’s active time. Currently, the active time includes the following cases TS 38.321 [7]:

When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while: - drx-onDurationTimer or drx-InactivityTimer configured for the DRX group is running; or - drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL is running on any Serving Cell in the DRX group; or - ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or - a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5). If this Serving Cell is part of a non-terrestrial network, the Active Time is started after the Scheduling Request transmission that is performed when the SR_COUNTER is 0 for all the SR configurations with pending SR(s) plus the UE-gNB RTT; or - a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a); or - there is an ongoing RACH-less LTM cell switch; or - there is an ongoing RACH-less handover in a terrestrial network.

It’s not clear whether Option 1-3 LP-WUS indication is applied to the timers mentioned above. In our understanding, since Option 1-3 is to solve the jitter issue of XR service, and C-DRX cycle  and on-duration configuration has taken XR service periodicity and jitter into account, Option 1-3 applied to drx-onDurationTimer is sufficient.

Proposal 7: UE only monitors the Option 1-3 LP-WUS indication within the legacy drx-onDurationTimer duration.

Besides, considering that Option 1-3 and Option 1-1/1-2 focuses on different phases of PDCCH monitoring, therefore, If Option 1-3 is supported, it can be configured with Option 1-1 or Option 1-2.

Proposal 8: If Option 1-3 is supported, it can be configured together with Option 1-1 or Option 1-2.

Support of LP-WUS use options

Following is the working assumption made in RAN1#118 meeting:

Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Note: Above can be revisited considering RAN2 decisions.

As to Option 1-3, though it can be used with Option 1-1 or Option 1-2, but the power saving gain is not as large as the other two options.  Considering RAN1’s working assumption, it can be deprioritized.

Proposal 9: RAN2 confirms RAN1’s working assumption that Option 1-3 is deprioritized for RRC_CONNECTED UE.

  - Configuration and (de)activation of LP-WUS monitoring

In RAN1#116 meeting, gNB RRC signalling based LP-WUS enable/disable is agreed and whether additional options on top of it is for further study.

Agreement For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance. - LP-WUS monitoring by UE is known to gNB. - FFS whether implicit/explicit indication from UE is necessary - In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

Considering the motivation of using LP-WUS in CONNECTED mode is to reduce the PDCCH monitoring power consumption while guarantee the latency performance, it is not necessary to adapt the on/off of LP-WUS monitoring frequently based on fast L1/L2 enable/disable signalling. Besides, UE assistance information can be introduced to assist gNB to enable/disable the LP-WUS monitoring. For example, when a UE is in lower battery status, UE can report to gNB that it prefers to activate the LP-WUS monitoring function. As to the condition based activation/deactivation mechanism, similar performance can be achieved by proper network configuration based on UE reported information, including UE assistant information and SR/BSR. Therefore, in our understanding, Option 3 and Option 4 can be achieved by network implementation.

Proposal 10: RRC signalling based approach is used to enable LP-WUS monitoring in RRC_CONNECTED.

Proposal 11: Once the UE receives LP-WUS monitoring configuration, it start LP-WUS monitoring based on the configuration.

Proposal 12: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring. LP-WUS monitoring is only enabled(activated) or disable(deactivated) by RRC signalling.

Proposal 13: UE assistance information can be introduced to assist gNB to enable/disable the LP-WUS monitoring.

  - Co-existence with other power saving mechanisms

Co-existence with DCP

In last meeting, RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE, while Option 1-2 and Option 1-3 are more flexible to trigger PDCCH monitoring in drx off period or within DRX active time, therefore, there’s no need to configure DCP and LP-WUS for RRC_CONNECTED PDCCH monitoring simultaneously.

Proposal 14: There’s no need to configure or activate DCP and any LP-WUS options for RRC_CONNECTED PDCCH monitoring simultaneously.

Co-existence with PDCCH monitoring skipping

For PDCCH skipping/SSSG switching, we think it can be combined with LP-WUS options on the table. For example, PDCCH skipping can be used as the indication of stop of PDCCH monitoring as in Figure 1. That is if WUR detects the LP-WUS, MR starts to monitor PDCCH until receives the PDCCH skipping indication.

Figure 5. LP-WUS coexistence with PDCCH skipping

Proposal 15: LP-WUS procedure and Rel-17 PDCCH skipping/SSSG switching can be configured or activated to one UE simultaneously.

- Conclusions

In this contribution, we discuss the LP-WUS operation in RRC_CONNECTED mode, and the following proposals are made.

Proposals:

Proposal 1: Configuration of Option 1-1 can include the following contents:

- Offset to indicate the UE the LP-WUS monitoring occasion before start of  the drx-onDurationTimer;

- UE’s starting position in the LP sequence;

- UE’s default behaviour when LP-WUS is not detected outside active time;

- UE’s default behaviour on periodic L1-RSRP and CSI reporting when the drx-onDurationTimer is not started.

Proposal 2: Whether LP-WUS related LS sequence configuration is included in Option 1-1 specific configuration is decided in RAN1.

Proposal 3: RAN2 should send LS to RAN1/4 that Option1-2-1 is excluded to avoid unnecessary discussion in RAN1/4.

Proposal 4：LP-WUS operation for UE configured with CA is supported. When CA is configured, LP-WUS operation is only configured on the PCell.

Proposal 5:  For the case UE has only one DRX group, LP-WUS operation can be configured on PCell of MN/MCG; for the case UE has two DRX groups, LP-WUS operation can be configured on SpCells.

Proposal 6: For Option1-1, existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and drx-InactivityTimer for the DRX group.

Proposal 7: UE only monitors the Option 1-3 LP-WUS indication within the legacy drx-onDurationTimer duration.

Proposal 8: If Option 1-3 is supported, it can be configured together with Option 1-1 or Option 1-2.

Proposal 9: RAN2 confirms RAN1’s working assumption that Option 1-3 is deprioritized for RRC_CONNECTED UE.

Proposal 10: RRC signalling based approach is used to enable LP-WUS monitoring in RRC_CONNECTED.

Proposal 11: Once the UE receives LP-WUS monitoring configuration, it start LP-WUS monitoring based on the configuration.

Proposal 12: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring. LP-WUS monitoring is only enabled(activated) or disable(deactivated) by RRC signalling.

Proposal 13: UE assistance information can be introduced to assist gNB to enable/disable the LP-WUS monitoring.

Proposal 14: There’s no need to configure or activate DCP and any LP-WUS options for RRC_CONNECTED PDCCH monitoring simultaneously.

Proposal 15: LP-WUS procedure and Rel-17 PDCCH skipping/SSSG switching can be configured or activated to one UE simultaneously.

Observation:

Observation 1: Option 1-2 is configured with C-DRX.

- References

- Chair notes RAN2#126;

- Chair notes RAN2#127;

- Chair notes RAN2#127bis;

- Chair notes RAN2#128;

- Chair notes RAN1#118;

- R1-2410909; LS on LP-WUS operation in CONNECTED mode;

- TS 38.321; Medium Access Control (MAC) protocol specification; (Release 18).