# 3GPP TSG-RAN WG2 Meeting #129                                           R2-2501031

Athens, Greece, Feb. 17th – 21st, 2025

Source: 			CMCC

Title:  		    Discussion on LCM for positioning

Agenda item:	8.1.2.3

Document for: 	Discussion

Introduction

With the RAN2#127bis and 128 meeting, RAN2 discussed general procedures for LCM for UE sided model for AI positioning case 1. The following agreements have been reached.

#127BIS 1: Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress). 2: Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress). 3: FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively). FFS the signalling details. 4: RAN2 will decide whether AI positioning will be a new method after further details from RAN1 are received. #128 1 For POS Case 1, RAN2 confirm that the existing unsolicited UE capability report mechanism in LPP can support UE to report the applicable functionality in both “proactive” and “reactive” as a baseline. - Proactive case: When the applicability change, UE can send an unsolicited LPP ProvideCapabilities message to LMF . - Reactive case: If the applicability changes based on the configuration in LPP ProvideAssistanceData message in step 3, UE can send an unsolicited LPP ProvideCapabilities message to LMF. Configuration details are FFS 2 As a baseline, If the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP Providelocationinformation message with error cause. FFS if other fallback options are considered

Within the post email discussion [026], some leftover issues have been further discussed. In the contribution, we will discuss the controversial issues.

Discussion

AI/ML positioning as a new method

Such an issue has been captured in the post email discussion. As some companies have pointed out that for both the legacy positioning and AI/ML Positioning, the signals and information are the same. The key difference lies in the specific measurement results utilized and whether the positioning outcome depends on AI/ML predictions. However, the AI/ML based positioning may introduce new assistance of data compared with legacy positioning methods. Given this, we prefer to introduce AI/ML positioning as an entirely new method to ensure forward compatibility. This approach will not only better adapt to future technological evolution but also provide more room for the further development of AI/ML positioning technologies.

Proposal 1: Introduce AI/ML positioning as a new method.

Fallback configuration

RAN2 has agreed that if the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning and reply with LPP ProvideLocationInformation message with error cause. We understand that the details on error cause can be discussed in stage3. Regarding fallback from AIML based positing to legacy positioning, we do not think any pre-configuration about fallback is needed. The fallback should be handled via LPP reconfiguration under network control, instead of UE autonomously fallback according to pre-configuration.

Proposal 2: The fallback should be handled via LPP reconfiguration under network control, instead of UE autonomously fallback according to pre-configuration.

RAN1 progress impacts

With RAN1#116bis and RAN1#117, we have the following work assumptions and agreement:

Work assumptions: For training data generation of AI/ML based positioning Case 1/2a/2b/3b, the label and its related data (e.g., time stamp) can be generated by:

- PRU

- Non-PRU UE with estimated location

- LMF

Note: transfer of label and its related data is out of RAN1 scope.

For the case that the ground truth label is generated by LMF, LMF generates the ground truth label based on some measurement transmitted by UE and TRP’s location coordinate. LMF provides the corresponding ground truth label and its related data (e.g. time stamp) to UE for model training. The existing signaling ProvideAssistanceData can be reused.

Proposal 3: At least ground truth label and its related data (e.g. time stamp) should be provided by LMF via signaling ProvideAssistanceData.

As per RAN1 progress in last meeting, for both UE initiated and LMF initiated case, the DL PRS configuration and existing assistance data IE of UE-based DL-TDOA and/or UE-based DL-AoD are needed to be provided by LMF to UE.

For training data collection of Case 1, in terms of DL PRS configuration for collecting training data, both options are feasible by using legacy mechanisms: Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. Option B. (LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE. Note: the UE can be a PRU and/or a Non-PRU UE. Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

For AI/ML positioning Case 1, regarding the assistance data provided from LMF to UE, for ensuring consistency between training and inference, - for each of the existing assistance data IE of UE-based DL-TDOA and/or UE-based DL-AoD, study whether it should be: (a) explicitly indicated, (b) implicitly indicated and/or (c) other; - Companies can provide inputs on further enhancements of existing assistance data, including new information - Note: this does not mean that training and inference phases are mentioned in assistance data. Table. Existing assistance data (supported up to Rel-18) that may be transferred from LMF to UE in UE-based DL-TDOA [1] or UE-based DL-AoD [2], as applicable.

Proposal 4: For data collection for UE sided POS model training, at least DL PRS configuration, existing assistance data IE of UE-based DL-TDOA and/or UE-based DL-AoD need to be provided by LMF to UE.

Conclusion

According to the above discussion, the following observations and proposals are given:

Proposal 1: Introduce AI/ML positioning as a new method.

Proposal 2: The fallback should be handled via LPP reconfiguration under network control, instead of UE autonomously fallback according to pre-configuration.

Proposal 3: At least ground truth label and its related data (e.g. time stamp) should be provided by LMF via signaling ProvideAssistanceData.

Proposal 4: For data collection for UE sided POS model training, at least DL PRS configuration, existing assistance data IE of UE-based DL-TDOA and/or UE-based DL-AoD need to be provided by LMF to UE.

Reference

[1] RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR

[2] [POST128][026][AIML] LCM Procedure for Positioning Case1 (Ericsson)