3GPP TSG-RAN WG2 Meeting #129	R2-2501041

Athens, GR, Feb 17th – 21th, 2025

Agenda Item:	8.3.3

Source:	CMCC

Title:	Simulation results for Measurement event predictions

WID/SID:				FS_NR_AIML_Mob

Document for:	Discussion

# 1 Introduction

In the previous RAN2 meetings, there were discussion on Simulation scenario/parameters, HO procedure simulation and System level simulation for Measurement event predictions. The agreements about simulation assumptions for Measurement event predictions were achieved to start the relevant simulations.

RAN2#125bis [1]:

Agreements:

- At least measurement event evaluation based on RRM measurement prediction result will be studied.   Direct measurement event prediction are is also allowed.

- Clarifications on what is being as input should be provided with results

- Start with A3 as a baseline.

- Measurement event prediction study can start after some further progress on RRM measurement prediction has been made

RAN2#127bis [2]:

Agreements

Measurement event prediction simulations will at least focus on intra-frequency FR2, case A, and second study goal (i.e. HO KPI improvement).   FFS what is KPI.

Companies can bring simulation results for intra-frequency measurement reduction for FR1 and report what they are doing.  Focus on temporal case B.

Companies will prioritize simulations on indirect method.   Companies can bring simulations on direct method and should report what method is being used.

Measurement event prediction results are expected in RAN2#129

Agreements on simulation assumptions

The Simulation assumption of RRM measurement prediction can be reused unless otherwise specified.

Companies can pick and report what they are using for filtering options (similar to RRM prediction)

Companies will focus on sub-case 2 for measurement event prediction.   Companies can simulate other sub-cases if they wish and report what they are using.

Leave the simulation parameter discussion for email discussion.  Pick only one value for A3.

It is up to company implementation how to model UE behavior after A3 event is trigger.  Focus on intermediate KPIs for this exercise.  System level KPI is FFFs

For measurement event prediction, traffic is not simulated.

Agreements on inputs/outputs and KPIs - For indirect measurement event prediction, the intermediate output (i.e., the output of RRM prediction model) is RSRP of serving/neighboring cells. The final output is the expected occurrence time of a certain measurement event (ex. event A3). - For direct measurement event prediction, the model output is the probability of event occurrence within a time window. - A3 event prediction should follow legacy rules (i.e. the “predicted” conditions have to persist for the duration of TTT). - As baseline, we will use RLF event prediction KPI: - for indirect: F1 score. the following can be reported: RSRP difference, missed event detection, false event detection. FFS how to define F1 score. - time difference of true time event reporting triggered and predicted time event reporting triggered, true event prediction. - for direct: F1 score. The following can be reported: missed event detection, false event detection, - Continue discussion over email discussion to see if there is a difference.

In this contribution, we provide our initial simulation results and observations of Indirect measurement event prediction for temporal domain case A.

# 2 Discussion

2.1 Simulation assumptions

2.1.1 Simulation scenario

In last RAN2#128 meeting [3], there were 4 definitions of indirect measurement event prediction agreed. In our simulation, we use Indirect measurement event prediction for temporal domain case A and RRM sub case 2 as starting point.

As per the agreed definition of Indirect measurement event prediction for temporal domain case A: In indirect measurement event prediction, future measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case A at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model.

And for RRM sub case 2, the measurements of L3 cell level RSRP are used as AI/ML model input to predict L3 cell level RSRP results.

Figure 1: RRM sub case 2

2.1.2 System level simulation

Regarding to System level simulation, there were two models (i.e. Option 2 and Option 3) agreed in RAN2#128 meeting for measurement event prediction for temporal domain case A. In our simulation, we use Option3 that HO preparation starts when an event is predicted to happen (i.e., t0), and HO command is sent when A3 entering conditions are met based on actual/real measurement and an event is predicted to be met for the duration of TTT.

Figure 2: Option3- Measurement event prediction for temporal domain case A [3]

2.1.3 L1/L3 filtering

During RAN2#127 meeting, RAN2 concluded that there are two L1/L3 filtering options that could be used in the simulations of RRM measurement prediction and up to company to report. In our simulation, we use sliding L1/L3 filtering option in temporal domain case A (measurement result are generated every 80ms).

Figure 3: Sliding L1/L3 filtering [4]

2.2 Simulation results Indirect measurement event prediction for temporal domain case A

In last RAN2#128 meeting, the baseline values were agreed for Indirect measurement event prediction for temporal domain case A. In our simulation, we set ETD (Event Timing Difference) to 80ms and OW length to 800ms. The key simulation assumptions and Report parameters are shown in Table 1 and Table 2.

Table 1: The key simulation assumptions in Case A

| Parameters | Value | Note |
| --- | --- | --- |
| A3 event offset (db) | 2 | Open for 3db |
| TTT (ms) | 320 | Open for one shorter value |
| UE speed (km/h) | 90 | Open for 60 and 120km/h |
| OW length (ms, note1) | 800 | Up to implementation |
| PW length (ms, note1) | 400 | Open for more values |
| ETD (ms, note1) | 80 | Open for more values |

Table 2: The Report parameters in Case A

| Report parameters |  | CMCC |
| --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3) | Option 1 |
|  | UE trajectory boundary processing option (option 1,2,3) | Option 2 |
|  | UE speed (30,60,90,120 Km/h) | 90 |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | N/A |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | N/A |
|  | Whether LOSsoft is modeled or not | Yes |
|  | spatial consistency option (A or B) | Procedure A |
|  | Number of TX beams | 32 |
|  | Number of RX beams | 4 |
|  | Measurement reduction rate(50%,…Note2) | N/A |
|  | Observation window(Note3) | 800ms |
|  | Prediction window (Note3) | 400ms |
|  | Any other parameters (Note 4) | N/A |
| Data Size (Number of Samples) | Training/validity | ≈194K/48K |
|  | Testing | ≈48K |
| AI/ML model input/output | Model input (Note 5) | Cell level RSRP of all cells |
|  | Model output(Note 6) | Cell level RSRP of all cells |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM |
|  | Model complexity in a number of parameters(M) | 5.6M |
|  | Model complexity in model size (e.g. Mbyte) | 64.14MB |
|  | Computational complexity [FLOPs] | 55.09M |

2.2.1 F1 score

In our simulation, the agreed definitions for true event prediction, false event detection and missed event detection in our simulation of indirect measurement event prediction were shown as below:

- Counter n3(true event prediction): it increases by 1 when a real event occurs around a predicted event with ETD, whose range is [0, 80ms] or vice versa;

- Counter n1(false event detection): it increases by 1 when no real event occurs around a predicted event with ETD, whose range is [0, 80ms];

- Counter n2(missed event detection): it increases by 1 when no event is predicted around a real event with ETD, whose range is [0, 80ms].

And they are used to calculate the KPIs for indirect measurement event prediction:

- Precision = n3/(n1+n3)

- Recall = n3/(n2+n3)

- F1 score = 2*Precision*Recall/(Precision + Recall)

The simulation results for Precision, Recall and F1 score of Indirect measurement event prediction for temporal domain case A is shown in Table 3.

Table 3: The simulation results of KPIs

| Precision | 0.9503 |
| --- | --- |
| Recall | 0.9472 |
| F1 score | 0.9535 |

For an AI model, the value of Precision is higher and close to 1, that means the prediction of measurement event occurs is more reliable. And the value of Recall is higher and close to 1, that means the probability of a missed measurement event is lower. And if the value of F1 score is close to 1, the better the comprehensive performance of the AI model in Precision and Recall these two aspects, and the prediction accuracy is better.

Based on the above simulation results, it could be observed that the values of Precision, Recall and F1 score all keep around 0.95 which is close to 1. Although the value of Recall is slightly lower (0.9472), F1 score is 0.9535 which is still above 0.95.

Observation 1: In our simulation of Indirect measurement event prediction for temporal domain case A, in the case that UE speed is 90 km/h and ETD is 80ms, the values of Precision is 0.9503, which is close to 1. That means for Indirect measurement event prediction for temporal domain case A, the prediction of measurement event occurs is reliable.

Observation 2: The values of Recall is slightly lower (0.9472), but it still around 0.95. That means for Indirect measurement event prediction for temporal domain case A, the probability of a missed measurement event is quite low.

Observation 3: The values of F1 score is 0.9535 which is close to 1. That means Indirect measurement event prediction for temporal domain case A could achieve high prediction accuracy.

2.2.2 System level simulation

In last RAN2#128 meeting, for System level simulation, it is agreed to reuse HO failure model and corresponding metrics i.e. HO failure rate, total number of HO attempts per UE per second from TS 36.839 [5]. In our System level simulation, we compare HoF rate of AI based with non-AI approach in the case that UE speed is 90 km/h and ETD is 80ms to evaluate the gain of HO performance which could be achieved by measurement event prediction. And the handover failures calculated in state 2 specified in TS 36.839.

-	In state 2: when the UE is attached to the source cell, a handover failure is counted if one of the following criteria is met:

1)	Timer T310 has been triggered or is running when the HO_CMD is received by the UE (indicating PDCCH failure) or

2)	RLF is declared in the state 2

The simulation results of SLS for Indirect measurement event prediction for temporal domain case A is shown in Table 4.

Table 4: The simulation results of SLS

|  | Ho number (per UE per second) | HoF rate (%) | HoF of state 2_1(%) (Timer T310 has been triggered or is running when the HO_CMD is received by the UE (indicating PDCCH failure);) |
| --- | --- | --- | --- |
| Non-AI | 0.2419 | 17.32% | 13.45% |
| AI | 0.2490 | 13.13% | 7.71% |

Based on the above simulation results, it could be observed that for Indirect measurement event prediction for temporal domain case A, the HoF rate (calculated in state 2) could decrease from 17.32% (Non-AI) to 13.13%, the gain of HO performance is about 4.19%. And the HoF rate calculated in state 2_1 could decrease from 13.45% (Non-AI) to 7.71%, the gain of HO performance could reach 5.74%.

In the case where the HoF rate calculated in state 2_1, the more gain of HO performance (5.74%) could be achieved. It seems that Indirect measurement event prediction for temporal domain case A is more helpful to reduce handover failure occurs before RLF.

Observation 4: In our simulation of Indirect measurement event prediction for temporal domain case A, in the case that UE speed is 90 km/h and ETD is 80ms, the HoF rate (calculated in state 2) could decrease from 17.32% (Non-AI) to 13.13%, the gain of HO performance is about 4.19%.

Observation 5: For Indirect measurement event prediction for temporal domain case A, when the HoF rate calculated in state 2_1, it could decrease from 13.45% (Non-AI) to 7.71%, the gain of HO performance could reach 5.74%.

Observation 6: In the case where the HoF rate calculated in state 2_1, the more gain of HO performance (5.74%) could be achieved. That means Indirect measurement event prediction for temporal domain case A is more helpful to reduce handover failure occurs before RLF.

# 3 Conclusion

In this paper, we provide our initial simulation results of Indirect measurement event prediction for temporal domain case A. We have the following observations:

Observation 1: In our simulation of Indirect measurement event prediction for temporal domain case A, in the case that UE speed is 90 km/h and ETD is 80ms, the values of Precision is 0.9503, which is close to 1. That means for Indirect measurement event prediction for temporal domain case A, the prediction of measurement event occurs is reliable.

Observation 2: The values of Recall is slightly lower (0.9472), but it still around 0.95. That means for Indirect measurement event prediction for temporal domain case A, the probability of a missed measurement event is quite low.

Observation 3: The values of F1 score is 0.9535 which is close to 1. That means Indirect measurement event prediction for temporal domain case A could achieve high prediction accuracy.

Observation 4: In our simulation of Indirect measurement event prediction for temporal domain case A, in the case that UE speed is 90 km/h and ETD is 80ms, the HoF rate (calculated in state 2) could decrease from 17.32% (Non-AI) to 13.13%, the gain of HO performance is about 4.19%.

Observation 5: For Indirect measurement event prediction for temporal domain case A, when the HoF rate calculated in state 2_1, it could decrease from 13.45% (Non-AI) to 7.71%, the gain of HO performance could reach 5.74%.

Observation 6: In the case where the HoF rate calculated in state 2_1, the more gain of HO performance (5.74%) could be achieved. That means Indirect measurement event prediction for temporal domain case A is more helpful to reduce handover failure occurs before RLF.

# Reference

- R2_125bis_ChairNotes_24-04-19_final, Apr 2024

- R2_127bis_ChairNotes_24-10-18_15-45_eom_clean, Oct 2024

- R2_128_ChairNotes_24-11-22_17-00_eom, Nov 2024

- R2-2407781, Summary of offline [AT127][026][AI Mob] Simulation assumptions, OPPO

- 3GPP TS 36.839: " Evolved Universal Terrestrial Radio Access (E-UTRA); Mobility enhancements in heterogeneous networks".

This models  a radio link/PDCCH failure occurring in source cells. This criterion is equivalent to the CQI measurement criterion for triggering the T310 and keeping T310 running.  As a result, the UE measurement report and/or the handover command will fail due to the bad radio conditions and hence a handover failure is declared. If before HO_CMD is issued the long term average wideband CQI is above Qin, we consider the radio link is recovered (equivalent to N311 is set to 1).