3GPP TSG-RAN WG2 Meeting #129	R2-2501042

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.6.3

Source:	CMCC

Title:	Discussion on L1 event triggered measurement reporting

Document for:	Discussion

Introduction

At the RAN2#128 meeting we reached the following agreements [1]:

RAN2 #128: Agreements on L1 event triggered MR - TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT. - TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam. - TTT is applied to the leaving condition. - Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation. - Either CSI-RS or SSB could be configured as candidate beam and the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type, i.e. the RS for current beam of serving cell is same as or QCLed with the QCL RS of the indicated TCI state, according to what is agreed in RAN1. - SSBRI and CRI is used to represent the candidate beam ID in LTM MR MAC CE. - For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE. - For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE. - A single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals. - Support the truncated measurement report MAC CE.

At the RAN #105 meeting, it was agreed to support conditional Intra-CU LTM [2]:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

Discussion

L1 events definition

The objective of L1 event-triggered measurements is still in progress, and the following events have been agreed upon so far:

RAN2#126 Agreement Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serv-ing cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

We propose that an additional use case to be considered for conditional LTM is “Early Sync Management of LTM Candidate Cells.” Effectively managing LTM candidate cells remains a challenging task for the network, as ensuring seamless switching and maintaining synchronization are critical for optimal performance. Among the various features available for LTM, the early synchronization procedure stands out as one of the most essential, playing a key role in enabling RACH-less handovers for UE to reduce handover delay.

However, in the RAN2#126 meeting, we have only agreed on the use case “Select the candidate beam/cell to trigger early synchronization”, and lack of validity management after the initial trigger.

RAN2#126 Agreement: - Event triggered L1 measurement should be designed for the following LTM purposes: - Select the candidate beam/cell to trigger early synchronization. - Select the target beam/cell and trigger LTM cell switch procedure.

Observation 1: The early synchronization procedure is the most important feature in RACH-less LTM, but currently, there is a lack of effective management for synchronized candidate cells, which can impact the performance of conditional LTM.

Consider two possible scenarios that may occur:

First, after the TCI state of the LTM candidate cell is pre-activated, the radio conditions may change before the UE receives the LTM cell switch command. Due to the lack of an appropriate measurement event, the network may not be aware of the change in radio conditions through the measurement report, which could result in the network indicating an activated TCI state that is not the best beam after the cell switch.

Second, after the TCI state of the LTM candidate cell is pre-activated, the radio conditions may deteriorate before the UE receives the LTM cell switch command. After the UE performs a RACH-less LTM handover, data transmission may not be possible on the deteriorated beam.

Therefore, we believe it is crucial to introduce the corresponding events to help the network address these use cases in the most efficient way. Based on this, we propose:

Proposal 1: Supporting the following events:

- Event LTM6: Beam of candidate cell becomes worse than absolute threshold

- Event LTM7: Beam of candidate cell becomes offset better than pre-activated beam of same candidate cell

Support of semi-persistent CSI-RS

In RAN1#118bis meeting, the working assumption regarding whether to support semi-persistent CSI-RS for candidate cell L1-RSRP measurement for gNB scheduled reporting is left for further discussion based on the Reply LS from RAN3.

Working Assumption In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective - Send an LS to RAN3 (CC RAN2) to ask for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload). Support of semi-persistent CSI-RS is subject to UE capability. Final LS in R1-2409283.

Since the Reply LS from RAN3 has confirmed it is feasible to specify the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS[3]. For gNB scheduled L1-RSRP measurement and reporting, compared with periodic CSI-RS, semi-persistent CSI-RS can be activated and deactivated flexibly for UE to measure the CSI-RSs for a subset of candidate cells, which could reduce the measurement and report overhead for UE.

The activation/deactivation of the existing serving cell's semi-persistent CSI-RS resource set is achieved through MAC CE, as described in TS 38.321 Clause 5.18.2:

TS 38.321 5.18.2 Activation/Deactivation of Semi-persistent CSI-RS/CSI-IM resource set The network may activate and deactivate the configured Semi-persistent CSI-RS/CSI-IM resource sets of a Serving Cell by sending the SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE described in clause 6.1.3.12. The configured Semi-persistent CSI-RS/CSI-IM resource sets are initially deactivated upon (re-)configuration by upper layers and after reconfiguration with sync. The MAC entity shall: 1> if the MAC entity receives an SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE on a Serving Cell: 2> indicate to lower layers the information regarding the SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE.

The semi-persistent CSI-RS of the candidate cell can also be activated/deactivated using MAC CE.

Proposal 2: The activation/deactivation mechanism for the candidate cell's semi-persistent CSI-RS can refer to a similar mechanism used in the serving cell.

The signaling procedure can be summarized as follows:

Fig 1 Activation procedure

First, the serving cell sends an activation request for the semi-persistent CSI-RS resource to the candidate cell. Upon receiving the request, the candidate cell starts transmitting the corresponding semi-persistent CSI-RS according to the RRC configuration and sends a success confirmation back to the serving cell. Finally, after receiving the successful activation confirmation, the serving cell notifies the UE via a dedicated MAC CE, allowing it to begin the corresponding semi-persistent CSI-RS measurements.

Fig 2 Deactivation procedure

For deactivation, the serving cell does not need to wait for the candidate cell's feedback before notifying the UE to deactivate the semi-persistent CSI-RS resource. The process is considered complete once the serving cell receives the successful deactivation confirmation from the candidate cell.

The aspects of these procedures involving XnAP and F1AP are discussed by RAN3. Once RAN1 finalizes the CSI-RS resource configuration method, RAN2 will need to design a dedicated MAC CE for the activation and deactivation of the candidate cell's semi-persistent CSI-RS resource.

Proposal 3: After RAN1 completes the CSI-RS resource configuration design, RAN2 needs to design a dedicated MAC CE for the activation and deactivation of the candidate cell's semi-persistent CSI-RS resource.

Conclusions

In this contribution, we analyse for measurement event evaluation, following are the observations and proposals.

Observations:

Observation 1: The early synchronization procedure is the most important feature in RACH-less LTM, but currently, there is a lack of effective management for synchronized candidate cells, which can impact the performance of conditional LTM.

Proposals:

Proposal 1: Supporting the following events:

- Event LTM6: Beam of candidate cell becomes worse than absolute threshold

- Event LTM7: Beam of candidate cell becomes offset better than pre-activated beam of same candidate cell

Proposal 2: The activation/deactivation mechanism for the candidate cell's semi-persistent CSI-RS can refer to a similar mechanism used in the serving cell.

Proposal 3: After RAN1 completes the CSI-RS resource configuration design, RAN2 needs to design a dedicated MAC CE for the activation and deactivation of the candidate cell's semi-persistent CSI-RS resource.

References

- R2-2410911 Report from session on R16/17/18 V2X/SL, R18/19 MOB and R19 NES.

- RP-242356 Revised Work Item: NR mobility enhancements Phase 4.

- R3-247911 Reply LS for LS on the support of semi-persistent CSI-RS resource for LTM candidate cells.