3GPP TSG-RAN WG2 Meeting #129		                          R2-2501043

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.4.3

Source: 	CMCC

Title:	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

Document for:	Discussion

- Introduction

At the RAN2#128 meeting, we had the following agreements[1]:

- The entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. FFS if LR measurement is needed.

- FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements

- FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation.

At the RAN2#127bis meeting, we reached the following agreements[2]:

Working assumption

- For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’.

- FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

At the RAN2#127 meeting, we reached the following agreements[3]:

- RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs.

- For serving cell measurement offloading (i.e., there is no serving cell measurement by MR):

- The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered.

- The exit condition is based on the LR measurement results.

- Discussion

In the LP-WUS discussion of RAN4, they listed several cases for measurement offloading/relaxation. In order to avoid duplication of work and conduct efficient discussions within a limited time, RAN2 should prioritize research based on the cases approved by RAN4.

Case #1 and #3 have been agreed by RAN4:

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

RAN4 to further discuss case #2:

| RRM measurement case index | MR serving cell measurement | MR neighbouring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #2 Relaxed case a | On with relaxation measurement | Off | ON |

Proposal 1: RAN2 prioritize research based on the cases approved by RAN4, which are Case #1 and #3.

2.1 Case #1

According to the conclusions of RAN4#111, measurement offloading is supported, and we reached the following agreements in RAN2#127:

RAN2#127 Agreement For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. - The exit condition is based on the LR measurement results.

In the 'Fully offloading case' agreed by RAN4, MR serving and neighboring cell measurement are stopped. The legacy cell re-selection procedure in TS 38.304 are:

- If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may not perform intra-frequency measurements.

- If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ,  the UE may choose not to perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority.

In our understanding, in order not to affect the existing cell (re)selection process in TS 38.304, this means that the threshold for offloading measurements to the LR is not less than the threshold for starting neighboring cell measurements. Otherwise, the MR cannot stop measuring neighboring cells. The relationship between the relevant thresholds is shown in Figure 1.

Figure 1 Case #1 threshold

Proposal 2: For case #1, the threshold for offloading measurements to the LR is not less than SIntraSearchP.

Considering that the latency of starting MR is as high as hundreds of milliseconds, we should be very cautious about offloading serving cell measurements to LR. If a UE is in a high mobility state, it is likely to reach the cell edge quickly, so the UE should use MR to perform neighbor measurements to prepare for possible handovers. If the UE performs measurement offloading and turns off MR at this time, on the one hand, due to the extremely high startup delay of the MR, the UE is likely to miss the best handover opportunity; on the other hand, it will also cause unnecessary conversion power consumption. Therefore, the mobility of UE should be considered for serving cell measurement offloading, that is, measurement offloading should not be performed when the UE is in a high-speed state.

Proposal 3: UEs with high-mobility do not perform serving cell measurement offloading.

2.2 Case #3

Case #3: Serving cell measurement relaxation

We would like to revisit the entry condition for Case #3, as we believe the conclusion from the RAN2 #128 meeting was not rigorous enough. The conclusion from the RAN2 #128 meeting was as follows:

RAN2#128 Agreement - The entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. FFS if LR measurement is needed.

However, consider the following scenario: the threshold for Case #1 is configured to be the same as SIntraSearchP, meaning Case #2 is not configured, and there is no transition region between Case #1 and Case #3. In this case, when the UE transitions from Case #1 to Case #3, there is only the LR measurement of the serving cell, which contradicts the existing conclusion that "at least if serving cell quality measured by MR is higher than the relaxation threshold," as shown in Figure 2.

Figure 2 Case #3 entry/exit condition

Therefore, in this case, the exit condition for Case #1 (i.e., LR lower than a certain threshold) is also the entry condition for Case #3 serving cell RRM relaxation, and this condition is based solely on the LR measurement.

For another entry condition from outside to inside, the question of whether LR measurement is required still remains. In our view, this depends on the network configuration. If the network has configured an LR threshold, then the LR measurement result should be considered.

Proposal 4: Whether LR measurement is needed depends on whether the network has configured the LR threshold.

Proposal 5: For case #3, the entry condition for serving cell RRM relaxation can be defined as:

- (Case #1 to Case #3) Serving cell quality measured by LR is lower than SIntraSearchP, e.g. RSRP and/or RSRQ, if offloading threshold for case #1 is configured to be the same as SIntraSearchP; or

- (Case #2 to Case #3) Serving cell quality measured by MR is lower than SIntraSearchP, e.g. RSRP and/or RSRQ, and LR measurement result could also be considered if LR threshold is configured; or

- (Cell edge to Case #3) Serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ, and LR measurement result could also be considered if LR threshold is configured.

Similarly, the exit condition needs to consider two situations.

Proposal 6: No separate exit condition is needed, and the exit condition can be defined as failing to meet the entry condition.

Proposal 7: For case #3, the exit condition for serving cell RRM relaxation can be defined as:

- (Case #3 to Case #1/#2) Serving cell quality measured by MR is higher than SIntraSearchP, e.g. RSRP and/or RSRQ, and LR measurement result could also be considered if LR threshold is configured; or

- (Case #3 to cell edge) Serving cell quality measured by MR is lower than relaxation threshold, e.g. RSRP and/or RSRQ, and LR measurement result could also be considered if LR threshold is configured.

Case3: Neighboring cell measurement relaxation

In RAN2#126, we agreed that the RRM measurement of the neighboring cell can only be performed by MR. In RAN2#127bis and #128, we reached the following agreements:

RAN2#127bis Agreements Working assumption - For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. - FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available. RAN2#128 Agreements - FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation.

For the ‘UE not at cell edge’ criterion, if the serving cell applies measurement relaxation while neighboring cells do not, it will significantly impact power-saving gains. Moreover, it introduces more complex procedures without providing sufficient positive benefits.

Proposal 8: The entry condition for R19 neighbour cell RRM measurement relaxation is the same as R19 serving cell RRM measurement relaxation.

In addition, in the discussion of RAN4, most companies support the scaling factor(s) larger than that of legacy Rel-16 UE power saving WI (i.e., 3).

RAN4 Agreement: Scaling factor(s) is/are >= 4. Further down-select from - Option 1: 1 fixed scaling factor - Option 2: multiple values configured by NW - Option 3: multiple values, scaling factor value depends on DRX cycle length (QC MTK apple xiaomi vivo)

Considering that LR cannot assist MR in performing neighbor cell measurement, in our opinion, R19 Neighboring cell measurement relaxation should have stricter evaluation conditions than R16. Therefore, the MR threshold in the entry condition of R19 Neighboring cell measurement relaxation should be higher than SSearchThresholdP in R16. As shown in Figure 3.

Figure 3 Case #3 R19 neighboring cell measurement relaxation

Proposal 9: The threshold in the entry condition of R19 neighboring cell measurement relaxation should be higher than SSearchThresholdP in R16, and lower than SIntraSearchP.

For the ‘UE with low mobility’ criterion, we think it is also necessary. Usually we think that a high-speed mobile UE has a higher handover probability, so the UE needs to perform accurate neighbor cell measurement. However, the relaxation factor currently discussed by RAN4 is very large. If a high-speed UE performs R19 neighbor cell measurement relaxation, it will bring risks in terms of mobility.

Proposal 10: For measurement relaxation of neighboring cell, to avoid mobility risks, the ‘UE with low mobility’ criterion is necessary.

2.3 Is case #2 supported?

RAN4 is still discussing whether to support case 2, that is, the MR performs relaxed measurements on the serving cell while not performing measurements on the neighboring cells. As shown in Figure 1, if case 2 is supported, its working area will be between case 1 and case 3.

UEs usually give priority to intra-frequency cell reselection, i.e. the network will configure the UE to perform intra-frequency measurements before inter-frequency measurements. Inter-frequency cell reselection are typically used in cases of coverage issues or for purposes such as load balancing.

In urban environments, due to the irregular shapes of cells, the UE may suddenly detect a better cell at a corner, leading to a handover. Therefore, the configuration of SIntraSearchP can be quite conservative.

In rural environments, cells are typically regular circular shapes, and the network can configure a lower intra-frequency measurement trigger threshold to help the UE save power.

So in our opinion, whether case #2 is needed depends on the SIntraSearchP configured by the network. If SIntraSearchP is configured very conservatively, then case 1 is sufficient; if SIntraSearchP is configured more aggressively, which means that the UE does not need to perform neighbor measurements in a larger area, then it is reasonable to support case 2 as a transition between case 3 and case 1. In short, case 2 can be supported, but it depends on the network configuration.

Observation 1: If SIntraSearchP is configured conservatively, the application scope of case #2 will be very limited, so it is a rare case, and it seems redundant when case1 is already agreed.

Observation 2: If SIntraSearchP is configured to a lower value, then case #2 will have a relatively larger application scope.

Proposal 11: Case #2 is a possible case depending on network configuration for various scenarios.

- Conclusions

In this contribution, we discussed the LP-WUS measurement relaxation/offloading in RRC_IDLE/INACTIVE mode, and following are the observations and proposals.

Observations:

Observation 1: If SIntraSearchP is configured conservatively, the application scope of case #2 will be very limited, so it is a rare case, and it seems redundant when case1 is already agreed.

Observation 2: If SIntraSearchP is configured to a lower value, then case #2 will have a relatively larger application scope.

Proposals:

Proposal 1: RAN2 prioritize research based on the cases approved by RAN4, which is Case #1 and #3.

Proposal 2: For case #1, the threshold for offloading measurements to the LR is not less than SIntraSearchP.

Proposal 3: UEs with high-mobility do not perform serving cell measurement offloading.

Proposal 4: Whether LR measurement is needed depends on whether the network has configured the LR threshold.

Proposal 5: For case #3, the entry condition for serving cell RRM relaxation can be defined as:

- (Case #1 to Case #3) Serving cell quality measured by LR is lower than SIntraSearchP, e.g. RSRP and/or RSRQ, if offloading threshold for case #1 is configured to be the same as SIntraSearchP; or

- (Case #2 to Case #3) Serving cell quality measured by MR is lower than SIntraSearchP, e.g. RSRP and/or RSRQ, and LR measurement result could also be considered if LR threshold is configured; or

- (Cell edge to Case #3) Serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ, and LR measurement result could also be considered if LR threshold is configured.

Proposal 6: No separate exit condition is needed, and the exit condition can be defined as failing to meet the entry condition.

Proposal 7: For case #3, the exit condition for serving cell RRM relaxation can be defined as:

- (Case #3 to Case #1/#2) Serving cell quality measured by MR is higher than SIntraSearchP, e.g. RSRP and/or RSRQ, and LR measurement result could also be considered if LR threshold is configured; or

- (Case #3 to cell edge) Serving cell quality measured by MR is lower than relaxation threshold, e.g. RSRP and/or RSRQ, and LR measurement result could also be considered if LR threshold is configured.

Proposal 8: The entry condition for R19 neighbour cell RRM measurement relaxation is the same as R19 serving cell RRM measurement relaxation.

Proposal 9: The threshold in the entry condition of R19 neighboring cell measurement relaxation should be higher than SSearchThresholdP in R16, and lower than SIntraSearchP.

Proposal 10: For measurement relaxation of neighboring cell, to avoid mobility risks, the ‘UE with low mobility’ criterion is necessary.

Proposal 11: Case #2 is a possible case depending on network configuration for various scenarios.

- References

- R2-2410912, Report from session on Rel-18 MIMO and MUSIM, Rel-19 MIMO, LPWUS, and SBFD.

- draft R2-2409212, Report from session on Rel-18 MIMO and MUSIM, Rel-19 MIMO, LPWUS, and SBFD.

- R2-2407572, Report from session on R18 MIMOevo, R18 MUSIM, R19 LP-WUS and R19 SBFD.