3GPP TSG-RAN WG2 Meeting #129							R2-2501063

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.7.6

Source:	China Telecom

Title:	Further Discussion on Rate Control for XR

Document for:	Discussion

1	Introduction

During RAN#105 meeting, a new objective was introduced in XR for NR Phase 3 WID to specify MAC layer XR rate control for uplink congestion adaption.

-	Specify uplink congestion signalling [RAN2]:

- Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion.

In RAN2#128 meeting, rate control was discussed and the following agreements were made:

Agreements on XR rate control - RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level. - gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN - Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB. - RAN2 will not discuss/support rate indication for DL unless WID is updated to include it by RANP. - RAN2 assumes that the congestion situation can be known at the gNB without any indication from the UE - FFS whether UL MAC CE rate query/preference is supported as UE recommendation to the NW or whether legacy MAC CE can serve this already. FFS in which scenarios this is useful.

According to the outcome of agreements, the details of rate indication and UL MAC CE query are still need to be discussed. In this contribution, we focus on open issue on XR rate control and provide our views.

2	Discussion

In the last meeting, RAN2 agreed to support rate indication at the per-QoS flow level. Regarding rate control, there are at least two options under consideration:

- Option 1: Flows are indicated by MAC CE.

- Option 2: Flows are indicated by RRC, while MAC CE is per DRB.

We advocate for Option 1, as it offers a straightforward approach to rate control. In contrast, Option 2 allows the network to configure which flows require rate control within the MAC CE. However, XR services encompass multiple distinct traffic flows, and congestion conditions for different flows may change dynamically. Consequently, the network would need to send an RRC Reconfiguration message each time flow configurations for MAC CE require modification.

Proposal 1: Agree that flows are indicated by MAC CE.

Furthermore, whether a single or multiple flow indications should be included in one MAC CE remains an open discussion. The Release 15 recommended bit rate MAC CE only supports a single DRB and has a fixed size. The existing mechanism is designed solely for codec adaptation for MMTEL voice or MMTEL video. These traffic flows may be mapped to one or multiple DRBs, depending on the local RRM strategy. According to the agreement from the last meeting, the RAN side can estimate congestion information at both per-DRB and per-QoS flow levels. Moreover, multiple traffic flows may experience congestion simultaneously. If a single-flow indication or DRB/LCH in one MAC CE is adopted, the network would need to send multiple MAC CEs to the UE simultaneously, which is inefficient for XR services. Additionally, XR traffic flows exhibit significant differences in data rate requirements, meaning not all flows require rate control. Based on this analysis, one rate control MAC CE can encompass multiple flows. The network can also configure the maximum number of QoS flows per MAC CE via RRC.

Proposal 2: One rate control MAC CE can comprise multiple flows, and the network can configure the maximum number of QoS flows per MAC CE via RRC.

As illustrated in Figure 1, the current Recommended Bit Rate MAC CE has a fixed size and consists of two octets. The Bit Rate field is 6 bits, comprising 56 utilized code points to indicate values ranging from 0 to 8000 kbps, while the X Bit Rate Multiplier is 1 bit. Additionally, to support a recommended bit rate with per-QoS flow granularity, a new MAC CE should be specified in Release 19.

Figure 1 Recommended bit rate MAC CE

Proposal 3: a new MAC CE should be specified in Rel-19 to support Per-QoS flow granularity recommended bit rate.

Per TS38.413 and TS23.501, the maximum number of QoS flows is 64. From the perspective of MAC CE, there is no need to retain the existing size of the QoS Flow ID within the MAC CE. To reduce overhead in the MAC CE, the QoS Flow ID can be mapped to a shorter ID configured by the RRC.

9.3.1.51	QoS Flow Identifier

This IE identifies a QoS flow within a PDU Session, or a MBS QoS flow within a MBS session. The definition and use of the QoS Flow Identifier is specified in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| QoS Flow Identifier | M |  | INTEGER (0..63, …) |  |

Proposal 4: To reduce overhead in the MAC CE, the QoS Flow ID can be mapped to a shorter ID configured by the RRC.

3 	Conclusion

Based on the above analysis, we have the following proposals:

Proposal 1: to agree the flows are indicated by MAC CE.

Proposal 2: One rate control MAC CE can comprise multiple flows, and the network can configure the maximum number of QoS flows per MAC CE via RRC.

Proposal 3: a new MAC CE should be specified in Rel-19 to support Per-QoS flow granularity recommended bit rate.

Proposal 4: To reduce overhead in the MAC CE, the QoS Flow ID can be mapped to a shorter ID configured by the RRC.

4	References

[1]	TR 38.838, Study on XR (Extended Reality) evaluations for NR, V17.0.0, 2022-04