3GPP TSG-RAN WG2 Meeting #129	R2-2501084

Athens, Greece, Feb. 17th – 21st, 2025

Source:	vivo

Title:	Discussion on new triggering condition of SHR for CHO with candidate SCGs

Agenda Item:	8.10.2.2

Document for:	Discussion and Decision

- Introduction

At the previous meetings, the agreements regarding the CHO with candidate SCGs were reached as follows.

- RAN2 125bis RAN2 to study failure and near failure scenarios for CHO with candidate SCGs. - RAN2 126 RAN2 to enhance the RLF report with additional information regarding the state of the two execution conditions. We see later if we also can enhance the SCGFailureInformation report. - RAN2 127 UE includes following information in RLF report: b. Time information regarding condition fulfilment for CHO with candidate SCGs. Details are FFS. We consider both the case when both CHO condition and associated CPC condition are fulfilled, and the case when CHO (or CPC) is fulfilled but CPC (or CHO) conditions are not fulfilled. c. Measurement results of PCells and PSCells. - RAN2 127bis Agreements - UE reports the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO), and the first met execution condition (as agreed by RAN3), for a failed CHO with candidate SCGs. Details FFS. - Include the elapsed time between the point in time of the first fulfilled condition and RLF in RLF report. Details FFS.. - RAN2 128 Agreements - RAN2 understands that current agreements is that the UE shall logs (in RLF report, SHR report and SCGFailureInformation) time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell). We don’t intend to do further or more granular enhancements. - Measurement results of PCells and PSCells and the time information (as agreed for RLF) are included in SHR and SCGFailureInformation also. We will check what the spec impact of this is, e.g. something in the spec today may already make the UE log this..

At the RAN2#128 meeting, it was agreed that the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO) will also be logged in the SHR report. However, the triggering condition of SHR is not clear.

In this contribution, we will provide further analysis of the triggering condition of SHR for CHO with candidate SCGs.

- Discussion

From the NW perspective, the NW expects the conditions for CHO and CPAC to be as close as possible for CHO with candidate SCGs. So if PCell changes with PSCell addition/change is successfully executed but if the time gap between the met conditions is large, the NW needs to know that situation, UE should report some information to assist the NW in optimizing the configuration.

For the above near-failure scenario, the SHR can be reused as the baseline for enhancement. However, how to trigger the SHR has not been discussed yet. The existing SHR triggering conditions include:

- ThresholdPercentageT304;

- ThresholdPercentageT310;

- ThresholdPercentageT312;

- RLF occurred at the source PCell during the DAPS handover while T304 was running.

However, none of the above conditions can be reused for the intention. The most straightforward way is introducing another triggering condition for the SHR report, i.e., the time gap between the time when conditions for CHO and CPAC are met exceeds a configured threshold.

Proposal 1: Introduce a new triggering condition of SHR for CHO with candidate SCGs, i.e., the time gap between the time when conditions for CHO and CPAC are met exceeds a configured threshold.

- Conclusion

Based on the above analysis, we make the following proposals:

Proposal 1: Introduce a new triggering condition of SHR for CHO with candidate SCGs, i.e., the time gap between the time when conditions for CHO and CPAC are met exceeds a configured threshold.

- Reference