3GPP TSG-RAN WG2 Meeting #129											R2-2501087

Athens, Greece, Feb.17th–21st, 2025

Agenda item:		8.5.3

Title: 	Remaining issues of on-demand SIB1

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

In this paper, we share some consideration on the remaining issues for on demand SIB1, with the latest way forward agreed in RAN#106 taken into consideration.

- Discussion

  - UL WUS configuration

- Issue 1: Confirm RAN1 agreed parameters

The following parameters have been agreed by RAN1 in the past two meetings:

| Purpose | Parameters |  |  |
| --- | --- | --- | --- |
| To which cell does the configuration applies | NES-CellId | PhysCellId |  |
|  |  | ARFCN-ValueNR |  |
|  | Random access common parameters | rsrp-ThresholdSSB |  |
|  |  | prach-RootSequenceIndex |  |
|  |  | msg1-SubcarrierSpacing |  |
|  |  | restrictedSetConfig |  |
|  | UL common parameters | frequencyBandList |  |
|  |  | absoluteFrequencyPointA |  |
|  |  | offsetToCarrier |  |
|  |  | p-Max |  |
|  |  | ULSubCarrierSpacing |  |
|  | SIB1-RequestConfig | ssb-PeriodicityServingCell |  |
|  |  | n-TimingAdvanceOffset |  |
|  |  | ss-PBCH-BlockPower |  |
|  |  | SSB-positionInBurst |  |
|  |  | tdd-UL-DL-ConfigurationCommon |  |
|  |  | rach-OccasionsSIB1 | Prach-ConfigurationIndex |
|  |  |  | msg1-FDM |
|  |  |  | msg1-FrequencyStart |
|  |  |  | zeroCorrelationZoneConfig |
|  |  |  | preambleReceivedTargetPower |
|  |  |  | preambleTransMax |
|  |  |  | powerRampingStep |
|  |  |  | ra-ResponseWindow |
|  |  |  | ssb-perRACH-Occasion |
|  |  | sib1-RequestPeriod |  |
|  |  | sib1-RequestResources | ra-PreambleStartIndex |
|  |  |  | ra-AssociationPeriodIndex |
|  |  |  | ra-ssb-OccasionMaskIndex |

In summary, the parameters include:

- NES cell identity

- UL common configuration

- Random access common parameters

- RACH occasions and preambles for SIB1 request

After reviewing all the parameters, we understand all of them are needed for on demand SIB1 request.

Proposal 1: Confirm that all the parameters agreed by RAN1 in the above table are needed for on demand SIB1 request.

Apart from all the parameters listed above, the following issues need to be addressed, which will impact the UL WUS configuration.

- Issue 2: The on demand SIB1 procedure

The existing on demand SI request procedure is categorized as follows:

- Msg 1 based on demand SI request: UE acquire the requested SI message if the RAR reception is successful for 4-step RACH.

- Msg3 based on demand SI request: UE acquire the requested SI message if the contention resolution is successful for both 2-step and 4-step RACH.

For the on demand SIB1 procedure, we have the following agreements in the past several meetings:

RAN2#126

RAN2 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell.

RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

RAN2#127

Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell).

Since the on demand SIB1 request is for SIB1 acquisition without initial access, we understand the following options can be considered for UE acquisition of SIB1 after sending the request:

- Option 1: UE acquire the request SIB1 if the RAR reception is successful

  - Option 1.1: The Msg3 and contention resolution is omitted.

  - Option 1.2: The Msg3 and contention resolution is still kept.

- Option 2: UE acquire the requested SIB1 if the contention resolution is successful for both 2-step and 4-step RACH.

  - Option 2.1: UE acquire the requested SIB1 after Msg4 for 4-step RACH.

  - Option 2.1: UE acquire the requested SIB1 after MsgB for -2-step RACH.

Figure 1. UE acquisition of SIB1

For option 1.1, there will be no contention resolution. UE start to acquire SIB1 and will re-initiate the request if the Kssb still indicate no SIB1.

For option 1.2, UE will re-initiate the request procedure if contention resolution fails and the Kssb still indicate no SIB1. For the case that the contention resolution fails but the Kssb indicate SIB1 broadcasting then UE does not need to re-initiate the request procedure as the SIB1 broadcasting may have been triggered by another UE.

For option 2.1 and 2.2, UE start to acquire SIB1 only when its own contention resolution is successful while network may have started the broadcasting before that due to the request from other UEs.

With the above analysis, we do not see much benefit of having the contention resolution for on demand SIB1 request as network would start SIB1 broadcasting without differentiate which UE sent the request. Even if the contention resolution for a UE fails, UE may still benefit from other SIB1 request thus no need to re-try.

Thus, option 1.1 is preferred with efficiency and simplicity.

Proposal 2: UE start to acquire SIB1 if the RAR reception is successful. The Msg3 and contention resolution is omitted for on demand SIB1 request procedure.

- Issue 3: The preambles for on demand SIB1 request

RAN1 has agreed to provide the ra-PreambleStartIndex from on demand SIB1 request. With 64 preamble in total, UE also needs to know which preambles among all the 64 are for SIB1 request.

Proposal 3: The total number of preambles applied for on demand SIB1 request should be provided to UE.

  - Target NES cell selection information

The following agreements have been made before on the triggering condition of WUS transmission and reselection priority for NES:

RAN2#126

Triggering condition of WUS transmission

If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

RAN2#127bis

NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

RAN2#128

New NES-specific reselection priority parameters for NES UEs are defined for the purpose of prioritizing/deprioritizing a NES frequency.

Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

With the above agreement, the following options can be considered on how to provide the information for UE to select a target NES cell to send the SIB1 request.

- Option 1: Include all the cell reselection related parameters together with the WUS configuration for NES cells in the new SIB

- Option 2: Add the NES cells as neighbor cells along with the NES-specific reselection priority and IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES in SIB3/4 and reuse the existing cell reselection parameters in SIB2/3/4 in target NES cell selection

Option 2 would be better with less signaling overhead thus is preferred from our side.

Proposal 4: Add the NES cells as neighbor cells along with the NES-specific reselection priority and IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES in SIB3/4 and reuse the existing cell reselection parameters in SIB2/3/4 in target NES cell selection.

  - Validity of the WUS configuration

The following agreements have been made last meeting on the validity of the WUS configuration:

RAN2#128

UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE acquired WUS configuration. FFS on SI validity area rather than cell level.

We understand the intention is to emphasize that if UE perform cell reselection to another cell, the newly provisioned WUS configuration would override the old ones and we left an FFS on SI validity area rather than cell level.

As we agreed to introduced a new SIB to provide the WUS configuration, this SIB can be cell specific or area specific per the existing structure:

- Cell specific: The SIB is only valid in the current cell that provides the SIB

- Area specific: The SIB is valid in an area identified by systemInformationAreaID, which consists of one or several cells

The intention of supporting area specific SIB is that some system information maybe common in multiple cells, e.g. PWS information, which would also be possible for the WUS configuration.

Figure 2. Valid area of WUS configuration

As shown in the above figure, since we allow a cell to provide its own WUS configuration, the WUS configuration for all the four cells can be provided in cell A and any of the four cells and be valid in these cells.

Proposal 5: The WUS configuration can be cell specific or area specific.

The following options can be considered on supporting the area specific WUS configuration:

- Option 1: Reuse the idea of area specific SIB. The SIB containing WUS configuration is valid in an area identified by systemInformationAreaID, which consists of one or several cells

- Option 2: Define and provide valid area for WUS configuration.

For option 1, although we have the concept of system information area ID now, this id is common for all the SIBs while the valid area for the SIB containing WUS configuration would be quite different from other SIBs. Thus it is preferred to go for option 2.

Proposal 6: Define and provide valid area for WUS configuration.

The valid area for WUS configuration can be a list of cell identities or a new system information area id referring to multiple cells while the mapping between the area id and cells are maintained by the network.

Proposal 7: The valid area for WUS configuration can be a list of cell identities or a new system information area id referring to multiple cells.

  - SIB1 request to the serving cell

The following way forward has been agreed in RAN#106 [1]:

- RAN confirms that Case 2 also includes the case where a particular Cell A can switch to become a NES Cell. This means that Cell A can provide UL-WUS configuration for itself before becoming a NES cell.

  - No further optimizations will be considered for NES UEs for this scenario.

  - No further optimizations to handle non-NES capable UEs will be considered for this scenario.  It is assumed that these UEs can be handled by careful deployment.

  - No WID update is required

- On RRC Connected mode

  - The scope of WID will not be updated to include RRC Connected.

  - However, RAN2 will address the specific case of RLF, to use the same procedure as idle/inactive to request OD-SIB1 transmission.  No need to change the scope of WI to indicate this.

- Issue 1: Cell A providing its own UL-WUS configuration

We have agree that a NES cell can provide its own WUS configuration. With the way forward from RAN#106, cell A can also provide its own WUS configuration before becoming a NES cell.

Thus, we understand there would be a case that UE needs to request SIB1 from the serving cell, e.g. no other cell is better than the serving cell according to the cell reselection criteria during a time interval TreselectionRAT while the serving cell K_SSB indicates no SIB1 broadcasting but the WUS configuration is valid at UE side.

- Issue 2: RLF in RRC Connected mode

When RLF happens, UE would start T311, perform cell selection and acquire system information from the cell. If the K_SSB of the selected cell indicates no SIB1 broadcasting, UE can initiate SIB1 request procedure towards the selected cell if the WUS configuration for that cell is valid.

Proposal 8: The following cases should be supported for UE to request SIB1 from the serving cell without performing cell reselection:

- For UE in RRC_IDLE or in RRC_INACTIVE, if the WUS configuration of the serving cell is valid at UE side while K_SSB indicates no SIB1 broadcasting, and no other cell is better than the serving cell according to the cell reselection criteria during a time interval, UE can request SIB1 from the serving cell if it does not have valid SIB1.

- For UE in RRC_CONNECTED while T311 is running, if the WUS configuration of the serving cell is valid at UE side while K_SSB indicates no SIB1 broadcasting, UE can request SIB1 from the serving cell.

- Conclusion and proposals

Based on the analysis in previous sections, the following proposals are given:

Proposal 1: Confirm that all the parameters agreed by RAN1 in the following table are needed for on demand SIB1 request.

| Purpose | Parameters |  |  |
| --- | --- | --- | --- |
| To which cell does the configuration applies | NES-CellId | PhysCellId |  |
|  |  | ARFCN-ValueNR |  |
|  | Random access common parameters | rsrp-ThresholdSSB |  |
|  |  | prach-RootSequenceIndex |  |
|  |  | msg1-SubcarrierSpacing |  |
|  |  | restrictedSetConfig |  |
|  | UL common parameters | frequencyBandList |  |
|  |  | absoluteFrequencyPointA |  |
|  |  | offsetToCarrier |  |
|  |  | p-Max |  |
|  |  | ULSubCarrierSpacing |  |
|  | SIB1-RequestConfig | ssb-PeriodicityServingCell |  |
|  |  | n-TimingAdvanceOffset |  |
|  |  | ss-PBCH-BlockPower |  |
|  |  | SSB-positionInBurst |  |
|  |  | tdd-UL-DL-ConfigurationCommon |  |
|  |  | rach-OccasionsSIB1 | Prach-ConfigurationIndex |
|  |  |  | msg1-FDM |
|  |  |  | msg1-FrequencyStart |
|  |  |  | zeroCorrelationZoneConfig |
|  |  |  | preambleReceivedTargetPower |
|  |  |  | preambleTransMax |
|  |  |  | powerRampingStep |
|  |  |  | ra-ResponseWindow |
|  |  |  | ssb-perRACH-Occasion |
|  |  | sib1-RequestPeriod |  |
|  |  | sib1-RequestResources | ra-PreambleStartIndex |
|  |  |  | ra-AssociationPeriodIndex |
|  |  |  | ra-ssb-OccasionMaskIndex |

Proposal 2: UE start to acquire SIB1 if the RAR reception is successful. The Msg3 and contention resolution is omitted for on demand SIB1 request procedure.

Proposal 3: The total number of preambles applied for on demand SIB1 request should be provided to UE.

Proposal 4: Add the NES cells as neighbor cells along with the NES-specific reselection priority and IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES in SIB3/4 and reuse the existing cell reselection parameters in SIB2/3/4 in target NES cell selection.

Proposal 6: Define and provide valid area for WUS configuration.

Proposal 7: The valid area for WUS configuration can be a list of cell identities or a new system information area id referring to multiple cells.

Proposal 8: The following cases should be supported for UE to request SIB1 from the serving cell without performing cell reselection:

- For UE in RRC_IDLE or in RRC_INACTIVE, if the WUS configuration of the serving cell is valid at UE side while K_SSB indicates no SIB1 broadcasting, and no other cell is better than the serving cell according to the cell reselection criteria during a time interval, UE can request SIB1 from the serving cell if it does not have valid SIB1.

- For UE in RRC_CONNECTED while T311 is running, if the WUS configuration of the serving cell is valid at UE side while K_SSB indicates no SIB1 broadcasting, UE can request SIB1 from the serving cell.

- Reference

- RP‑243297	Way forward on NES