3GPP TSG-RAN WG2 Meeting #129										R2-2501089

Athens, Greece, Feb. 17th – 21st, 2025                                 	Revision of R2-2410166

Agenda item:		8.4.2

Title: 	Procedure and configuration of LP-WUS for IDLE and INACTIVE mode

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

In this contribution, we will further discuss the RAN2 related issues on procedure and configuration of LP-WUS for IDLE/INACTIVE modes and give our proposals.

- Discussion

2.1 Provision of LP-WUS related configuration

Based on the RAN1 agreement, the LP-SS design and LP-WUS design have been discussed, but has not concluded yet. Considering that LP-SS configuration and LP-WUS configuration highly depend on RAN1 agreement for LP-SS and LP-WUS design, RAN2 can postpone the discussion for LP-WUS and LP-SS related configuration until there is agreement for the LP-SS and LP-WUS design in RAN1.

Proposal 1: RAN2 can postpone the discussion on LP-WUS and LP-SS related configuration until there is agreement for the LP-SS and LP-WUS design in RAN1.

2.2 Activation/deactivation of LP-WUS

Since it has been agreed in RAN2#125bis that LP-WUS related configuration for IDLE/INACTIVE state is provided via system information, the provision of LP-WUS configuration via SIB can be seen as an implicit activation to apply LP-WUS configuration for all the UEs supporting this function. Meanwhile, if the LP-WUS configuration is no longer broadcast via SIB, it can be assumed that application of LP-WUS configuration (and also the LP-WUS monitoring) is deactivated.

However, it’s also possible that the UE would not apply LP-WUS configuration immediately on reception of LP-WUS configuration and the application of LP-WUS configuration will be activated later, e.g., per-UE via a dedicated signaling, e.g., RRC connection release message. During SI stage, such process has been discussed but no decision has been made.

The main motivation to activate/deactivate LP-WUS for a specific UE or part of UEs via dedicated signaling is that it may be not suitable to always activate the LP-WUS monitoring for all the UEs as soon as the LP-WUS configuration is broadcast. For example, LP-WUS can be applied to diverse UE types, e.g., IIoT UE, (e) Redcap UE, NR UE with XR traffic and so on, the device type or traffic type of UE may be another consideration for activating or deactivating LP-WUS for a UE. For example, the UEs with delay-sensitive services may not be suitable to monitor LP-WUS considering the possible large wake up latency caused by LP-WUS, which may have unexpected impacts on the delay-sensitive service. The exact latency that may be increased by LP-WUS is still under evaluation in RAN1, e.g., to take into account the LP-WUS processing time, MR transition time for ramp up, Time/frequency synchronization of MR etc. Per our understanding, if the additional latency caused by LP-WUS is far larger than the duration that random access procedure may use, e.g. if the increased latency is several seconds or more, the LP-WUS in IDLE/INACTIVE modes should be activated/deactivated by UE dedicated signaling, e.g., to activate LP-WUS only for the UE without delay-sensitive services or to deactivate LP-WUS when the UE starts to perform delay-sensitive services.

In a summary, from RAN2 perspective, we can see the need to activate/deactivate the application of LP-WUS configuration via dedicated signaling. But this can be later decided after the RAN1 has clear conclusion on the potential latency caused by LP-WUS monitoring.

Observation 1: LP-WUS monitoring may result in additional latency compared to legacy paging procedure and hence, it is useful to support per UE activation of LP-WUS monitoring in addition to the per cell configuration.

Proposal 2: In addition to the SIB signaling for LP-WUS configuration, dedicated activation/deactivation of LP-WUS per UE is supported.

If activation/deactivation of LP-WUS in IDLE/INACTIVE modes by dedicated signaling is supported, the gNB firstly needs to be aware of the UE capability of supporting LP-WUS in IDLE/INACTIVE modes and then can be able to activate/deactivate the LP-WUS for the UE by dedicated signaling.

Meanwhile, for the paging gNB, to only know the UE capability would be not sufficient as UE may not have been activated the use of LP-WUS, it also needs to know whether the LP-WUS is activated for this UE. Therefore, whether the LP-WUS monitoring has been activated for this UE (which can only be performed on UE supporting LP-WUS) needs to be transmitted to the paging gNB by Paging or RAN Paging.

Proposal 2a: If activation/deactivation of LP-WUS in IDLE/INACTIVE modes by UE dedicated signaling is supported, send LS to RAN3 to provide LP-WUS activation/deactivation indication to the paging gNB, e.g., via NGAP Signaling, NGAP PAGING and RAN PAGING message.

2.3 LP-WUS monitoring procedure and LP-WUS subgrouping

In RAN1#116 meeting [4], the following agreements have been achieved on the wake up procedure:

Agreement For the case where a UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not. Agreement It is supported that the UE monitors the legacy PO after receiving LP-WUS indicating wake-up. - FFS: support of UE monitoring dynamic PO Conclusion For idle/inactive mode, how to map a UE to a subgroup ID for LP-WUS is left to RAN2 to decide.

According to the RAN1 agreement, both “LP-WUS+ Paging” and “LP-WUS+PEI+paging” alternatives can be supported. Moreover, it can be seen that even though the PEI is configured, the UE may or may not monitor PEI. The result is that gNB cannot know whether a UE monitors PEI or not when the condition for UE to monitor PEI is met. Therefore, the gNB should send LP-WUS, PEI and paging when both the condition for UE to monitor LP-WUS and the condition for UE to monitor PEI are met.

Observation 2: Per the RAN1 agreements, gNB should send LP-WUS, PEI and paging when both the condition for UE to monitor LP-WUS and the condition for UE to monitor PEI are met.

In RAN2#128 meeting, the following agreements have been made for the UE_ID based LPWUS subgrouping [10]:

- FFS on the following options - Option 1: The subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping. - Option 2 The subgrouping number for UE_ID LPWUS subgrouping includes an offset K which is configurable or fixed. - Option 3: The formula for UE_ID based PEI subgrouping is reused.

For Option 1:

Figure 1 shows an example that PEI subgroup number is considered in UE ID based LP-WUS subgroup formula with assumption that PEI subgroup number is 4, and N*Ns =1. Based on which,  we can see that UEs in one LP-WUS are mapped to different PEI groups equally, thus PEI monitoring can decrease false wake-up furthermore, and can decrease UE power saving.

Figure 1: example that PEI subgroup number is considered in UE ID based subgroup

Observation 3: When PEI subgroup number is considered in the UE ID based LP-WUS subgroup formula, the UEs in one LP-WUS subgroup can be evenly mapped to different PEI subgroup.

For Option 2:

Figure 2(a) shows an example that an offset K=2 is included in the subgrouping number calculation as follows for UE_ID based LPWUS subgrouping also with PEI subgroup number = 4 and N*Ns=1.

SubgroupID = (floor(UE_ID/(N*Ns*K)) mod subgroupsNumForUEID_LP-WUS) + (subgroupsNumPerPO_LP-WUS - subgroupsNumForUEID_LP-WUS),

Figure 2a: example that an offset K=2 is included in the subgrouping number calculation for UE_ID based LPWUS subgrouping with PEI subgroup number = 4 and N*Ns=1

Based on which, we can see that UEs in one LP-WUS can only be mapped to some PEI groups (e.g. UEs in one LP-WUS subgroup can only be mapped to 2 subgroups).

Figure 2(b) shows another example that an offset K=5 is included in the subgrouping number calculation for UE_ID based LPWUS subgrouping also with assumption that PEI subgroup number is 5 and N*Ns=1 with the same formula as for figure 2a.

Figure 2(b): example that an offset K =5 is included in the subgrouping number calculation for UE_ID based LPWUS subgrouping with PEI subgroup number is 4 and N*Ns=1

Based on which, we can see that UEs in one LP-WUS can be mapped to different PEI groups. And with more UE IDs, we can see that UEs in one LP-WUS are mapped to different PEI groups equally.

Figure 2(c) shows an example that an offset K=8 is included in the subgrouping number calculation for UE_ID based LPWUS subgrouping also with assumption that PEI subgroup number is 5 and N*Ns=1 with the same formula as for figure 2a.

Figure 2(c): example that an offset K=8 is included in the subgrouping number calculation for UE_ID based LPWUS subgrouping with PEI subgroup number is 4 and N*Ns=1

Based on which, we can see that UEs in one LP-WUS can be mapped to different PEI groups equally.

With more subgrouping proof, we can see that when the value of offset K is less than the PEI subgroup number, the UEs in one LP-WUS may always be mapped to same PEI subgroup; when the value of offset K is larger than or equal to the PEI subgroup number, the UEs in one LP-WUS can be mapped to different PEI subgroups equally.

Observation 4: If offset K is considered in the UE ID based LP-WUS subgroup formula: when the value of offset K is less than the PEI subgroup number, the UEs in one LP-WUS may always be mapped to same PEI subgroup; when the value of offset K is larger than or equal to the PEI subgroup number, the UEs in one LP-WUS can be mapped to different PEI subgroups equally.

For Option 3:

Figure 3 shows an example that PEI subgroup number is not considered in UE ID based LP-WUS subgroup formula also with assumption that PEI subgroup number is 4. Based on which, we can see that UEs in one LP-WUS are always mapped to same PEI group, thus PEI monitoring cannot decrease false wake-up but increase UE power saving(e.g. it does not decrease false wake-up, but UE needs motoring PEI furthermore), which should be avoided.

Figure 3: example for that PEI subgroup number is not considered in UE ID based subgroup

Observation 5: When neither PEI subgroup number nor offset K is considered in the UE ID based LP-WUS subgroup formula, the UEs in one LP-WUS subgroup are always mapped to same PEI group in some cases(e.g. when the PEI subgroup number is a integer factor of the LP-WUS subgroup number).

For UE power saving, Option 1 or option 2 should be used in the UE ID based LP-WUS subgrouping formula. Considering the smaller the subgrouping number is, the more equalization that the UEs are distributed among subgroups (e.g. LP-WUS subgroups, PEI subgroups, and/or POs), if offset K is used to decide the UE-ID based LP-WUS subgrouping (Option 2), the smallest offset K should be used (e.g. the PEI subgroup number is used when the PEI monitoring condition is met, and 1 is used when the PEI monitoring condition is not met).

Proposal 3: Offset K is considered in the UE ID based LP-WUS subgroup formula, and the offset K equals to the PEI subgroup number when the PEI monitoring condition is met, and the offset K equals to 1 when the PEI monitoring condition is not met.

In the last meeting, some companies proposed that subgroupsNumPerPO defined for PEI is considered in the UE ID based LP-WUS subgroup, some companies proposed that subgroupsNumForUEID defined for PEI is considered in the UE ID based LP-WUS subgroup.Considering in the following cases, AMF may provide CN assigned PEI Subgroup ID but not provide LP-WUS Subgroup ID:

- Based on SA2 LS [11], the AMF PEIPS Assistance Information (e.g. CN assigned PEI Subgroup ID) is independent from the AMF LP-WUSPS Assistance Information (e.g. CN assigned LP-WUS Subgroup ID), AMF may provide CN assigned PEI Subgroup ID but not provide LP-WUS Subgroup ID.

- UEs supporting LP-WUS may access from a rel-18 AMF or early, in which case, AMF may provide CN assigned PEI Subgroup ID but not provide LP-WUS Subgroup ID.

When AMF provides CN assigned PEI Subgroup ID but not provide LP-WUS Subgroup ID, UE-ID based LP-WUS subgrouping but CN assigned PEI subgrouping will be used.

Observation 5: The UE monitoring UE ID based LP-WUS subgroup may monitor a CN assigned PEI or UE ID based PEI.

For less wake up by LP-WUS, RAN2 should try to allocate different LP-WUS IDs for UEs in same PEI group. Thus, when determining the UE-ID based LP-WUS subgrouping, subgroupsNumPerPO defined for PEI instead of subgroupsNumForUEID defined for PEI should be used.

Thus, the following formula can be used to determine the UE ID based LP-WUS subgroup in this case:

LP-WUS Subgroup ID = floor (UE_ID/(N*Ns*K)) mod subgroupsNumForUEIDofLPWUS) + subgroupsNumofLPWUS - subgroupsNumForUEIDofLPWUS

Where:

                - K: equals to the subgroupsNumPerPO defined for PEI when the condition for UE to monitor PEI is met and equals to 1 otherwise.

Proposal 4: The UE determines the UE ID based LP-WUS subgroup ID by the formula:

LP-WUS Subgroup ID = floor (UE_ID/(N*Ns*K)) mod subgroupsNumForUEIDofLPWUS) + subgroupsNumofLPWUS - subgroupsNumForUEIDofLPWUS

When the condition for UE to monitor PEI is met, K= subgroupsNumPerPO defined for PEI; else, K=1.

2.4 Time offset between LP-WUS and PO

As agreed in RAN1, UE reports one wake-up delay (e.g. one of candidate values) via UE capability reporting, and wake-up delay is minimum gap time between LP-WUS reception and MR to start PDCCH monitoring. If UE reports, it can implicitly indicate that UE supports LP-WUS in RRC_IDLE/RRC_INACTIVE state. From the network perspective, as agree in RAN1, time offset value(s) between a LO and a reference PO/PF is/are configured. In order to support the different values for wake up delay, network can configure different time offset values, and a time offset value is larger than or equal to a reported wake-up delay + PEI offset if PEI is configured. However, the different time offset values may cause the multiple LP-WUS occasions for same PO scattered over the time domain, too many LP-WUS occasions per PO is not convenient for LP-WUS resource scheduling, network can configure some time offsets that can be used. In which case, UE should determine the time offset based on its reported wake-up delay from the time offsets configured by network, e.g. the UE selects the minimum time offset configured by network that is larger than or equal to its reported wake-up delay. If there is no time offset configured by network that is larger than or equal to its reported wake-up delay, the UE does not monitor LP-WUS.

Proposal 5a: If UE report its wake-up delay, it can implicitly indicate that UE supports LP-WUS in RRC_IDLE/RRC_INACTIVE state.

Proposal 5b: Network configures time offset(s) that can be used between LO and MR PDCCH monitoring.

Proposal 5c: UE determine the time offset to be used based on its reported wake-up delay and the time offset(s) configured by network, e.g. select the minimal time offset configured by network that is larger than or equal to its reported wake-up delay. If there is no time offset configured by network that is larger than or equal to its reported wake-up delay, the UE does not monitor LP-WUS.

Considering that the LP-WUS time offset capability will be used for paging gNB to determine the LP-WUS occasion, the LP-WUS wake-up delay capability should be included in ue-RadioPagingInfo-r17 IE of UECapabilityInformation message, since the ue-RadioPagingInfo-r17 IE is already included in UERadioPagingInformation container(e.g. which is included in RAN3 IE of UE Radio Capability for Paging IE) and is transparently passed from gNB to AMF, from AMF to gNB and from one gNB to another gNB, the LP-WUS capability will not be lost if the gNB is rel-17 or later.

Proposal 6: The UE reported wake-up delay is included in ue-RadioPagingInfo-r17 IE of UECapabilityInformation message so that it can be delivered to paging gNB transparently.

2.5 About whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session

Whether LP_WUS can be used when the UE has an emergency PDU session depends on the increased delay caused by LP-WUS, which should be evaluated in RAN1 first. And if LP-WUS cannot be used for UE having an emergency PDU session, SA2 and RAN3 specification change will be necessary, for example. AMF should provide the emergency PDU session indication in CN paging, RAN paging and Core Network Assistance Information for RRC INACTIVE IE, and then RAN2 can specify that UE having an emergency PDU session will not monitor LP-WUS.

Observation 6: Whether LP_WUS can be used when the UE has an emergency PDU session depends on the increased delay caused by LP-WUS.

Proposal 7a: Send LS to RAN1 to evaluate the increased delay caused by LP-WUS.

Proposal 7b: RAN2 waits for RAN1’s conclusion on the increased delay caused by LP-WUS and SA2/RAN3 agreements, to decide whether RAN2 need to specify that UE having an emergency PDU session will not monitor LP-WUS.

2.6 About SA2 LS Reply on LP-WUS subgrouping

In the SA2 LS Reply on LP-WUS subgrouping [11], except conforming to use the existing CN based PEI subgrouping mechanism as the basis for the CN based LP-WUS subgrouping, SA2 has further provide two following issues:

1) 	SA2 noted that the name “LP-WUS” can be mis-understood, as SA2 understands that the Wake Up Signal is transmitted with high power to enable a large coverage area, and low power refers to a low power wake up receiver used in the mobile.

SA2 invites feedback on appropriate terminology and naming of the “LP-WUS” feature. SA2 will then align the terminology and name, based on the feedback.

Considering that the LP-WUS is the abbreviation of Low-Power Wake Up Signal, which is used for UE power saving based on low power OOK waveform with or without overlaid OFDM sequences, both the power of wake up signal and receiver are low. The coverage of LP-WUS may be smaller than the NR cell coverage, and RAN1 has already agreed the following LP-WUS monitoring condition:

Agreement Confirm the following working assumption with the addition of the note at the bottom: Working Assumption From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode, - The UE may start LP-WUS monitoring if - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

Since the terminology is defined by RAN1, and have common understanding in RAN, RAN2 can reply the LS to indicate:

RAN2 has discussed and confirmed the terminology of LP-WUS, which is the abbreviation of Low-Power Wake Up Signal. The coverage of LP-WUS may be smaller than the NR cell coverage, and RAN1 has already agreed that the UE may start LP-WUS monitoring only if the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB.

2)	Initial Registration may be performed on a pre-Rel 19 gNB (or a later gNB that does not support LP-WUS) that analyses the UE’s UE Radio Access Capabilities to create the UE Radio Capability for Paging Information and send it to the AMF, see clause 5.4.4.3 of TS23.501. It will be important that LP-WUS related aspects of UE Radio Capability for Paging Information are not lost when Initial Registration is performed on a pre-Rel-19 gNB. If the LP-WUS aspects are lost, then there can be paging problems, when for example, paging is performed by a gNB that supports LP-WUS.

SA2 invites RAN2 and RAN3 to investigate this and provides solution if any issue is identified.

SA2 was not sure whether this issue also impacts RedCap, eRedCap and other features that are signalled within the UE Radio Capability for Paging Information. SA2 invites RAN2 and RAN3 to investigate this.

To avoid that a gNB not supporting LP-WUS does not send the LP-WUS capability to AMF and/or other gNB, RAN2 can include the LP-WUS capability (e.g. AS capability) in ue-RadioPagingInfo-r17 IE of UECapabilityInformation message, the ue-RadioPagingInfo-r17 IE is already included in UERadioPagingInformation container (e.g. which is included in RAN3 IE of UE Radio Capability for Paging IE) and is transparently passed from gNB to AMF, from AMF to gNB and from one gNB to another gNB. Thus, the LP-WUS capability will not be lost if the gNB is rel-17 or later. But for early gNB before rel-17, it can only be based on NW implementation to avoid the LP-WUS capability lost issue, e.g. the gNBs in one tracking area and RAN paging area either all are gNB before rel-17, or all are gNBs of rel-17 or later. And RAN2 can confirm that the LP-WUS capability is independent from the other features(e.g. LP-WUS can be be supported with or without RedCap, eRedCap and other features that are signalled within the UE Radio Capability for Paging Information),  Thus, RAN2 can reply the LS to indicate:

RAN2 will include the LP-WUS capability(e.g. AS capability) in ue-RadioPagingInfo-r17 IE of UECapabilityInformation message, the ue-RadioPagingInfo-r17 IE is already included in UERadioPagingInformation container(e.g. which is included in RAN3 IE of UE Radio Capability for Paging IE) and is transparently passed from gNB to AMF, from AMF to gNB and from one gNB to another gNB. Thus, the LP-WUS capability will not be lost if the gNB is rel-17 or later.

RAN2 confirms that the LP-WUS capability is independent from the other features (e.g. can work with or without other features that are signalled within the UE Radio Capability for Paging Information).

Proposal 8:RAN2 reply the SA2 LS to indicate that:

For the issue 1):

RAN2 has discussed and confirmed the terminology of LP-WUS, which is the abbreviation of Low-Power Wake Up Signal. The coverage of LP-WUS may be smaller than the NR cell coverage, and RAN1 has already agreed that the UE may start LP-WUS monitoring only if the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB.

For the issue 2)

RAN2 will include the LP-WUS capability(e.g. AS capability) in ue-RadioPagingInfo-r17 IE of UECapabilityInformation message, the ue-RadioPagingInfo-r17 IE is already included in UERadioPagingInformation container(which is included in RAN3 IE of UE Radio Capability for Paging IE) and is transparently passed from gNB to AMF, from AMF to gNB and from one gNB to another gNB. Thus, the LP-WUS capability will not be lost if the gNB is rel-17 or later.

RAN2 confirms that the LP-WUS capability is independent from the other features (e.g. can work with or without other features that are signalled within the UE Radio Capability for Paging Information).

2.7 Separate band issue

LS from RAN1 to RAN2 and RAN4 [12]:

In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE.

RAN1 respectfully asks RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs.

RAN2 response [13]:

During RAN WG2 discussions, many companies raised concerns that if LP-Receivers support only subset of the bands supported by the main receiver and if LP-WUS, Paging and RACH are executed in the LP-WUS bands it might lead to overload of these bands if the NW set the LP-WUS frequency as a higher priority. This will impact user performance and decrease the utilisation of higher frequency bands. In order to ensure the LP-WUS capable UEs are camping on the band LR implemented, the NW would need to offload LP capable UEs to LR band.

On the other hand if the NW operator does not offload the UEs to the band LP receiver supports, only a subset of UEs would profit from LP-WUS feature.

Some companies do not see that overload problem would occur as the NW implementation can control the offload of the UEs to the lower bands in dedicated way.

In regards to the overload problem, RAN WG2 has not finished evaluations and would need to study the solutions more.

RAN4 Agreement [14]:

RAN4 will further discuss whether any technical issues regarding the LS from RAN1.

From Rel-19 RAN4 RRM requirement of MR offloading and relaxation perspective, RAN4 assumed LR and MR are operating on the same carrier frequency as baseline. FFS for the case of MR and LR working on different carrier frequencies if it is supported in RAN1/2.

Issue for further discussion based on contribution driven:

For WUS monitoring assumption, further discuss whether or not the same carrier frequency is assumed for LR and MR.]

In RAN#106, we have agreed the following way forward for this issue [15]:

For idle/inactive mode, consider a standardized solution for intra-frequency case (e.g. solution 1, 2, and 3 in RP-242656).  For Rel-19, inter-frequency solutions (e.g. solution 4) will not be considered.

Detailed discussions and final solution selection is left up to WGs.

And the solutions discussed in RAN#106 are as follows:

- Solution 1: MR is prioritized (via SIBs) to camp on the same band as LR (System information, Paging, RACH on LR band)

- Solution 2: MR is prioritized to camp on the same band as LR (System information, Paging on LR band; and RACH on another MR band based on reselection criteria)

- Solution 3:  MR is not prioritized to camp on the same band as LR but can be redirected via dedicated signaling depending on LR band load (up to gNB implementation)

- Solution: 4- MR (SI/Paging/RACH) is camped on a different band than LR. LR can wake up MR on the band it is camped on

For the case when UE support LP-WUS reception in a subset of bands, we need to down select from solution 1/2/3 as solution 4 has been excluded by RANP due to complexity.

Among all the bands supported by UE, there would be two kinds of bands:

- Bands supporting both MR and LR

- Bands supporting MR only

For solution 2, UE acquire system information, receive LP-WUS and paging in one band (MR+LR), perform cell reselect to another cell (MR only) and get access. To support this, we need to introduce new trigger for cell reselection, e.g. traffic triggered cell reselection.

For solution 3, UE acquire system information, receive LP-WUS, paging and get access in one band (MR+LR) and get released and redirected or handover to another cell (MR only).

Solution 2 and 3 allows UE to receive LP-WUS in a band supporting both MR and LR but reselect or be redirected to another band supporting MR only for the follow up access. More delay will be introduced for the band/cell switch in both solutions while Solution 2 requires extra spec impact and change in the UE behavior as cell reselection is mainly to ensure UE to stay in the best cell

While for solution 1, the only concern is the potential overload issue, which is not that serious from our perspective as there would be limited LP-WUS UEs and only those in connected state with traffic can bring network load while those in idle/inactive would not trouble the network a lot.

With the above analysis, we understand that solution 1 can be adopted as the baseline solution for the case that UE support LP-WUS reception in a subset of bands among all the bands supported by UE while solution 3 can be the last resort conducted by network if overloaded.

Proposal 9: For the case that UE support LP-WUS reception in a subset of bands among all the bands supported by UE, both solution 1 and solution 3 are supported. It is up to network to prioritize frequencies supporting both MR and LR, i.e. solution 1, and/or redirect UE to a frequency supporting MR only if frequencies supporting both MR and LR are overloaded, i.e. solution 3.

- Conclusion

The following proposals are given based on the above analysis:

Provision of LP-WUS related configuration

Proposal 1: RAN2 can postpone the discussion on LP-WUS and LP-SS related configuration until there is agreement for the LP-SS and LP-WUS design in RAN1.

Activation/deactivation of LP-WUS

Observation 1: LP-WUS monitoring may result in additional latency compared to legacy paging procedure and hence, it is useful to support per UE activation of LP-WUS monitoring in addition to the per cell configuration.

Proposal 2: In addition to the SIB signaling for LP-WUS configuration, dedicated activation/deactivation of LP-WUS per UE is supported.

Proposal 2a: If activation/deactivation of LP-WUS in IDLE/INACTIVE modes by UE dedicated signaling is supported, send LS to RAN3 to provide LP-WUS activation/deactivation indication to the paging gNB, e.g., via NGAP Signaling, NGAP PAGING and RAN PAGING message.

LP-WUS monitoring procedure and LP-WUS subgrouping

Observation 2: Per the RAN1 agreements, gNB should send LP-WUS, PEI and paging when both the condition for UE to monitor LP-WUS and the condition for UE to monitor PEI are met.

Observation 3: When PEI subgroup number is considered in the UE ID based LP-WUS subgroup formula, the UEs in one LP-WUS subgroup can be evenly mapped to different PEI subgroup.

Observation 4: If offset K is considered in the UE ID based LP-WUS subgroup formula: when the value of offset K is less than the PEI subgroup number, the UEs in one LP-WUS may always be mapped to same PEI subgroup; when the value of offset K is larger than or equal to the PEI subgroup number, the UEs in one LP-WUS can be mapped to different PEI subgroups equally.

Observation 5: When neither PEI subgroup number nor offset K is considered in the UE ID based LP-WUS subgroup formula, the UEs in one LP-WUS subgroup are always mapped to same PEI group in some cases(e.g. when the PEI subgroup number is a integer factor of the LP-WUS subgroup number).

Proposal 3: Offset K is considered in the UE ID based LP-WUS subgroup formula, and the offset K equals to the PEI subgroup number when the PEI monitoring condition is met, and the offset K equals to 1 when the PEI monitoring condition is not met.

Observation 5: The UE monitoring UE ID based LP-WUS subgroup may monitor a CN assigned PEI or UE ID based PEI subsequently.

Proposal 4: The UE determines the UE ID based LP-WUS subgroup ID by the formula:

LP-WUS Subgroup ID = floor (UE_ID/(N*Ns*K)) mod subgroupsNumForUEIDofLPWUS) + subgroupsNumofLPWUS - subgroupsNumForUEIDofLPWUS

When the condition for UE to monitor PEI is met, K= subgroupsNumPerPO defined for PEI; else, K=1.

Time offset between LP-WUS and PO

Proposal 5a: If UE report its wake-up delay, it can implicitly indicate that UE supports LP-WUS in RRC_IDLE/RRC_INACTIVE state.

Proposal 5b: Network configures time offset(s) that can be used between LO and MR PDCCH monitoring.

Proposal 5c: UE determine the time offset to be used based on its reported wake-up delay and the time offset(s) configured by network, e.g. select the minimal time offset configured by network that is larger than or equal to its reported wake-up delay. If there is no time offset configured by network that is larger than or equal to its reported wake-up delay, the UE does not monitor LP-WUS.

Proposal 6: The UE reported wake-up delay is included in ue-RadioPagingInfo-r17 IE of UECapabilityInformation message， so that it can be delivered to paging gNB transparently.

About whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session

Observation 6: Whether LP_WUS can be used when the UE has an emergency PDU session depends on the increased delay caused by LP-WUS.

Proposal 7a: Send LS to RAN1 to evaluate the increased delay caused by LP-WUS.

Proposal 7b: RAN2 waits for RAN1’s conclusion on the increased delay caused by LP-WUS, to decide whether LP_WUS can be used when the UE has an emergency PDU session.

About SA2 LS Reply on LP-WUS subgrouping

Proposal 8:RAN2 reply the SA2 LS to indicate that:

For the issue 1):

RAN2 has discussed and confirmed the terminology of LP-WUS, which is the abbreviation of Low-Power Wake Up Signal. The coverage of LP-WUS may be smaller than the NR cell coverage, and RAN1 has already agreed that the UE may start LP-WUS monitoring only if the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB.

For the issue 2)

RAN2 will include the LP-WUS capability(e.g. AS capability) in ue-RadioPagingInfo-r17 IE of UECapabilityInformation message, the ue-RadioPagingInfo-r17 IE is already included in UERadioPagingInformation container(which is included in RAN3 IE of UE Radio Capability for Paging IE) and is transparently passed from gNB to AMF, from AMF to gNB and from one gNB to another gNB. Thus, the LP-WUS capability will not be lost if the gNB is rel-17 or later.

RAN2 confirms that the LP-WUS capability is independent from the other features (e.g. can work with or without other features that are signalled within the UE Radio Capability for Paging Information).

Separate band issue

Proposal 9: For the case that UE support LP-WUS reception in a subset of bands among all the bands supported by UE, both solution 1 and solution 3 are supported. It is up to network to prioritize frequencies supporting both MR and LR, i.e. solution 1, and/or redirect UE to a frequency supporting MR only if frequencies supporting both MR and LR are overloaded, i.e. solution 3.

- References

- RP-241824 Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO, RAN#105, March, 2024

- Draft_R2_125bis_Meeting_Report_v1, RAN2#125bis, April, 2024

- Draft_R2_126_Meeting_Report_v1, RAN2#126, May, 2024

- Draft_Minutes_report_RAN1#116_v020-202402, RAN1#116, February, 2024

- Draft_Minutes_report_RAN1#116b_v030, RAN1#116bis, April, 2024

- Draft_Minutes_report_RAN1#117_v010, RAN1#117, February, 2024

- Draft_RAN2_127_Meeting_Report_v1

- Draft_Minutes_report_RAN1#118_v010, RAN1#118, August, 2024

- Draft_RAN2_127b_Meeting_Report_v1

- Draft_RAN2_128_Meeting_Report_v1

- S2-2412876, LS Reply on LP-WUS subgrouping

- R2-2407921 LS on LP-WUS operation in IDLE/INACTIVE mode

- R2-2410957 Reply LS to R2-2407921/ R1-2407559

- R4-2416861 WF on RRM requirements for NR_LPWUS, RAN4 112bis, vivo

- RP‑243266	Way forward on Rel-19 LP-WUS separate band issues