3GPP TSG-RAN WG2 Meeting #129										    R2-2501090

Athens, Greece, Feb. 17th – 21st, 2025                                        Revision of R2-2410167

Agenda item:		8.4.3

Title: 	RRM measurement relaxation and offloading in RRC_IDLE and RRC_INACTIVE mode

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

In this contribution, we will discuss the RAN2 related issue on RRM measurement relaxation and offloading for IDLE/INACTIVE modes and give our proposals.

- Discussion

Based on the RAN2#126 agreement that “RAN2 understand that the RRM measurement of the neighbouring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise”, it can be understood that LP-WUR can only be used for serving cell measurement.

Considering that serving cell measurement offloading means that UE will only perform the serving cell measurement based on LP-WUR, and the main radio can be turn off for UE power saving, which has also been agreed in RAN4#111, it can be understood that serving cell measurement offloading means that UE will only perform the serving cell measurement based on LP-WUR, and stop MR measurement (e.g. both the serving sell measurement based on MR and neighbor cell measurement are stopped).

Proposal 1: RAN2 confirms that serving cell measurement offloading means that UE will only perform the serving cell measurement based on LP-WUR, and stop MR measurement for both serving cell and neighbor cell.

Since RAN4 has agreed to support the RRM measurement relaxation, e.g. the following case#3, RAN2 has agreed that RRM measurement of the neighbouring cell can only be performed by MR, it can be understood that MR serving cell measurement can be relaxed(e.g. use larger measurement cycle than legacy) when LP-WUR measurement is started. And with the LP-WUR measurement aided, same measurement accuracy can be obtained although the MR serving cell measurement is relaxed.

| RRM measurement case index | MR serving cell measurement | MR neighbouring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

Proposal 2: RAN2 confirms that serving cell measurement relaxation means that UE performs serving cell measurement based on MR with larger measurement cycle than legacy, while perform serving cell measurement based on LP-WUR simultaneously.

  - Serving cell measurement relaxation

Based on the current agreement in RAN2 that the RRM measurement of the neighbour cell can only be performed by MR, it can be assumed that RRM measurement relaxation is to relax the serving cell measurement when the LP-SS based measurement is performed. In NB-IoT and eMTC, serving cell measurement relaxation is supported, e.g. NW can configure numDRX-CyclesRelaxed in SIB, and the UE supporting WUS shall relax serving cell measurement requirement defined with the DRX cycle from the DRX cycle length of DRX cycle to the DRX cycle length of min (DRX cycle * numDRX-CyclesRelaxed, 10.24s), the similar mechanism can be used for NR serving cell measurement relaxation.

Proposal 3: The NB-IoT serving cell measurement relaxation mechanism is used as the baseline for NR serving cell measurement relaxation, e.g. NW can configure numDRX-CyclesRelaxed in SIB, and the NR UE supporting LP-SS based measurement may relax serving cell measurement requirement from M1*N1*DRX cycle to min( M1*N1*DRX cycle * numDRX-CyclesRelaxed, 10.24s).

In RAN2#128 meeting[7], RAN2 has agreed that the entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. Considering that serving cell measurement relaxation can be performed only when the LP-SS based measurement is executed, the entry condition for serving cell RRM relaxation should also include LR measurement result, e.g. the LP-RSRP and/or LP-RSRQ is higher than a relaxation threshold, if configured.

Proposal 4: The entry condition for serving cell RRM relaxation should also include LR measurement result, e.g. the LP-RSRP and/or LP-RSRQ should be higher than a relaxation threshold, if configured.

- For UE with OOK-based WUR, both of the RSRP measured by MR and OOK-based WUR of the serving cell are larger than an RSRP threshold and optionally both of the RSRQ measured by MR and OOK-based WUR  of the serving cell is larger than an RSRQ threshold (if configured), the NR serving cell measurement relaxation can be performed.

- for UE with OFDM-based WUR, both of the RSRP measured by MR and OFDM-based WUR of the serving cell is larger than an RSRP threshold and optionally both of the RSRQ measured by MR and OFDM-based WUR  of the serving cell is larger than an RSRQ threshold (if configured), the NR serving cell measurement relaxation can be performed.

  - Neighbor cell measurement relaxation

Since RAN2 understands that the RRM measurement of the neighboring cell can only be performed by MR, the neighboring cell measurement accuracy has not changed when introducing LP-WUS, thus the neighbor cell measurement relaxation should not be changed, e.g. the legacy Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’ can reused for neighbour cell measurement relaxation.

Proposal 5a: The legacy Rel-16 criteria for ‘low mobility’ is reused for neighbour cell measurement relaxation.

Since the RRM measurement of the neighboring cell can only be performed by MR, when UE perform neighboring cell measurement relaxation, the MR is still in “on” state, at most serving cell measurement relaxation can be performed, and serving sell measurement offloading should not be triggered. In which case, UE can always obtain the serving cell’s MR measurement results. Whether only the serving cell’s MR measurement results or combination of serving cell’s MR measurement results and LR measurement results is used for cell reselection evaluation depends on RAN1 and/or RAN4 conclusion.

Proposal 5b: RAN2 confirms that when UE performs neighboring cell measurement relaxation, the MR is still in “on” state, e.g. at most serving cell measurement relaxation can be performed, and serving sell measurement offloading should not be triggered.

Proposal 5c: When UE performs neighboring cell measurement relaxation, whether only the serving cell’s MR measurement results or combination of serving cell’s MR measurement results and LR measurement results is used for cell reselection evaluation depends on RAN1 and/or RAN4 conclusion.

  - Serving cell measurement offloading

In order to achieve the UE power saving gain by LP-WUS/WUR, the RRM measurement on serving cell and neighbouring cell via MR may be skipped when UE is using LP-WUS or MR is in ultra-deep sleep. However, UE still needs to ensure that IDLE/INACTIVE mode measurements are possible during this state. In order to avoid or minimize the performance degradation of mobility handling in RRC_IDLE or RRC_INACTIVE when MR is stopped, RAN2 has already agreed in SI stage [2] that: ultra-deep sleep state could be entered when the serving cell quality of both LP-SS and MR SSB is larger than a threshold, e.g. UE is not at cell edge. As legacy, the relaxed measurement criterion for UE not at cell edge [3] can be used to decide whether the UE is at cell edge. Whether a threshold is separately defined for LP-SS and MR SS can be further considered, e.g. depending on the measurement performance of LP-WUS defined by RAN4.

-------------------------------Relaxed measurement criterion for UE not at cell edge in 38.304------------------------------

The relaxed measurement criterion for UE not at cell edge is fulfilled when:

-	Srxlev > SSearchThresholdP, and,

-	Squal > SSearchThresholdQ, if SSearchThresholdQ is configured,

Where:

-	Srxlev = current Srxlev value of the serving cell (dB).

-	Squal = current Squal value of the serving cell (dB).

-------------------------------Relaxed measurement criterion for UE not at cell edge in 38.304------------------------------

Proposal 7a: RAN2 confirms that the relaxed measurement criterion for UE not at cell edge (e.g. based on (LP-) RSRP value, and optional (LP-) RSRQ value) is used for UE to decide whether to enter or exit MR serving cell measurement offloading for UE in IDLE/INACTIVE modes.

Furthermore, some UEs may move quickly, and the resumption of Main Radio may require longer duration (e.g. several seconds), the serving cell measurement relaxation of UE MR based on LP-WUR should consider UE mobility, e.g. only the UEs with low mobility can apply the serving cell measurement relaxation of UE MR. As legacy, the Relaxed measurement criterion for UE with low mobility [3] can be used to decide whether the UE is in low mobility.

------------------------------Relaxed measurement criterion for UE with low mobility in 38.304------------------------------

The relaxed measurement criterion for UE with low mobility is fulfilled when:

-	(SrxlevRef – Srxlev) < SSearchDeltaP,

Where:

-	Srxlev = current Srxlev value of the serving cell (dB).

-	SrxlevRef = reference Srxlev value of the serving cell (dB), set as follows:

-	After selecting or reselecting a new cell, or

-	If (Srxlev - SrxlevRef) > 0, or

-	If the relaxed measurement criterion has not been met for TSearchDeltaP:

-	The UE shall set the value of SrxlevRef to the current Srxlev value of the serving cell.

------------------------------Relaxed measurement criterion for UE with low mobility in 38.304------------------------------

Proposal 7b: The relaxed measurement criterion for UE with low mobility (e.g. based on (LP-) RSRP change) is reused for UE to decide whether to enter or exit MR serving cell measurement offloading for UE in IDLE/INACTIVE mode.

- Case 1: UE has not perform MR serving cell measurement offloading and may perform MR serving cell measurement on MR SSB and LP-SS

When the UE is not performing MR serving cell measurement offloading, the UE may perform RRM measurements based on MR SSS and LP-SS, the UE can decide whether it is in low mobility state and not at cell edge based on MR SSS and/or LP-SS change (e.g. based on (LP) RSRP change, (LP) RSRP value, optionally LP-RSRQ value).

Proposal 8a: When the UE is not performing MR serving cell measurement offloading, serving cell quality change and quality based on MR SSB and LP-SS measurement can be used to determine whether to enter MR serving cell measurement offloading for UE in IDLE/INACTIVE mode.

- Case 2: UE has performed MR serving cell measurement offloading and perform MR serving cell measurement only on LP-SS

While the UE performing MR serving cell measurement offloading, UE perform serving cell measurement based on LP-SS only. If the UE determines that it is neither in stationary state nor is at the cell edge based on the LP-SS measurement, the UE starts to perform serving cell measurements based on main radio (e.g. MR SSB).

Proposal 8b: The UE performing MR RRM measurement offloading should resume measurements based on MR for serving cell measurements when the UE is neither in low mobility state nor at the cell edge.

As legacy, the Relaxed measurement criterion for UE with low mobility (e.g. the LP-SS RSRP change value is less than a threshold) can be used to decide whether the UE is in low mobility, and relaxed measurement criterion for UE not at cell edge (e.g. the LP-SS RSRP value is lower than an RSRP threshold and the LP-SS RSRQ value is lower than an RSRQ threshold, if configured) can be used to decide whether the UE is at the cell edge.

Proposal 8c: When the UE is performing serving cell measurement based on LP-SS only, whether it is in low mobility state is determined based on whether LP-SS RSRP change is less than a threshold, and whether it is at the cell edge is determined based on whether the LP-SS RSRP value is lower than a threshold and optionally the LP-RSRQ value is lower than an RSRQ threshold, if configured.

However, if the entry conditions for serving cell measurement offloading is satisfied, e.g. in low mobility state or not in the cell age, under some special case, UE couldn’t perform serving cell measurement offloading, e.g. the serving cell measurement offloading can be performed only if there is neither a NR inter-frequency nor a NR inter-RAT frequency with a reselection priority higher than the reselection priority of the current NR frequency.

Proposal 9: The serving cell measurement offloading can be performed only if there is neither a NR inter-frequency nor a NR inter-RAT frequency with a reselection priority higher than the reselection priority of the current NR frequency.

- Conclusion

Proposal 1: RAN2 confirms that serving cell measurement offloading means that UE will only perform the serving cell measurement based on LP-WUR, and stop MR measurement for both serving cell and neighbor cell.

Proposal 2: RAN2 confirms that serving cell measurement relaxation means that UE performs serving cell measurement based on MR with larger measurement cycle than legacy, while perform serving cell measurement based on LP-WUR simultaneously.

Serving cell measurement relaxation

Proposal 3: The NB-IoT serving cell measurement relaxation mechanism is used as the baseline for NR serving cell measurement relaxation, e.g. NW can configure numDRX-CyclesRelaxed in SIB, and the NR UE supporting LP-SS based measurement may relax serving cell measurement requirement from M1*N1*DRX cycle to min( M1*N1*DRX cycle * numDRX-CyclesRelaxed, 10.24s).

Proposal 4: The entry condition for serving cell RRM relaxation should also include LR measurement result, e.g. the LP-RSRP and/or LP-RSRQ should be higher than a relaxation threshold, if configured.

- For UE with OOK-based WUR, both of the RSRP measured by MR and OOK-based WUR of the serving cell are larger than an RSRP threshold and optionally both of the RSRQ measured by MR and OOK-based WUR  of the serving cell is larger than an RSRQ threshold (if configured), the NR serving cell measurement relaxation can be performed.

- For UE with OFDM-based WUR, both of the RSRP measured by MR and OFDM-based WUR of the serving cell is larger than an RSRP threshold and optionally both of the RSRQ measured by MR and OFDM-based WUR  of the serving cell is larger than an RSRQ threshold (if configured), the NR serving cell measurement relaxation can be performed.

Neighbor cell measurement relaxation

Proposal 5a: The legacy Rel-16 criteria for ‘low mobility’ is reused for neighbor cell measurement relaxation.

Proposal 5b: RAN2 confirms that when UE performs neighboring cell measurement relaxation, the MR is still in “on” state, e.g. at most serving cell measurement relaxation can be performed, and serving sell measurement offloading should not be triggered.

Proposal 5c: When UE performs neighboring cell measurement relaxation, whether only the serving cell’s MR measurement results or combination of serving cell’s MR measurement results and LR measurement results is used for cell reselection evaluation depends on RAN1 and/or RAN4 conclusion.

Serving cell measurement offloading

Proposal 7a: RAN2 confirms that the relaxed measurement criterion for UE not at cell edge (e.g. based on (LP-) RSRP value, and optional (LP-) RSRQ value) is used for UE to decide whether to enter or exit MR serving cell measurement offloading for UE in IDLE/INACTIVE modes.

Proposal 7b: The relaxed measurement criterion for UE with low mobility (e.g. based on (LP-) RSRP change) is reused for UE to decide whether to enter or exit MR serving cell measurement offloading for UE in IDLE/INACTIVE mode.

Proposal 8a: When the UE is not performing MR serving cell measurement offloading, serving cell quality change and quality based on MR SSB and LP-SS measurement can be used to determine whether to enter MR serving cell measurement offloading for UE in IDLE/INACTIVE mode.

Proposal 8b: The UE performing MR RRM measurement offloading should resume measurements based on MR for serving cell measurements when the UE is neither in low mobility state nor at the cell edge.

Proposal 8c: When the UE is performing serving cell measurement based on LP-SS only, whether it is in low mobility state is determined based on whether LP-SS RSRP change is less than a threshold, and whether it is at the cell edge is determined based on whether the LP-SS RSRP value is lower than a threshold and optionally the LP-RSRQ value is lower than an RSRQ threshold, if configured.

Proposal 9: The serving cell measurement offloading can be performed only if there is neither a NR inter-frequency nor a NR inter-RAT frequency with a reselection priority higher than the reselection priority of the current NR frequency.

- References

- RP-240801 Revised WID Low-power wake-up signal and receiver for NR (LP-WUS_WUR) , RAN#103

- Draft_R2_126_Meeting_Report_v1, RAN2#126, May, 2024

- 3GPP TR 38.869 V18.0.0 (2023-12), Study on low-power wake-up signal and receiver for NR

- 3GPP TR 38.304 V18.1.0 (2024-03), NR;User Equipment (UE) procedures in Idle mode and RRC Inactive state

- Draft_RAN2_127_Meeting_Report_v1

- Draft_RAN2_127b_Meeting_Report_v1

- Draft_RAN2_128_Meeting_Report_v1