3GPP TSG-RAN2 Meeting #129	R2-2501095

Athens, Greece, 17 - 21 February 2025	Revision of R2-2410087

Agenda Item:	8.4.4 Procedures for LP-WUS in RRC_CONNECTED

Source: 	Ericsson

Title:  	LP-WUS in Connected

Document for:	Discussion and Decision

# Introduction

In this contribution LP-WUS in connected mode is discussed further.

For the RAN1 and RAN2 agreements related the reader is referred to the Annex.

# Discussion

## Overview

The following options are discussed in RAN1 and RAN2:

Option 1-1 (DCP like):

- UE monitors LP-WUS before the drx-OnDurationTimer.

- When LP-WUS is detected the UE starts drx-OnDurationTimer and enters Active Time.

- UE can be configured to report periodic CSI/ L1-RSRP when LP-WUS is not detected.

Option 1-2:

- UE monitors LP-WUS periodically outside Active Time.

- When LP-WUS is detected the UE starts a new timer and enters Active Time.

- UE can be configured to report periodic CSI/ L1-RSRP during the OnDuration period when LP-WUS is not detected.

Option 1-3:

- UE monitors LP-WUS periodically inside Active Time.

- MAC is not aware of the LP-WUS monitoring in L1.

- FFS details and interaction with PDCCH skipping/adaptation, if any.

## RAN1 LS

With the DRX command the gNB can make the UE stop drx-onDurationTimer and drx-InactivityTimer, if running, and force the UE to go into short or long DRX.

RAN1 asked whether the DRX commands, i.e. DRX Command MAC CE and Long DRX Command MAC CE, can be used when option 1-1 or 1-2 is configured [1].

A DRX command used with option 1-1 stops drx-onDurationTimer and drx-InactivityTimer, if running. And the UE would start monitoring LP-WUS outside Active Time.

A DRX command used with option 1-2 stops lpwus-OnDurationTimer and drx-InactivityTimer, if running. And the UE would start monitoring LP-WUS outside Active Time.

(Long) DRX command MAC CE can be used with option 1-1 and 1-2 to stop drx-onDurationTimer (option 1-1), lpwus-OnDurationTimer (option 1-2) and drx-InactivityTimer. When outside Active Time the UE starts monitoring LP-WUS.

## Option 1-2

Both RAN1 and RAN2 agreed that with option 1-2:

- a new timer is started when LP-WUS is detected

- option 1-1 and option 1-2 are not configured together

- other DRX parameters are not affected, e.g. when there is new UL/DL transmission the UE starts drx-InactivityTimer.

In this contribution the new timer is denoted lpwus-OnDurationTimer. Similar as with the legacy drx-onDurationTimer when lpwus-OnDurationTimer is started the UE enters Active Time:

With option 1-2 when the UE monitors LP-WUS outside Active Time and LP-WUS is detected then the UE enters Active Time and starts monitoring PDCCH.

RAN1 agreed that option 1-2 is enabled/disabled using RRC signalling and it is FFS whether there are additional conditions or signalling for activation and deactivation:

Agreement

For RRC CONNECTED mode, from RAN1 perspective,

- PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling

  - FFS whether to support UE assistance.

- LP-WUS monitoring by UE is known to gNB.

  - FFS whether implicit/explicit indication from UE is necessary

- In case LP-WUS monitoring is enabled, following options are further studied

  - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring

  - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance.

  - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer.

  - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

RAN2 can confirm that LP-WUS is enabled/disabled using RRC signalling:

RAN2 confirms that option 1-2 is enabled/disabled using RRC signalling.

The LP-WUS configuration, including the lpwus-OnDurationTimer DRX parameter, is included in a new lpwus-Config-r19 IE in PhysicalCellGroupConfig IE in the RRCReconfiguration message (similar as the dcp-Config-r16 IE):

The LP-WUS configuration is included in a new lpwus-Config-r19 IE in PhysicalCellGroupConfig IE in RRCReconfiguration message. Network only configures lpwus-Config-r19 when drx-Config is configured.

RAN1 agreed that with option 1-1 and 1-2 there is no impact on RRM/RLM/BFD measurements, and that similar as with DCP, the UE can be configured to report periodic CSI/L1-RSRP when LP-WUS is not detected. This means that while there is no traffic, i.e. the UE is outside Active Time and LP-WUS is not detected, then the UE still has to measure RLM/BFD every C-DRX cycle using MR. This is however less power consuming than when periodic CSI/L1-RSRP reporting is configured and the UE has to transmit in UL. Periodic CSI/L1-RSRP reporting significantly reduces the power saving gains of LP-WUS. Instead of periodic reporting every C-DRX cycle, using LP-WUS the NW can request aperiodic CSI/L1-RSRP less often:

LP-WUS provides limited power saving gains in connected, i.e. even there is no traffic, the MR still has to measure RLM/BFD every C-DRX and report periodic CSI/L1-RSRP, if configured.

Option 1-1 provides some power saving gains and option 1-2 does not provide further power savings when the same C-DRX cycle is configured as with option 1-1. But option 1-2 provides significant DL latency reduction for the initial DL packets when a short LP-WUS periodicity is configured. With option 1-1 the latency can also be reduced by configuring a short C-DRX cycle. But configuration of a short LP-WUS periodicity with option 1-2 is less power consuming compared to configuration of a short C-DRX cycle with option 1-1. In the latter case the UE has to measure RLM/BFD more frequently which increases the power consumption.

The specification impact is reduced when option 1-1 is considered as a special configuration/case of option 1-2, i.e. when the LP-WUS periodicity equals the C-DRX cycle length, then option 1-2 equals option 1-1:

The LP-WUS periodicity with option 1-2 can be equal to the C-DRX cycle length. The LP-WUS periodicity is larger or equal to the time offset.

During Rel-16 the support of DCP with short DRX was discussed in both RAN1 and RAN2. RAN1 did not have time to finish their work and DCP is not supported with short DRX. There is no RAN2 impact to support LP-WUS with short DRX:

From RAN2 perspective LP-WUS can be configured with short DRX.

With DCP the UE is required to start the drx-OnDurationTimer when the UE is not able to monitor the DCP, e.g. when the time between the end of the Active Time is shorter than the DCP time offset. Or when the UE is not able monitor DCP because it has to perform other activities (e.g. during BWP switching, measurement gaps or when monitoring CFRA response during beam failure recovery). The same principle should be applied with option 1-1. However further discussion is needed with option 1-2, because when the LP-WUS periodicity is very short, and the UE is not able to monitor LP-WUS in one of the occasions then very soon the UE will be able to monitor LP-WUS in the next occasion again:

When the LP-WUS periodicity equals the C-DRX cycle length and the UE is not able to monitor the LP-WUS occasion then the UE starts lpwus-OnDurationTimer. FFS when a smaller LP-WUS periodicity is configured and the UE is not able to monitor one of the LP-WUS occasions.

Similar as with the DCP configuration a parameter ps-WakeUp is useful for improved reliability and robustness:

The UE can be configured (e.g. lpwus-WakeUp) to start lpwus-OnDurationTimer when the UE is able to monitor LP-WUS but LP-WUS is not detected outside Active Time.

## Minimum time gap and preferred time offset

RAN1 is still discussing the details of minimum time gap and preferred time offset:

- UE signals one value per SCS for the minimum time gap in UE capability.

- UE can indicate a preferred time offset via UAI signalling

- FFS whether there is a single time offset for the UE at any time.

With DCP the UE can signal a minimum time gap (in slots) per SCS supported in FR1, FR2-1 and FR2-2 in licensed and unlicensed spectrum (see drx-Adaptation-r16 and drx-Adaptation-r17).

With LP-WUS the time offset refers to the time between LP-WUS reception by the LR and start of PDCCH monitoring by the MR.

When the UE is configured with multiple time offsets, and it is left to UE implementation which time offset to use, the network resources are wasted. Therefore it is proposed:

The gNB configures a single time offset which is equal or longer than the minimum time gap signaled in the LP-WUS UE capability.

The motivation to signal a preferred time offset via UAI is to enable a different sleep state in the UE e.g. light or deep sleep:

|  | Relative Power (unit) | Transition time (ms) | Transition energy (ms) |
| --- | --- | --- | --- |
| Micro sleep | 45 | 0 | 0 |
| Light Sleep | 20 | 6 | 100 |
| Deep Sleep | 1 | 20 | 450 |

If configured the UE can report a preferred time offset, i.e. try to change the configured offset:

If configured, the UE can signal a preferred time offset via UAI signalling.

RAN1 indicated that for Idle and Inactive the UE may not support LP-WUS on all the supported bands [3]. The DCP capability is per UE (see drx-Adaptation-r16 and drx-Adaptation-r17). RAN2 should discuss and clarify whether the UE in connected mode may not support LP-WUS on all supported bands:

If the UE may not support LP-WUS in connected mode on all supported bands, then the LP-WUS UE capability for connected mode includes a supportedBandListNR IE.

## CA, secondary DRX and DC

RAN1 agreed that LP-WUS is supported with both CA and DC. It is FFS for which cells LP-WUS is applicable. RAN plenary confirmed that FR2 is within the scope of the LP-WUS WID [1,2]. This then also implies that LP-WUS with secondary DRX can be evaluated.

The UE is typically configured with CA to provide the highest throughput and shortest latency. Based on RRM measurement reporting and load balancing the CA configuration may change using RRCReconfiguration, i.e. carriers can be added or removed. Based on traffic demand and to save power SCells may be deactivated or enter dormant state (dormant BWP is used).

The DRX configuration applies to all the cells of the Cell Group (CA/DC) or DRX group (secondary DRX), i.e. all the cells are in the same DRX state (e.g. Active Time) and the same DRX parameter applies to all the cells of the Cell/DRX Group, e.g. one drx-OnDurationTimer running for the Cell/DRX Group. The scheduler uses the full capacity provided by the Cell/DRX group to schedule traffic.

With DC and secondary DRX the Cell and DRX groups can be in different DRX states e.g. MCG is inside Active State and SCG is outside Active State. With DC the timing is not aligned, i.e. the DRX cycles and drx-OnDurationTimer may start/stop at different times. With secondary DRX the timing is aligned between the DRX groups, i.e. the DRX cycles in the default and secondary DRX group is started/stopped at the same time.

With secondary DRX a shorter drx-OnDurationTimer and drx-InactivityTimer for FR2 can be configured to enable the secondary DRX group on FR2 to go into sleep more quickly and save power:

Cross carrier scheduling is not supported with secondary DRX. The trigger for aperiodic CSI and PUSCH reporting is on the same carrier e.g. in PCell of the default DRX group, but the cell for which CSI is reported may be in the same or other DRX group (frequency range). The network does not trigger aperiodic for the other DRX group, when the other DRX group is outside Active Time, i.e. the UE is only required to measure during Active Time.

DCP can be configured with CA or DC, i.e. the primary cell in the cell group can be configured with DCP. When DCP is triggered then the drx-OnDurationTimer is started and all the serving cells of the cell group start monitoring PDCCH and enter Active Time. This is not applicable for cells that are deactivated or in dormant state, i.e. the UE does not monitor PDCCH on those cells. DCP is not supported with secondary DRX, because secondary DRX was introduced under TEI and it was agreed to keep it simple.

In case the UE does not support LP-WUS on some of the cells/bands in connected mode then the UE can be reconfigured such that LP-WUS is supported on PCell/PSCell. The UE may support LP-WUS using one or multiple LRs e.g. each LR supports a limited number of bands/frequencies to simplify the implementation. Or the UE may support LP-WUS only in one DRX group (e.g. FR1 only) or only in one Cell group.

In the following it is assumed that option 1-1 is a special case/configuration of option 1-2, i.e. LP-WUS periodicity equals the C-DRX cycle length.

LP-WUS should be supported with CA and DC on the primary cell of the cell group, similar as with DCP:

LP-WUS can be configured with CA and DC on the primary cell of the cell group. When LP-WUS is detected and the lpwus-OnDurationTimer is started then all the serving cells of the cell group enter Active Time.

LP-WUS should also be supported with secondary DRX and configured per DRX group. As stated before secondary DRX was kept simple because it was treated. There are not technical reasons to not support LP-WUS with secondary DRX, similar as with AC/DC.

Furthermore, the secondary DRX group functionality can be improved with LP-WUS. Because one of the limitations of secondary DRX was that when a new traffic burst arrives and FR2 is sleeping, then data transmissions on FR2 can only start at the end of the DRX cycle, i.e. when drx-OnDurationTimer on FR2 is started. This limitation, i.e. DL delay for new data is eliminated with option 1-2 where the UE monitors LP-WUS periodically outside Active Time. Note that potential cross carrier issues are avoided, i.e. no need for cross carrier scheduling:

LP-WUS can be configured with secondary DRX, i.e. one LP-WUS is configured on the PCell of the default DRX group and another LP-WUS on a serving cell of the secondary DRX group. When LP-WUS is detected and the lpwus-OnDurationTimer of the DRX group is started then all the serving cells of the DRX group enter Active Time.

## Activation/deactivation

RAN1 and RAN2 agreed that LP-WUS monitoring is enabled/disabled via RRC signalling. It is FFS whether there is additional L1/L2 signalling, timers or conditions to activate/deactivate LP-WUS monitoring when LP-WUS is configured.

Using existing RRM/CSI measurement reporting the gNB can configure or de-configure LP-WUS via RRCReconfiguration, e.g. when the UE is inside/outside LP-WUS coverage. When LP-WUS is configured the gNB uses LP-WUS to wake-up the UE irrespective whether the UE is inside or outside LP-WUS coverage. To avoid ping-pong, i.e. frequent reconfigurations, the gNB can decide to configure LP-WUS after a certain time period when the UE has reported good coverage and immediately de-configure LP-WUS when the quality drops below a certain threshold. This can be left to gNB implementation.

It is important that LP-WUS it is reliable and robust, i.e. the case that the UE is using LP-WUS but the NW is not should be avoided, i.e. causes inter-operability issue. When the NW is using LP-WUS but the UE directly monitors PDCCH does not cause inter-operability issue, i.e. only wastes resources.

For example when the UE is outside LP-WUS coverage and cannot receive LP-WUS then the UE should monitor PDCCH directly. It is possible that the UE suddenly loses LP-WUS coverage and there is delay in the RRM/CSI measurement reporting, then the UE may become temporarily unreachable until LP-WUS is de-configured. Typically RRM and CSI reporting is not configured periodically to avoid high signalling load, but event based (RRM) or aperiodic (CSI). Independent on how the reporting is configured, the UE performs RRM/RLM/BFD measurements much more often than the reporting, and a local decision by the UE on current measurements can take much quicker effect.

To resolve the problem that the UE suddenly may lose LP-WUS coverage before LP-WUS is de-configured by the gNB an LR exit condition can be configured, similar when LP-WUS is configured in SIB for UEs in idle/inactive. In connected mode the UE is not required to measure LP-SS, i.e. an exit condition based on LP-SS measurements is not possible. But the exit condition can be based on RRM/RLM/BFD measurements, and RLM/BFD measurements are preferred because the UE is also required to measure RLM/BFD outside Active Time at least every C-DRX cycle, i.e. exit condition is also detected outside Active Time when there is no traffic:

The LP-WUS configuration can include an exit condition based on RLM measurements. When the exit condition is fulfilled the UE stops monitoring LP-WUS and monitors PDCCH directly.

## Option 1-3

Option 1-3 concerns PDCCH monitoring triggered by LP-WUS during Active Time (except when the drx-InactivityTimer is running):

There are existing features like PDCCH skipping, SSSG switching and cross slot scheduling that can be used effectively on this very short time scale where light/micro sleep is possible during Active Time. For example with PDCCH skipping the UE may enter micro or light sleep temporarily while sending and receiving data. The pdcch-SkippingDurations that can be configured are between {1, 2, 3, …, 20, 30, 40, 50, 60, 80, 100} ms.

RAN2 should not discuss enhancements on a very short time scale which do not impact MAC. Furthermore few companies see the benefit of option 1-3 in addition to existing L1 power saving methods:

Option 1-3 is not supported.

## Paging PDCCH

In connected mode the UE does not receive Paging PDSCH, but the UE is required to monitor Paging PDCCH, e.g. to receive ETWS/CMAS warning messages. If LP-WUS is configured in SIB, then the UE in connected mode can use the idle/inactive LP-WUS to monitor Paging PDCCH:

If LP-WUS in idle/inactive mode is configured in SIB then the UE in connected mode may use LP-WUS monitoring as in idle/inactive mode to receive Paging PDCCH e.g. SI-change and ETWS/CMAS indication in connected mode.

## Specification impact

DCP monitoring is specified in RAN1 38.213 and lower layers provide an indication to MAC when DCP is detected (38.321):

3>	if DCP indication associated with the current DRX cycle received from lower layer indicated to start drx-onDurationTimer, as specified in TS 38.213 [6]; or

With DCP there is an implicit assumption that lower layers are aware of the MAC Active Time, i.e. the UE is only required to monitor DCP outside Active Time before the OnDuration. It is proposed to follow the same principle with LP-WUS:

Lower layer indicates when LP-WUS is detected outside Active Time.

The difference between LP-WUS and DCP is that:

- Lower layer indication that LP-WUS is detected may happen during any LP-WUS occasion outside Active Time during a DRX cycle, not only before the LP-WUS occasion before the OnDuration.

- Dependent on the configuration and traffic it is possible that lpwus-OnDurationTimer is started multiple times during a DRX cycle, however this is not considered a typical case.

In case LP-WUS is captured in a similar way as DCP in MAC, then the two cases described above are also captured implicitly. RAN1 will discuss the impact on 38.213, and hopefully the impact is similar as with DCP, i.e. the same principles apply but there are only more monitoring occasions.

## Subgrouping

RAN1 agreed that subgrouping is also supported in connected mode:

Agreement

For RRC CONNECTED mode, maximum number of LP-WUS information bits is up to X bits

- FFS value X, which is no more than [8 or 16]

The configured OnDuration of UEs in connected mode typically do not overlap, or at least an attempt is made to minimize overlap. During the OnDuration the gNB may want to reach a specific UE, and when the start of the OnDurations coincide, then the LP-WUS can wake-up multiple UEs and cause false alarms.

The number of “individual” LP-WUS occasions that can be configured in connected mode depends on:

- C-DRX cycle length

- Numerology, i.e. number of slot per C-DRX cycle

- LP-WUS duration (1 or more slots)

- LP-WUS bandwidth (up to 5 Mhz)

The number of required LP-WUS occasions depends on the number of UEs in connected mode, and penetration of the feature, i.e. whether the UE supports the feature.

Subgroup info can be included in the LP-WUS configuration in connected mode. Wait for RAN1 progress to discuss further details.

# Summary

RAN2 is kindly asked to discuss LP-WUS in connected mode:

Observation 1	LP-WUS provides limited power saving gains in connected, i.e. even there is no traffic, the MR still has to measure RLM/BFD every C-DRX and report periodic CSI/L1-RSRP, if configured.

Proposal 1	(Long) DRX command MAC CE can be used with option 1-1 and 1-2 to stop drx-onDurationTimer (option 1-1), lpwus-OnDurationTimer (option 1-2) and drx-InactivityTimer. When outside Active Time the UE starts monitoring LP-WUS.

Proposal 2	With option 1-2 when the UE monitors LP-WUS outside Active Time and LP-WUS is detected then the UE enters Active Time and starts monitoring PDCCH.

Proposal 3	RAN2 confirms that option 1-2 is enabled/disabled using RRC signalling.

Proposal 4	The LP-WUS configuration is included in a new lpwus-Config-r19 IE in PhysicalCellGroupConfig IE in RRCReconfiguration message. Network only configures lpwus-Config-r19 when drx-Config is configured.

Proposal 5	The LP-WUS periodicity with option 1-2 can be equal to the C-DRX cycle length. The LP-WUS periodicity is larger or equal to the time offset.

Proposal 6	From RAN2 perspective LP-WUS can be configured with short DRX.

Proposal 7	When the LP-WUS periodicity equals the C-DRX cycle length and the UE is not able to monitor the LP-WUS occasion then the UE starts lpwus-OnDurationTimer. FFS when a smaller LP-WUS periodicity is configured and the UE is not able to monitor one of the LP-WUS occasions.

Proposal 8	The UE can be configured (e.g. lpwus-WakeUp) to start lpwus-OnDurationTimer when the UE is able to monitor LP-WUS but LP-WUS is not detected outside Active Time.

Proposal 9	The gNB configures a single time offset which is equal or longer than the minimum time gap signaled in the LP-WUS UE capability.

Proposal 10	If configured, the UE can signal a preferred time offset via UAI signalling.

Proposal 11	If the UE may not support LP-WUS in connected mode on all supported bands, then the LP-WUS UE capability for connected mode includes a supportedBandListNR IE.

Proposal 12	LP-WUS can be configured with CA and DC on the primary cell of the cell group. When LP-WUS is detected and the lpwus-OnDurationTimer is started then all the serving cells of the cell group enter Active Time.

Proposal 13	LP-WUS can be configured with secondary DRX, i.e. one LP-WUS is configured on the PCell of the default DRX group and another LP-WUS on a serving cell of the secondary DRX group. When LP-WUS is detected and the lpwus-OnDurationTimer of the DRX group is started then all the serving cells of the DRX group enter Active Time.

Proposal 14	The LP-WUS configuration can include an exit condition based on RLM measurements. When the exit condition is fulfilled the UE stops monitoring LP-WUS and monitors PDCCH directly.

Proposal 15	Option 1-3 is not supported.

Proposal 16	If LP-WUS in idle/inactive mode is configured in SIB then the UE in connected mode may use LP-WUS monitoring as in idle/inactive mode to receive Paging PDCCH e.g. SI-change and ETWS/CMAS indication in connected mode.

Proposal 17	Lower layer indicates when LP-WUS is detected outside Active Time.

Proposal 18	Subgroup info can be included in the LP-WUS configuration in connected mode. Wait for RAN1 progress to discuss further details.

# References

- R1-2410909, LS on LP-WUS operation in CONNECTED mode, LS out, Docomo, To: RAN2, RAN1#119

# Annex: Agreements

## RAN1 agreements (selection)

LOs and MOs

Agreement

LP-WUS occasions (LOs) are defined for LP-WUS monitoring.

- Each LO has one or more LP-WUS monitoring occasions (MOs), where UE can monitors for LP-WUS transmission in each of the LP-WUS MOs.

  - Different LP-WUS MOs may correspond to different beams in multi-beam operation

  - FFS whether or not each LO is defined as a time window that covers the corresponding LP-WUS MOs

  - FFS details

- It is at least supported that a UE monitors LOs with a configured periodicity.

Agreement

LP-WUS monitoring occasions (MOs) are configured by RRC, where UE can monitor for LP-WUS transmission in RRC CONNECTED mode.

- FFS whether to define a time window for Mos

- It is at least supported that a UE can monitor MOs with a periodicity.

  - FFS details of the periodicity, e.g. derived from DRX cycle, separately configured

CSI/L1-RSRP reporting

Agreement

For option 1-2 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective.

  - …

- No impact on RRM/RLM/BFD measurement requirements is assumed

- For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1)

  - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up

  - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not

Agreement

- For Option 1-1 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective.

  - …

  - No impact on RRM/RLM/BFD measurement requirements is assumed

  - For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP)

    - Periodic CSI/L1-RSRP is not reported during the time given by the configured drx-onDurationTimer if UE is not indicated to wake-up

    - Periodic CSI/L1-RSRP is periodically reported during the time given by the configured drx-onDurationTimer regardless if UE is indicated to wake-up or not

      - Note: Periodic CSI/L1-RSRP is reported during CDRX active time as in legacy specification

Minimum time gap

Agreement

For RRC CONNECTED mode, minimum time gap between LP-WUS reception and MR to start PDCCH monitoring is introduced considering at least following

- LP-WUS processing time

- MR transition time for ramp up

- Time/frequency synchronization of MR

- FFS whether UE can report supported minimum time gap from candidate values

FFS: Whether the minimum time gap values can be more than one

Agreement

For RRC CONNECTED mode, support UE capability report for determination of minimum time gap between LP-WUS reception and MR to start PDCCH monitoring.

- FFS: exact value(s) of the minimum time gap

- FFS: support of multiple minimum time gaps

- FFS whether the reported value includes the duration for time/frequency synchronization of MR

Agreement

Select one of the following alternatives in RAN1#118bis

Alt 1: For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting.

  - FFS: X

  - FFS: definition of reported value

Alt 2: For RRC CONNECTED mode, UE reports multiple values for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting.

  - FFS: X

  - FFS: definition of reported value

  - Different minimum time gaps correspond to different sleep states

  - Companies are encouraged to details on how the reported values are to be used by the network

Agreement

For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting.

- FFS: X

- FFS: definition of reported candidate value

- UE can indicate its preference for configured time offset between LP-WUS and PDCCH reception using UAI mechanism. FFS details. UAI is optional.

- FFS: A single time offset is configured for a UE at a given time.

RAN1#119

Agreement

LP-WUS in RRC CONNECTED mode does not include Scell dormancy indication.

- Note: LP-WUS indication does not impact SCell dormancy behavior

Agreement

For LP-WUS CONNECTED mode operation, the following case is not supported:

- Both Option 1-1 and Option 1-2 are simultaneously configured for the same UE in a given cell

Agreement

- For LP-WUS MOs in connected mode for Option 1-1, following is considered further discussion and possible down-selection

  - Approach 1:

    - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

    - UE selectively monitors LP-WUS in the LP-WUS MOs identified by other parameters such as time offset prior to the start of drx-onDurationTimer.

  - Approach 2:

    - LP-WUS MOs, including periodicity and offset, are identified based on the slot that drx-onDurationTimer would start and a new RRC parameter indicating the time offset until the slot that drx-onDurationTimer would start.

  - FFS: An RRC parameter indicates the duration where the UE monitors LP-WUS [consecutively in time]

    - FFS: the consecutive LP-WUS MOs in time identified by the RRC parameter are associated with a same C-DRX cycle;

    - FFS: LP-WUS MOs in time identified by the RRC parameter provides consistent indication for the same C-DRX cycle of the UE, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOs associated with the same C-DRX cycle.

- For LP-WUS MOs in connected mode for Option 1-2, following is considered further

  - Approach 1:

    - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

    - UE monitors LP-WUS in the LP-WUS MOs and starts a new timer for PDCCH monitoring triggered by LP-WUS, after a time offset.

  - Approach 2:

    - Periodicity/offset of the slot that the new timer for PDCCH monitoring may be potentially triggered by LP-WUS are configured by RRC parameters.

    - LP-WUS MOs, including periodicity and offset, are identified based on the slot that new timer would start and a new RRC parameter indicating the time offset until the slot that new timer would start.

  - FFS: An RRC parameter indicates the duration where the UE monitors LP-WUS [consecutively in time]

    - FFS: the consecutive LP-WUS MOs in time identified by the RRC parameter are associated with the same slot that the new timer would start;

    - FFS: LP-WUS MOs in time identified by the RRC parameter provides consistent indication for the same slot that the new timer would start, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOs associated with the same slot that the new timer would start.

Agreement

For Option 1-2 of LP-WUS CONNECTED mode operation, for explicit indication to stop the new timer for PDCCH monitoring in Option 1-2 and drx-InactivityTimer for the DRX group, RAN1 identified the following potential solution

- Use existing DRX Command MAC CE and Long DRX Command MAC CE

- When UE is not in C-DRX active time, UE monitors LP-WUS

Send LS to RAN2 to ask the feasibility of the above potential solution. LS in R1-2410909.

Conclusion

For Option 1-1 of LP-WUS CONNECTED mode operation, RAN1 assumes existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and/or drx-InactivityTimer for the DRX group

- When UE is not in C-DRX active time, UE monitors LP-WUS

Send LS to RAN2 to ask whether RAN2 has same understanding or not. LS in R1-2410909.

Agreement

For Option 1-2, for LP-WUS CONNECTED mode operation, the periodic CSI/L1-RSRP reporting operation is same as Rel-16 DCP, that is the UE can be configured with a parameter to enable/disable periodic CSI/L1-RSRP reporting, respectively:

- If the parameter is NOT configured:

  - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is not reported.

  - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time.

- If the parameter is configured:

  - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the time given by the configured drx-onDurationTimer in DRX-Config for the case when UE is outside C-DRX active time.

  - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time.

- FFS: Whether RAN1 spec impact is required.

For future meetings:

Companies are encouraged to consider [Fri]Combined Proposal 3.2-1/3.2-2c of R1-2410906 for further discussions in future meetings.

## RAN2 agreements (selection)

LP-WUS in connected

In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1.

For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1.

For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline.

RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE.

The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

Understanding of Option 1-2-1 and 1-2-2

For option 1-2,

            - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.)

            - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1.

Option 1-2 related

For Option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2.

Option 1-2 (Handling of drx-onDurationTimer, periodic CSI/L1-RSRP reporting, etc.)

          - drx-onDurationTimer is not started with Option 1-2 LP-WUS.

          - For Option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer, for the case when UE is outside C-DRX active time.

          - For option 1-2, if UE receives DRX command MAC CE or Long DRX command MAC CE, UE stops the new timer triggered by LP-WUS.

Option 1-1 and 1-2

Don’t support Option 1-1 and Option 1-2 simultaneously configured for the same UE.

LP-WUS for CA and DC

- For both option 1-1 and option 1-2

    - ?? The LP-WUS can also be configured with secondary per DRX group.

    - ?? In NR-DC case, LP-WUS can be configured separately in MCG and SCG, and LP-WUS could trigger the PDCCH monitoring of all activated serving cells within the same cell group.